3GPP TSG-RAN WG2 Meeting #123                                  	    R2-2308189

Toulouse, France, 21- 25 August, 2023

Agenda Item:	7.16.2.4

Source: 	MediaTek Inc.

Title:  	Model Control and Model Monitoring

Document for:	Discussion and decision

Introduction

In RAN1#110b-e meeting, RAN1 made following agreements related to model selection, activation, deactivation, switching, and fallback. We call the above operations as model control for short to facilitate the discussion.

Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms

In RAN2#121b-e meeting, RAN2 made following agreements related to model selection, activation, deactivation, switching and fallback.

For the CSI compression and beam management use cases, model/function selection/(de)activation/switching/fallback can be UE-initiated or gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model). For the positioning use case, model/function selection/(de)activation/switching/fallback can be UE-initiated or LMF-/ gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model).

In this contribution, we figure out the generic procedures of model control for UE sided models and two-sided models identified by RAN1 and apply them to the different use cases.

Discussion

RAN1 also agreed to study model monitoring at least for the purposes of model control and model update and elaborated on how model monitoring interacts with model control for different use cases.

CSI feedback enhancement In CSI compression using two-sided model use case: - NW-side performance monitoring: NW monitors the performance and make decisions of model activation/ deactivation/updating/switching. Impact to enable performance monitoring using an existing CSI feedback scheme as the reference, including the association between AI/ML scheme and existing CSI feedback scheme for monitoring, are considered. Note: The metric for monitoring and comparison includes intermediate KPI and eventual KPI. - UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/deactivation/updating/switching. Impact on triggering and means for reporting the monitoring metrics, including periodic/semi-persistent and aperiodic reporting, and other reporting initiated from UE, are considered. Beam Management For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Type1 performance monitoring: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE may have different operations - Option1: UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - Option2: UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - Indication from NW for UE to do LCM operations - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type2 performance monitoring (UE-side performance monitoring): - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring - If it is for UE-side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - UE reporting of beam measurement(s) based on a set of beams indicated by gNB - Signalling, e.g., RRC-based, L1-based - Note: Performance and UE complexity, power consumption should be considered Positioning Enhancement - If certain type of data is necessary for computing monitoring metric: - How an entity can be used to provide the given type of data for calculating monitoring metric: companies requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric - Potential assistance signalling and procedure to facilitate an entity providing data for calculating monitoring metric - Potential UE-network interaction: e.g., model monitoring decision indication between UE and network - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - gNB at least for Case 3a (with gNB-side model) - LMF at least for Case 2b and 3b (with LMF-side model) - If model monitoring does not require ground truth label (or its approximation). - Monitoring metric: e.g., statistics of measurement(s) compared to the statistics associated with the training data. - Note: the measurement(s) may or may not be the same as model input. - Assistance signalling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc. - report of the calculated metric and/or model monitoring decision - If model monitoring requires and is provided ground truth label (or its approximation) - Monitoring metric: statistics of the difference between model output and provided ground truth label. - Provisioning of ground truth label and associated label quality. - Assistance signalling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc. - report of the calculated metric and/or model monitoring decision

Table 1 is trying to figure out the generic procedures of model control for UE sided models and two-sided models and the applicable use cases. Although there is no sufficient progress for AI-based positioning for model monitoring and model control, RAN1 discussed which entity is responsible to derive monitoring metric and agreed that UE derives the monitoring metric at least for case 1(UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning) and case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning). But model control based on the monitoring metric has not been concluded for AI-based positioning. It is assumed that the agreements for the generic aspects are applicable to AI-based positioning.

Table 1 Network-decided vs. UE-decided for different use cases

| Network-decided, Network-monitoring, |  | Network-decided, UE-monitoring |  |
| --- | --- | --- | --- |
| - CSI compression: NW-side performance monitoring: NW monitors the performance and make decisions of model activation/ deactivation/updating/switching - BM: Type1 performance monitoring of Option1: UE sends reporting to NW (e.g., for the calculation of performance metric at NW |  | - CSI compression: UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/deactivation/updating/switching - BM: Type1 performance monitoring of option 2: UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - POS case1, case 2a (performance report to LMF) |  |
| UE-decided, decision reported to network, network configuration for event | UE-decided, UE autonomous, decision reported to network |  | UE-decided, UE autonomous, decision not reported to network |
| : |  |  |  |
| - BM: Type2 performance monitoring (UE-side performance monitoring): - Indication/request/report from UE to gNB for performance monitoring - If it is for UE-side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation |  |  |  |

It can be observed that for at least for UE-sided models and two-sided models, network-decided approach is considered for all use cases, including CSI compression, BM-case 1 and BM-case 2. UE-decided approach is only considered for UE-sided model, like BM-case 1/2 where AI/ML training, inference as well as model monitoring is performed at the UE side. However, there are some uncertainties for UE-decided approach, e.g., what events/conditions are required to be configured by the network and whether UE needs to report its decision to the network, which needs more RAN1 inputs. Furthermore, UE-decided approach assumes that model monitoring is performed at the UE side.

Observation 1: Network-decided control considers both cases that model monitoring is performed at UE side and/or network side for at least UE-sided and two-sided model.  UE-decided control considers the case that model monitoring is performed at the UE side.

Proposal 1: RAN2 study following mechanisms of model control for UE-sided and two-sided model. FFS on other mechanism. Take the generic procedures in table 1 as starting point for model control and monitoring.

- Network-decided control with model monitoring performed at network side and/or UE side,

- UE-decided control with model monitoring performed at UE side.

For network sided model, the model monitoring and model control should be performed by the network itself, although it’s possible that certain UE measurement and reports are required for the purpose of model monitoring.

Proposal 2: RAN2 assumes that network performs model monitoring and model control for network-sided model.

Conclusion

Observation:

Observation 1: Network-decided control considers both cases that model monitoring is performed at UE side and/or network side for at least UE-sided and two-sided model.  UE-decided control considers the case that model monitoring is performed at the UE side.

Proposals:

Proposal 1: RAN2 study following mechanisms of model control for UE-sided and two-sided model. FFS on other mechanism. Take the generic procedures in table 1 as starting point for model control and monitoring.

- Network-decided control with model monitoring performed at network side and/or UE side,

- UE-decided control with model monitoring performed at UE side.

Proposal 2: RAN2 assumes that network performs model monitoring and model control for network-sided model.

Reference

[1] R1-2306235, TR38.843 v0.1.0: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface