3GPP TSG RAN WG2 #123 Toulouse, France	R2-2308314

Aug 21st-25th, 2023

Agenda item:	7.14.4

Source:	CMCC

Title:	Discussion on QoE in NR-DC

Document for:	Discussion

1	Introduction

In RAN2 122 [1], consensus has been reached on QoE in NR-DC, and leaving serveral FFS and future works, which are shown in the following:

FFS how to handle the QoE report transmission if there is only one SRB and the QoE report is not (explicitly) configured for that SRB (to be checked if this is possible according to latest RAN3 agreements)

FFS how the above case works if SCG is deactivated or released.

In this contribution, a discussion on QoE in NR-DC will be conducted.

2	Discussion

2.1 QoE report in NR-DC

In the previous discussion, RAN2 had agreed to use explicit indication per QoE configuration for indicating reporting SRBs. Under normal circumstances, the SRB indicated by the explicit indication should be established. However, if the indicated SRB is not configured, the behavior of the terminal is not defined.

In RAN3#120[2], RAN3's view was basically consistent with RAN2, that is, UE can be explicitly indicated for reporting QoE on the SRB, but RAN3 did not consider the case where the indicated SRB is not configured.

RAN3 understanding is that the UE can be explicitly instructed whether to report over SRB5 or SRB4.

Therefore, we believe that in NR-DC, if the UE is only configured with one of SRB4 or SRB5, and the indicated SRB is the other, the UE should use the currently configured SRB4/SRB5 to report QoE in order to ensure the availability of QoE reporting.

Proposal 1: UE can use configured SRB4/SRB5 for QoE reporting in NR-DC if the explicit indicated SRB is not configured.

Then UE can indicate that the QoE configuration comes from the NW exists conflict.

Proposal 2: UE can indicate RAN via RRC message that indicated SRB is not configured and request RRC Reconfiguration for suitable QoE configuration.

Another issue is about the QoE measurement after the SCG is deactivated or released. RAN3 has preliminarily discussed this issue and reached a conclusion.

If the SN configured a UE with QoE measurements, at SN release, the QoE/RVQoE configuration can be released. Whether the SN-configured QoE/RVQoE configuration information can be passed to the MN in case of SN release needs to be further discussed.

From the perspective of maintaining the continuity of QoE measurement, if the SN configuration for QoE/RVQoE is not released when the SN is released, the UE or MN should retain the relevant configurations.

For SCG deactivation, we believe that there is no need to retain the related measurement configurations. However, this issue can wait for the conclusion from RAN3.

Proposal 3: RAN2 to wait for RAN3's conclusion on whether the SN-configured QoE/RVQoE configuration information can be passed to the MN when SN release.

2.2 MN-SN coordination

Based on the previous conclusions from RAN3, for m-based QoE, the MN can send the QoE reference and MCE IP address to the SN, to ensure that the SN can use the correct parameters to transmit QoE. Vice versa, the SN can also send the QoE reference and MCE IP address to the MN for the same purpose.

When MN configures a UE with m-based QoE, it may indicate to SN: the QoE Reference, the MCE IP address. FFS for other information (e.g., RRC ID)

and,

If the SN is asked by the MN to forward to the MCE the QoE reports pertaining to a measurement configured by the MN, the MN should indicate to the SN the QoE Reference, the MCE IP Address and the RRC ID.

also, MN is also responsible for s-based QoE configuration.

MN is responsible to configure the s-based QoE to UE. If the M-based QoE configuration is received by the MN, the MN should make the decision on the UE selection and on which node sends the QoE configuration to the UE.

Similarly, when the MN needs to send an SN-associated QoE report, it should have the relevant information such as the MCE IP address. Therefore, if the MN receives a QoE report from the SN on SRB4, it should already be aware of the information related to sending QoE reports.

However, if the QoE configuration is directly downlinked to the SN by the OAM, the SN should send the MCE IP address, QoE reference, and RRC ID to the MN together.

Proposal 4: If QoE configuration is send directly to SN, SN can send QoE Reference, MCE IP address and RRC ID to MN.

As for RVQoE MN-SN coordination, it’s under RAN3’s discussion now. RAN2 can wait for RAN’s conclusion.

3	Summary

This contribution discusses Rel-17 leftover topics for QoE, the following are the proposals that may be taken into consideration:

Proposal 1: UE can use configured SRB4/SRB5 for QoE reporting in NR-DC if the explicit indicated SRB is not configured.

Proposal 2: UE can inform RAN via RRC message that indicated SRB is not configured and request RRC Reconfiguration for suitable QoE configuration.

Proposal 3: RAN2 to wait for RAN3's conclusion on whether the SN-configured QoE/RVQoE configuration information can be passed to the MN when SN release.

Proposal 4: If QoE configuration is send directly to SN, SN can send QoE Reference, MCE IP address and RRC ID to MN.

References

- R2-2306542, Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia)

- RAN3_120_agenda_20230530_EOM1.