3GPP TSG-RAN WG2 Meeting #123	R2-2308441

Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.4.2.2

Source:	Ericsson

Title:	LTM handling for fast recovery and RRC Re-establishment

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#121bis meeting the following was agreed concerning the UE actions on failure detection while the UE is configured with LTM candidate cells:

Following behaviors of LTM supervisor timer are agreed:

- - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE;

- - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch;

- - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS)

LTM supervisor timer is RRC layer timer.

At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution.

This paper proposes a simple fast recovery solution for LTM, aiming to minimize the work in Rel-18.

# 2	Discussion

## 2.1	Fast recovery in Conditional Handover (CHO)

When a UE is configured with CHO candidate cells (i.e. UE has stored CHO candidate cell configuration(s)) and a failure is detected e.g. Radio Link Failure (RLF) or expiry of timer T304, due to HO failure or CHO execution failure, the UE initiates an RRC Re-establishment procedure, starts timer T311, and performs cell selection.

If the selected cell is a cell for which the UE has a stored CHO candidate cell configuration and the UE is configured to perform fast recovery (‘attemptCondReconfig’ set to TRUE in the IE ConditionalReconfiguration) the UE applies the stored CHO candidate cell configuration associated to the selected cell and perform actions as specified in 5.3.5.3, otherwise the UE continues with the RRC re-establishment procedure, as shown below:

***************************************************************************************************************************

### 5.3.7	RRC connection re-establishment

[…]

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

[…]

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG or mobility from NR failure, and

1>	if attemptCondReconfig is configured; and

[…]

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig:

[…]

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

[…]

1>	else:

[…]

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

***************************************************************************************************************************

When RLF, CHO failure or HOF is detected, the UE initiates RRC re-establishment, performs cell selection while timer T311 is running and if the selected cell is a CHO candidate cell the UE performs a CHO execution, otherwise the UE continues with the RRC re-establishment procedure.

## 2.2	Fast recovery in LTM

When the UE is configured with LTM, the UE also has stored LMT candidate cell configuration(s). An LTM cell switch failure is agreed to be controlled by a supervision timer, and when the timer expires, the UE considers LTM failure and initiates RRC re-establishment. That would be the same UE behaviour in case of a HO failure or RLF.

When RLF, HO failure or LTM failure is detected, the UE initiates RRC re-establishment, which leads the UE to perform cell selection while timer T311 is running.

As in CHO, the UE also has stored LTM candidate cell configuration(s), in the form of an RRCReconfiguration. If the outcome of cell selection is a cell for which the UE has a stored LTM candidate cell configuration, and the UE continues with the RRC re-establishment, there is a chance that the UE anyways would receive from the network in the first RRCReconfiguration after re-establishment similar parameters as stored in the LTM candidate cell configuration, which would be a waste and that would be a procedure with increased delay and signalling, compared to a fast recovery.

Thus, it seems reasonable to design a fast recovery procedure in case the UE selects a cell while timer T311 is running which is an LTM candidate cell. In that case, instead of continuing with the RRC re-establishment procedure, the UE would perform an LTM execution using the LTM candidate cell configuration associated to the selected cell, similar to what is already the case for CHO.

(LTM fast recovery) When RLF, HO failure or LTM failure is detected, the UE initiates RRC re-establishment and performs cell selection while timer T311 is running. If the selected cell is an LTM candidate cell, the UE performs an LTM cell switch to the selected LTM candidate cell.

If the selected cell is not an LTM candidate cell, the UE continues with an RRC re-establishment procedure.

The fast recovery in the case the UE is configured with CHO is controlled by the field ‘attemptCondReconfig’. LTM fast recovery could also be controlled by a parameter a single parameter called ‘attempt-LTM’, within the IE LTM-Config-r18, as follows:

RRCReconfiguration-v18xy-Ies ::=        SEQUENCE {

ltm-Config-r18                 SetupRelease {LTM-Config-r18}                         OPTIONAL, -- Need M

nonCriticalExtension                    SEQUENCE {}                                                    OPTIONAL

}

[…]

LTM-Config-r18 ::=   SEQUENCE {

lte-ReferenceConfiguration-r18        OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL,   -- Cond FirstLTM-Candidate

ltm-CandidateToReleaseList-r18        LTM-CandidateToReleaseList-r18 OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         LTM-CandidateToAddModList-r18  OPTIONAL,   -- Need N

ltm-CandidateNoResetL2-List-r18       SetupRelease { FFS }           OPTIONAL,   -- Need M

...

attempt-LTM-r18              		  ENUMERATED {true}              OPTIONAL,

}

As in CHO, LTM fast recovery is controlled by a single parameter e.g. upon failure detection, if ‘attempt-LTM’ is included in LTM-Config-r18 the UE performs LTM fast recovery, otherwise it performs RRC re-establishment.

In CHO, fast recovery is only allowed to be performed once. There were proposals to define a counter whose value would relate to configurable maximum number of attempts for CHO fast recovery, but the final solution was intended to be simple, since fast recovery was already an opportunistic approach to optimize the recovery performance from rare failure cases when CHO was configured. For LTM fast recovery, we should also aim to have a simple approach, so we propose we only allow LTM fast recovery to be performed once, when a failure is detected.

CHO fast recovery is only allowed to be performed once.

As in CHO, the UE is only allowed to perform LTM fast recovery one time (no need to define a counter for a maximum number of fast recovery attempts).

In CHO, when the UE applies the stored CHO candidate cell configuration (stored RRCReconfiguration) due to a fast recovery (cell selection while timer T311 is running) the UE releases all other CHO candidate cell configurations. By doing that, if this fast recovery procedure also fails (e.g. timer T304 expires), the UE would not have further CHO candidate cell configurations to performed another fast recovery, i.e., CHO fast recovery is only performed one time. That is show below:

***************************************************************************************************************************

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC):

1>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3:

2>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

[…]

***************************************************************************************************************************

The simplest solution for LTM fast recovery is to adopt a similar approach, to make sure that LTM fast recovery is only performed once. One difference is that in CHO, if that CHO fast recovery procedure would succeed, these other CHO candidate configurations would have been anyways deleted (as there is no subsequent CHO executions). However, subsequent LTM cell switches are supported in the normal operation, so if we want to allow that to also be possible after a fast recovery, we have to define another modelling in RRC to allow only one fast recovery attempt. While we acknowledge the differences between CHO and LTM when it comes to subsequent executions, LTM fast recovery is an opportunistic approach, already providing some gains over the legacy RRC re-establishment in terms of signalling and latency in accessing the network. Thus, optimizing it further seems unnecessary. Hence, we propose that when the UE applies the stored LTM candidate cell configuration due to fast recovery, the UE simply deletes the other LTM candidate configuration(s), and LTM related configuration(s) such as the lower layer measurement configuration for LTM, the reference configuration, and the configurations related to the acquisition of early UL sync (for TA acquisition) and early DL sync (TCI state configurations for LTM candidates).

When the UE applies the stored LTM candidate configuration due to an LTM fast recovery, the UE deletes LTM related configuration(s), so LTM fast recovery only happens one time.

The IEs and fields for some of these LTM related configurations are still under discussion with RAN1. In the latest version of the running CR for RRC, under email discussion when the paper was drafted, the complete LTM candidate configuration(s) are stored within VarLTM-UE-Config, while the other configuration(s) per LTM candidate cell are within VarLTM-Config, LTM-Config and the ltm-CSI-ReportConfigToAddModList instances for all Serving cells.Below we show an example in RRC on how the feature could be modelled, having existing fields and IEs in the running CR as reference:

***************************************************************************************************************************

##### 5.3.5.x.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3, the UE shall:

[…]

1> if the LTM cell switch execution is triggered by the indication of lower layers:

2> apply the LTM configuration in ue-LTM-Config within VarLTM-UE-Config related to the LTM candidate cell configuration identity as received from lower layers according to clause 5.3.5.3;

1> else (LTM cell switch execution upon cell selection performed while timer T311 was running):

2> apply the LTM configuration in ue-LTM-Config within VarLTM-UE-Config related to the LTM candidate cell configuration of the LTM candidate cell selected while timer T311 was running, according to clause 5.3.5.3;

2> remove all entries within the MCG VarLTM-UE-Config, if any;

2> remove all entries within the MCG VarLTM-Config, if any;

2> remove LTM-Config, if any;

2> remove the ltm-CSI-ReportConfigToAddModList instances for all Serving cells;

Editor’s Note: FFS update the removal of LTM related information depending on the outcome of RAN1 parameters discussion.

Editor’s Note: FFS on whether a sub-clause is created for LTM release e.g. in case that is called by multiple procedures.

***************************************************************************************************************************

In CHO, the steps the UE performs for a CHO execution upon CHO fast recovery are the same as in a legacy CHO execution, which are the same as in a legacy HO procedure. However, as LTM cell switch has various options controlled by the LTM cell switch command, such as a beam indication, and indicated by the CU to the Candidate DU in the normal case, some simplifications may be needed for the LTM fast recovery, so the UE behaviour is predictable even when the Candidate DU is not expecting the UE, at least for the RLF case, or in the case of a failed LTM cell switch which was triggered towards another Candidate DU.

Figure 1 LTM cell switch inter-DU

A simplification is to enable the UE to perform actions according to the RRC configuration is the UE perform Random access when LTM cell switch is triggered by a fast recovery i.e. no RACH-less LTM cell switch solution is supported in Rel-18 for when the LTM cell switch is triggered by LTM fast recovery.

The UE always performs random access when LTM cell switch is triggered during LTM fast recovery.

## 2.3	RRC Re-establishment and LTM

In CHO, when the UE continues with RRC Re-establishment e.g. when selected cell is not a CHO candidate cell, or when the UE is not configured with ‘attemptCondReconfig’, the CHO related configurations (CHO candidate cell configurations and CHO related MeasConfig) are deleted, which simplifies network implementation at the target gNodeB in which the UE is trying to re-establish.

***************************************************************************************************************************

#### […]

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

[…]

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig:

[…]

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

[…]

1>	else:

[…]

2>	remove all the entries within the MCG VarConditionalReconfig, if any;

2>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

[…]

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

[…]

***************************************************************************************************************************

In our view, it also makes sense to define similar actions for a UE configured with LTM when the UE continues with an RRC re-establishment procedure.

When the UE configured with LTM continues with an RRC re-establishment, the UE removes the LTM related configurations (LTM candidate cell configurations, LTM related measurement configuration and TA establishment configuration).

This could be modelled in RRC as follows:

***************************************************************************************************************************

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

[…]

1>	if attemptCondReconfig is configured; and

[…]

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig:

[...]

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

[…]

1>	if attempt-LTM is configured; and

1>	if the selected cell is one of the LTM candidate cells in ue-LTM-Config within VarLTM-UE-Config:

2>	perform LTM cell switch execution procedure for the selected LTM candidate cell and perform actions as specified in 5.3.5.x.6;

1>	else:

[…]

2> remove all entries within the MCG VarLTM-UE-Config, if any;

2> remove all entries within the MCG VarLTM-Config, if any;

2> remove LTM-Config, if any;

2> remove the ltm-CSI-ReportConfigToAddModList instances for all Serving cells;

Editor’s Note: FFS on whether a sub-clause is created for LTM release e.g. in case that is called by multiple procedures.

***************************************************************************************************************************

There is also a set of actions that the UE needs to perform when RRC Re-establishment continues instead of CHO fast recovery (e.g., MAC reset and release of spCellConfig), in case ‘attemptCondReconfig’ is not configured, or, the selected cell while timer T311 is running is not a CHO candidate cell. Similar actions may be defined for the UE when the parameter controlling whether the UE uses LTM fast recovery or not (e.g. ‘attempt-LTM’) is not configured, or when the selected cell is not an LTM candidate cell.

When the parameter controlling LTM fast recovery (e.g. ‘attempt-LTM’) is not configured, the UE perform the same actions as when ‘attempt-CHO’ is not configured.

When the selected cell while timer T311 is running is not an LTM candidate cell, the UE perform the same actions as when the selected cell while timer T311 is running is not a CHO candidate cell.

# TP to TS 38.331

#### 5.3.5.x	LTM configuration and execution

[...]

##### 5.3.5.x.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3, the UE shall:

[…]

1> if the LTM cell switch execution is triggered by the indication of lower layers:

2> apply the LTM configuration in ue-LTM-Config within VarLTM-UE-Config related to the LTM candidate cell configuration identity as received from lower layers according to clause 5.3.5.3;

1> else (LTM cell switch execution upon cell selection performed while timer T311 was running):

2> apply the LTM configuration in ue-LTM-Config within VarLTM-UE-Config related to the LTM candidate cell configuration of the LTM candidate cell selected while timer T311 was running, according to clause 5.3.5.3;

2> remove all entries within the MCG VarLTM-UE-Config, if any;

2> remove all entries within the MCG VarLTM-Config, if any;

2> remove LTM-Config, if any;

2> remove the ltm-CSI-ReportConfigToAddModList instances for all Serving cells;

### […]

### 5.3.7	RRC connection re-establishment

#### […]

#### 5.3.7.2	Initiation

[…]

Upon initiation of the procedure, the UE shall:

[…]

1>	if UE is not configured with attempt-LTM; or

1>	if UE is not configured with attemptCondReconfig:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, except SRB0 and broadcast MRBs;

2>	release the MCG SCell(s), if configured;

2>	if MR-DC is configured:

3>	perform MR-DC release, as specified in clause 5.3.5.10;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release overheatingAssistanceConfig, if configured and stop timer T345, if running;

2>	release idc-AssistanceConfig, if configured;

2>	release btNameList, if configured;

2>	release wlanNameList, if configured;

2>	release sensorNameList, if configured;

2>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

2>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

2>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

2>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

2>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured stop timer T346e associated with the MCG, if running;

2>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

2>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

2>	release releasePreferenceConfig, if configured stop timer T346f, if running;

2>	release onDemandSIB-Request if configured, and stop timer T350, if running;

2>	release referenceTimePreferenceReporting, if configured;

2>	release sl-AssistanceConfigNR, if configured;

2>	release obtainCommonLocation, if configured;

2>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

2>	release musim-LeaveAssistanceConfig, if configured;

2>	release ul-GapFR2-PreferenceConfig, if configured;

2>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

2>	release propDelayDiffReportConfig, if configured;

2>	release rrm-MeasRelaxationReportingConfig, if configured;

2>	release maxBW-PreferenceConfigFR2-2, if configured;

2>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

2>	release minSchedulingOffsetPreferenceConfigExt, if configured;

#### […]

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

[…]

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG or mobility from NR failure, and

1>	if attemptCondReconfig is configured; and

1>	if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig:

2>	if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, or LTM cell switch failure or mobility from NR failure; and

1>	if attempt-LTM is configured; and

1>	if the selected cell is one of the LTM candidate cells in ue-LTM-Config within VarLTM-UE-Config:

2>	perform LTM cell switch execution procedure for the selected LTM candidate cell and perform actions as specified in 5.3.5.x.6;

[…]

1>	else:

2>	if UE is configured with attempt-LTM; or

2>	if UE is configured with attemptCondReconfig:

3>	reset MAC;

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

3>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

3>	release overheatingAssistanceConfig , if configured and stop timer T345, if running;

3>	if MR-DC is configured:

4>	perform MR-DC release, as specified in clause 5.3.5.10;

3>	release idc-AssistanceConfig, if configured;

3>	release btNameList, if configured;

3>	release wlanNameList, if configured;

3>	release sensorNameList, if configured;

3>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

3>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

3>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

3>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

3>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured and stop timer T346e associated with the MCG, if running;

3>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

3>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

3>	release releasePreferenceConfig, if configured and stop timer T346f, if running;

3>	release onDemandSIB-Request if configured, and stop timer T350, if running;

3>	release referenceTimePreferenceReporting, if configured;

3>	release sl-AssistanceConfigNR, if configured;

3>	release obtainCommonLocation, if configured;

3>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

3>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

3>	release musim-LeaveAssistanceConfig, if configured;

3>	release propDelayDiffReportConfig, if configured;

3>	release ul-GapFR2-PreferenceConfig, if configured;

3>	release rrm-MeasRelaxationReportingConfig, if configured;

3>	release maxBW-PreferenceConfigFR2-2, if configured;

3>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

3>	release minSchedulingOffsetPreferenceConfigExt, if configured;

3>	suspend all RBs, and BH RLC channels for the IAB-MT, except SRB0 and broadcast MRBs;

2>	remove all the entries within the MCG VarConditionalReconfig, if any;

2>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2> remove all entries within the MCG VarLTM-UE-Config, if any;

2> remove all entries within the MCG VarLTM-Config, if any;

2> remove LTM-Config, if any;

2> remove the ltm-CSI-ReportConfigToAddModList instances for all Serving cells;

Editor’s Note: FFS on whether a sub-clause is created for LTM release e.g. in case that is called by multiple procedures.

2>	release the PC5 RLC entity for SL-RLC0, if any;

2>	start timer T301;

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

NOTE 2:	This procedure applies also if the UE returns to the source PCell.

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	(LTM fast recovery) When RLF, HO failure or LTM failure is detected, the UE initiates RRC re-establishment and performs cell selection while timer T311 is running. If the selected cell is an LTM candidate cell, the UE performs an LTM cell switch to the selected LTM candidate cell.

Proposal 2	If the selected cell is not an LTM candidate cell, the UE continues with an RRC re-establishment procedure.

Proposal 3	As in CHO, LTM fast recovery is controlled by a single parameter e.g. upon failure detection, if ‘attempt-LTM’ is included in LTM-Config-r18 the UE performs LTM fast recovery, otherwise it performs RRC re-establishment.

Proposal 4	As in CHO, the UE is only allowed to perform LTM fast recovery one time (no need to define a counter for a maximum number of fast recovery attempts).

Proposal 5	When the UE applies the stored LTM candidate configuration due to an LTM fast recovery, the UE deletes LTM related configuration(s), so LTM fast recovery only happens one time.

Proposal 6	The UE always performs random access when LTM cell switch is triggered during LTM fast recovery.

Proposal 7	When the UE configured with LTM continues with an RRC re-establishment, the UE removes the LTM related configurations (LTM candidate cell configurations, LTM related measurement configuration and TA establishment configuration).

Proposal 8	When the parameter controlling LTM fast recovery (e.g. ‘attempt-LTM’) is not configured, the UE perform the same actions as when ‘attempt-CHO’ is not configured.

Proposal 9	When the selected cell while timer T311 is running is not an LTM candidate cell, the UE perform the same actions as when the selected cell while timer T311 is running is not a CHO candidate cell.

# References

R2-2308435, RRC running CR for LTM, Ericsson, 3GPP TSG-RAN WG2 Meeting #123, Toulouse, France, 21st – 25th August 2023

R2-2308437, Discussion on remaining RRC open issues for LTM, Ericsson, 3GPP TSG-RAN WG2 Meeting #123, Toulouse, France, 21st – 25th August 2023

RP-231475, Revised WID on Further NR mobility enhancements, MediaTek, Apple