3GPP TSG-RAN WG2 Meeting #123	R2-2308595

Toulouse, France, Aug 21 – 25, 2023

Title		Discussion on higher layer aspects for sidelink positioning

Source	LG Electronics

Document for	Discussion and Decision

Agenda Item	7.2.2

WID/SID	NR_pos_enh2

# 1	Introduction

In this paper, we discuss higher layer aspects for sidelink positioning.

# 2	Discussion

## 2.1	SLPP session management

### 2.1.1	SLPP session ID

In RAN2#122 meeting [4], RAN2 did not make an agreement for SLPP session ID. In email discussion [6], RAN2 is discussing the need of session ID.

Agreements:

FFS how session IDs are managed between multiple UEs.

It may not be feasible to distinguish SLPP session only using transaction ID. One anchor UE can be involved in multiple sessions to support one and more target UEs, where different target UEs may use valid transaction ID within its own session. Moreover, multiple session can be triggered from one target UE. In complicated situation, it is difficult for anchor UE to distinguish each session from mixed transaction IDs.

By introducing of SLPP session ID, UE can effectively distinguish mixed sequences of transactions occurring in multiple different SLPP sessions from multiple different UEs, then UE can properly check validation of associated transaction within distingushed session. We think explicit SLPP session ID is necessary for sidelink positioning. Therefore, RAN2 at this meeting should confirm to introduce SLPP session ID at least for UE-only sidelink positioning operation.

Proposal 1.	SLPP sessions ID should be introduced and added in SLPP header.

### 2.1.2	Multiple sessions

In RAN2#121bis-e meeting [7], RAN2 agreed that a single SLPP session is created to support a single location request.

Agreements:

R2 agree that for session-based SLPP, a SLPP session is used among UEs in PC5-only case in order to obtain location related measurements/location estimates, to transfer assistance data, or to exchange of capabilities.

RAN2 agree that for session-based SLPP, a single SLPP session is created to support a single location request at least in case of a single target UE; FFS how sessions work if there are multiple target UEs in a single location request.

According to TS 37.355, a single LPP session is used to support a single location request, and multiple LPP sessions can be used between the same endpoints to support multiple different location requests. But there is no statement that multiple LPP sessions can be created to support a single location request. LPP is point-to-point protocol between UE and LMF, and it is assumed that UE is always connected to LMF in Uu-based positioning. Technically, there is no significant need to use multiple LPP sessions for a single location request.

However, SLPP is point-to-multipoint between multiple UEs, moreover, it cannot be assumed that UE is always connected to all other UEs in sidelink positioning due to UE’s dynamic nature. Thus, a single SLPP session cannot be considered as stable in sidelink positioning. For instance, if anchor UE moves outside of PC5 coverage, positioning result is not guaranteed. An ongoing SLPP session might be needed to be modified e.g. to find new anchor UE, which would inevitably increase latency. It is evident that multiple SLPP sessions offer several advantages. By establishing multiple SLPP sessions, it becomes possible to establish connections with multiple UEs, taking advantage of their diverse capabilities and accommodating dynamic situations. The ability to transition quickly and dynamically between multiple SLPP sessions can help reduce latency that may arise from session modification or reestablishment.

Strictly speaking, “a single LPP/SLPP session is used/created to support a single location request” does not mean that multiple sessions cannot be supported to support a single location request. In LPP case, So, LMF can invoke additional location request to create another LPP session up to LMF implementation, if needed. In SLPP case, on the other hand, SLPP protocol should be able to create one and more SLPP sessions upon a single location request without additional request by upper layer, which needs to specify UE operation in standard.

Proposal 2.	Multiple SLPP sessions can be created to support a single location request.

### 2.1.3	Explicit session operation/management signaling

In email discussion [6], RAN2 is discussing the need of introducing explicit session operation/management signaling and majority view is that explicit session operation/management signaling is not necessary.

Nevertheless, we still see the benefit of explicit session management signaling. For example, UE may store session information from SLPP message to use it anytime during session is activated and progressing, and to check if next message is valid. If anchor UE does not know session status, e.g. deactivation, pause or release at the right time, UE keeps to store session information at least until session expired, which could impact on power consumption and UE complexity. Fast session control can be simply and clearly achieved by explicit session operation/management signaling.

We also see some explicit signaling can be joint with SLPP message, in this case, explicit signaling can simply add an IE in the SLPP header, same as SLPP session ID. It does not introduce significant overhead, meanwhile, enable to taking an advantage of explicit signaling.

Proposal 3.	Explicit session operation/management signaling is introduced and added in SLPP header.

## 2.2	SLPP reliable transport

In RAN2#122 meeting [4], RAN2 discussed on SLPP reliable transport and agreed to support it for unicast, but FFS for groupcast.

Agreements:

SLPP over PC5-U/Uu will support reliable transport for at least unicast. FFS groupcast.

For unicast, SLPP messages are transmitted from one transmitting UE to one receiving UE using a dedicated unicast connection. This is the same as LPP point-to-point communication. So, LPP reliable transport mechanism (i.e. LPP acknowledgement and retransmission) can be utilized for ensuring reliable transmission of SLPP messages in the unicast scenario.

For groupcast, SLPP messages are transmitted from one transmitting UE to multiple receiving UEs via a groupcast connection. Naturally, LPP point-to-point reliable transport functionality may not be suitable for groupcast. The observed problem is sporadic transmission failures from different UEs during SLPP groupcast communication. There is a need to address sporadic failures in groupcast.

Proposal 4.	SLPP over PC5-U/Uu will support reliable transport for groupcast.

We propose three approaches for groupcast reliable transport:

- Unicast retransmission on failed UE

: by leveraging the unicast retransmission mechanism, this approach aims to combat sporadic transmission failures encountered during SLPP groupcast transmission. It allows the target UE to specifically address and retransmit the failed message to the specific anchor UE, potentially improving the overall reliability of the communication.

- Selective acknowledgment detection on the transmitting UE

: the transmitting UE resends failed SLPP messages via groupcast and considers successful groupcast transmission based on combination of all previous acknowledgments.

- Duplication detection on the receiving UE

: the receiving UE stores SLPP messages from the transmitting UE and performs acknowledgment duplication detection. By detecting the duplication of acknowledgments based on the sequence numbers, this mechanism on the receiving side can help combat sporadic transmission failures in SLPP groupcast transmission.

Proposal 5.	RAN2 to discuss following approaches for groupcast reliable transport:

- Unicast retransmission on failed UE

- Selective acknowledgment detection on the transmitting UE

- Duplication detection on the receiving UE

## 2.3	Anchor UE selection

In RAN2#122 meeting [4], RAN2 discussed on anchor UE selection and seven parameters were listed, as below;

Agreements:

Anchor UE selection is supported by information about the candidate anchor UEs.  At least the following list can be discussed for use in anchor UE selection:

1.	UE roles

2.	Supported positioning method

3.	In coverage or not

4.	RSRP

5.	LOS/NLOS

6.	Location

7.	PLMN

A normative requirement on which anchor UEs to select (e.g., ranking) will not be specified.

RAN2 impact of this information to be determined.

FFS which information would be determined statically/dynamically.

For now, RAN2 should decide (1) what parameters are used for anchor UE selection and (2) which procedure is used for parameter transfer.

### 2.3.1	What parameters are used for anchor UE selection?

All listed parameters would be useful for anchor UE selection. Despite the existence of various capabilities and parameters that can affect signalling overhead, it is necessary to include all relevant and useful capabilities and parameters in the SLPP request/provide capabilities message. This inclusion is crucial for achieving a successful sidelink positioning operation by appropriate anchor UE.

Among listed parameters, 6. Location is not clear. Location information can be divided into following sub-parameters.

- Ability to provide location information

: Anchor UE’s location information is necessary to perform location calculation. Unlike gNB, the decision whether to provide UE location information can be left to UE's capability due to privacy (it may depend on UE type and preference).

- Accuracy/integrity of location information

: Since the absolute location is calculated based on the relative positions between anchor UEs and target UE, any inaccuracies in anchor UE's location will directly impact the accuracy of the resulting estimated location for target UE. Thus, ensuring the accuracy of the anchor UE's location is crucial for achieving more precise location estimation.

- Velocity and direction information

: For V2X use cases (i.e. moving UEs), selecting an anchor UE with a velocity and direction similar to the target UE can enhance the success rate of positioning by minimizing the chances of the anchor UE moving out of PC5 coverage between itself and the target UE during sidelink positioning operation.

- Zone ID

: Zone ID can be utilized to provide information about the distance range between UEs, which can be provided from zoneConfig in SIB21 or SIB26. By considering the Zone ID and selecting UEs within a reasonable distance range, the likelihood of maintaining reliable sidelink positioning can be increased.

In addition, various types of UE can be involved in sidelink positioning such as ‘moving UE’, ‘stationary UE’, ‘normal UE’, ‘RSU-type UE’, etc. It is advantageous to prioritize the selection of 'stationary UE,' 'public/infrastructure UE,' and 'RSU-type UE' over 'moving UE' and 'normal UE'. This preference stems from the benefits associated with the type of UE, such as their fixed and known locations, which contribute to higher positioning accuracy and reduced positioning failures.

Additionally, RAN2 can kindly ask to RAN1 on additional AS parameters for anchor UE selection e.g., synchronization source/error information, supported frequency ranges for SL-PRS, etc.

As a result, we made following proposal for anchor UE selection parameters:

Proposal 6.	Anchor UE selection is supported by following information:

- UE roles

- Supported positioning method

- In coverage or not

- RSRP

- LOS/NLOS

- Location

  - Ability to provide location information

  - Accuracy/integrity of location information

  - Velocity and direction information

  - Zone ID

- PLMN

- Type of UE

- RAN2 to ask RAN1 on additional AS parameters for anchor UE selection.

### 2.3.2	Which procedure is used for parameter transfer?

Each parameter can be included in either/both discovery procedure (i.e. discovery metafield) or/and SLPP procedure (i.e. SLPP request/provide capability messages).

For this, we think there are two considerations:

- Session-based operation and session-less operation

: It is efficient to include the parameters for session-less operation in discovery procedure. E.g., UE roles and supported positioning method are essential parameters regardless of session-based or -less operation.

- Static character and dynamic character

: Static parameters (e.g. UE roles, supported positioning method, type of UE) are unchangeable (at least not changed frequently) UE capabilities, so static parameters would be transferred on discovery procedure before positioning operation. That could avoid SLPP capability exchange. On the other hand, dynamic parameters (e.g. in coverage or not, RSRP, LOS/NLOS, Location, PLMN) are changeable, so dynamic parameters would be transferred on SLPP procedure before and during positioning operation.

As a result, we made following proposal for anchor UE selection procedure:

Proposal 7.	Anchor UE selection is supported by following procedures:

- Discovery procedure - including UE roles, Supported positioning method, Type of UE

- SLPP capability exchange procedure - including In coverage or not, RSRP, LOS/NLOS, Location, PLMN

## 2.4	SLPP between UE and LMF

In RAN2#122 meeting [4], RAN2 agreed SLPP carried over NAS is used between UE and LMF.

Agreements:

SLPP carried over NAS is used between UE and LMF. FFS on how to manage the session/transaction.

In email discussion [6], RAN2 is discussing the need of introducing explicit SLPP session ID in case of LMF-involved sidelink positioning and majority view is that explicit SLPP session ID is not necessary in case of LMF-involved sidelink positioning, as same as Uu-based positioning, because NAS can distinguish SLPP session with routing IDs and correlation IDs associated with location service requests.

According to agreement, SLPP will be used between UEs as well as between UE and LMF. Duplication of specification can be avoided i.e. SLPP messages between UE and LMF will be defined in SLPP specification (TS 38.355). It is naturally preferred to have unified SLPP format even between different endpoints i.e. common header and message structures.

So, even though SLPP session ID is not necessary for LMF-involved sidelink positioning, there is no harm to use SLPP session ID for LMF-involved sidelink positioning. Also, it could enable unified solution and easy session management.

Proposal 8.	Unified structure is preferred in both cases, SLPP between UEs (i.e. UE-only sidelink positioning) and SLPP between UE and LMF (i.e. LMF-involved sidelink positioning).

# 3	Conclusion

Proposal 1.	SLPP sessions ID should be introduced and added in SLPP header.

Proposal 2.	Multiple SLPP sessions can be created to support a single location request.

Proposal 3.	Explicit session operation/management signaling is introduced and added in SLPP header.

Proposal 4.	SLPP over PC5-U/Uu will support reliable transport for groupcast.

Proposal 5.	RAN2 to discuss following approaches for groupcast reliable transport:

- Unicast retransmission on failed UE

- Selective acknowledgment detection on the transmitting UE

- Duplication detection on the receiving UE

Proposal 6.	Anchor UE selection is supported by following information:

- UE roles

- Supported positioning method

- In coverage or not

- RSRP

- LOS/NLOS

- Location

  - Ability to provide location information

  - Accuracy/integrity of location information

  - Velocity and direction information

  - Zone ID

- PLMN

- Type of UE

- RAN2 to ask RAN1 on additional AS parameters for anchor UE selection.

Proposal 7.	Anchor UE selection is supported by following procedures:

- Discovery procedure - including UE roles, Supported positioning method, Type of UE

- SLPP capability exchange procedure - including In coverage or not, RSRP, LOS/NLOS, Location, PLMN

Proposal 8.	Unified structure is preferred in both cases, SLPP between UEs (i.e. UE-only sidelink positioning) and SLPP between UE and LMF (i.e. LMF-involved sidelink positioning).

# 4	References

- R2-2306334, SLPP session management and operation – LG Electronics

- R2-2306335, SLPP reliable transport functionality – LG Electronics

- R2-2306336, Sidelink positioning parameters for Anchor UE selection – LG Electronics

- R2-2306544, Report from session on positioning and sidelink relay (RAN2#122) – Session Chair (MediaTek)

- TS 23.586, Architectural Enhancements to support Ranging based services and Sidelink Positioning (Release 18)

- R2-2307660, Summary of [Post122][402][POS] SLPP session handling (Intel)

- R2-2304204, Report from session on positioning and sidelink relay (RAN2#121bis-e) – Session Chair (MediaTek)