3GPP TSG-RAN WG2 Meeting #123											   R2-2308818

Toulouse, France, 21-25 Aug, 2023

Source: 	Xiaomi

Title:  	Discussion on RRC aspects for L1/L2-Triggered Mobility

Agenda Item:	7.4.2.2

Document for:	Discussion and Decision

Introduction

According to the WID [1] of the Rel-18 mobility enhancements as quoted below, one objective of L1/L2 based inter-cell mobility is included.

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Note 2: Only SSB-based L1 measurement is supported in this release. - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 3: FR2 specific enhancements are not precluded, if any. Note 4: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG, prioritizing MCG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In this contribution, we provide our understandings for RRC aspects of LTM.

Discussion

  - Scenarios

Note 4: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG, prioritizing MCG

In the Revised WID [1] on Further NR mobility enhancements, the scenarios of LTM has been clarified:

SCG LTM has been de-prioritized in the revised WID but not removed. SCG LTM in Rel-18 is only focus on the PSCell change without MN involvement. Actually, some principle for MCG LTM also can be directly applied to SCG LTM. It may be better to continue to consider SCG LTM for now. If too much extra work is needed to support SCG LTM, SCG LTM can be removed in Rel-18.

Observation 1: Some principle for MCG LTM also can be directly applied to SCG LTM in Rel-18.

Proposal 1：RAN2 can prioritize MCG LTM but not remove SCG LTM for now.

For the RRC modelling of the LTM candidate configuration, RAN2 has agreed to use one RRCReconfiguration message for each candidate target configuration. But how to configure MCG LTM and SCG LTM is FFS. Up to now, LTM for MCG with SCG has not been supported and MCG LTM and SCG LTM is done independently. Hence, one LTM candidate configuration can contain either the MCG LTM configuration or the SCG LTM configuration (not both).

Proposal 2: One LTM candidate configuration can contain either the MCG LTM configuration or the SCG LTM configuration (not both).

For Rel-18 LTM, there is two types of LTM have been provided by some companies:

- One-shot LTM: UE needs to release the LTM configurations after cell switch

- Subsequent LTM: UE keep the LTM configurations for subsequent LTM

Whether a subsequent LTM cell switch procedure is explicitly indicated to the UE can be discussed. In the WID [1], maintenance for multiple candidate cells to allow fast application of configurations for candidate cells has been captured. UE should maintain LTM configurations for subsequent LTM and the subsequent LTM cell switch should be a default behaviour for Rel-18 LTM. If subsequent LTM is not needed, network can explicitly release the configurations.

Proposal 3：The subsequent LTM cell switch should be a default behaviour for Rel-18 LTM. And a subsequent LTM cell switch does not need to be explicitly indicated to the UE.

For LTM, one scenario that UE comes back to the initial source cell after the initial LTM also can be considered. How to support the scenario need to be decided by RAN2. The similar issue has been discussed in subsequent CPAC, and RAN2 assumes that if the UE need to be able to return to a current SCG by conditional procedure, then the network could explicitly configure a candidate configuration for that cell. Hence, the similar principle can be reused for LTM. If the UE needs to be able to return to the initial source cell during subsequent LTM, the network could explicitly configure a LTM candidate configuration for the initial source cell.

Proposal 4: If the network wants to use the initial source cell as a LTM candidate cell, the network could explicitly configure a LTM candidate configuration for the initial source cell.

  - Compliance check

In the previous meeting [2], RAN2 has reached the following agreement for early decoding and early validity check:

The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed.

NOTE X: It is up to UE implementation whether the compliance check for an RRCReconfiguration message received as part of an LTM-Config IE is performed upon the reception of the message or during an LTM cell switch procedure (when the message is required to be applied).

Editor’s Note: It is FFS whether further actions are needed from the UE when a reconfiguration failure is detected because of an early compliance check of an LTM candidate

For LTM, when to perform compliance check of the candidate configuration can be discussed. For legacy CHO and CPAC, it is up to UE implementation whether the compliance check for an RRCReconfiguration received as part of ConditionalReconfiguration is performed upon the reception of the message or upon CHO, CPA and CPC execution. Because the UE can perform compliance check and apply the candidate cell configuration simultaneously, there is no extra interruption in the processing when the UE start “CHO, CPA and CPC”. Therefore, the same principle for conditional reconfiguration can be reused for LTM, as the NOTE X in the running CR for LTM. The following are extracted from the running 38.331 CR [3] for the compliance check:

Observation 2: As captured in the running 38.331 CR, it is up to UE implementation whether the compliance check for an RRCReconfiguration message received as part of an LTM-Config IE is performed upon the reception of the message or during an LTM cell switch procedure.

For the FFS “whether further actions are needed from the UE when a reconfiguration failure is detected because of an early compliance check of an LTM candidate” captured as EN, there are two options for UE behaviour can be discussed:

- Option 1: If compliance check fails, UE performs RRC re-establishment

- Option 2: UE can inform the network that UE identifies an inconsistency and indicate the configuration ID associated with the candidate configuration triggering compliance check failure and network can reconfigure the configuration.

Option 1 is the legacy UE behaviour for the reconfiguration failure and compliance check failure is an infrequent case. Hence, some optimizations like option 2 is not needed.

Proposal 5: If an early compliance check fails, UE should follow the legacy UE behaviour and trigger the RRC re-establishment.

For LTM, the reference configuration and the LTM candidate configuration can be indicated by the RRCReconfiguration message as a part of the LTM configuration. And the complete LTM candidate cell configuration is generated by UE based on the reference configuration and the LTM candidate configuration, which is also one RRCRecofniguration message. RAN2 can clarify to perform compliance check for which configurations in LTM. In the current procedure, UE may check the reference configuration and LTM candidate cell configuration separately. However, upon cell switch, UE applies the complete LTM candidate configuration. For LTM, a more direct approach is to perform the compliance check for the complete LTM candidate cell configurations

Proposal 6: For LTM, UE performs the compliance check for the complete LTM candidate cell configurations.

  - Release of LTM configurations

In RAN2#119bis-e [4], RAN2 has assumed that candidate cell configuration can only be modified/released by Network (FFS later whether some optimization should be applied e.g. for release). Some optimizations have be provided in previous meeting, e.g. UE autonomously releases candidate cell configuration based on a timer/conditions. These UE-based methods for release might cause that UE and network have different understanding of candidate cell configuration stored in UE.

For a UE in RRC_CONNECTE, the network can always modified/released the LTM configuration through the explicit indication (i.e. ltm-CandidateToReleaseList and ltm-CandidateToAddModList). For some scenarios of mobility (e.g. HO, CHO/CPAC execution, SCG release and so on), the network can explicitly release the invalid LTM configurations, if needed. Hence, in RRC_CONNECTED the LTM configurations can only be modified/released by the network.

Observation 3: For the UE in RRC connected mode, the network can explicitly release the invalid LTM configurations via Rel-18 ltm-CandidateToReleaseList IE, if needed.

Proposal 7: In RRC_CONNECTED, the LTM configurations can only be modified/released by the network.

For a UE going to RRC_IDLE/RRC_INACTIVE, it is nature to release all LTM configurations.

Proposal 8: Upon going to RRC_IDLE or RRC_INACTIVE, UE should release all LTM configurations.

  - Failure recovery by LTM

In previous meetings [3], RAN2 introduced a RRC layer timer to supervise LTM. And if the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment.

And RAN2 has intended to support fast recovery to a candidate cell by LTM execution, at RLF or LTM execution failure (for MCG). RAN2 has agreed the UE can also execute any L3 handover command sent by the network while configured with LTM candidate cells. Similar to RLF and LTM failure, fast recovery by LTM execution also can be supported for legacy handover failure.

Proposal 9: At handover failure (i.e. re-configuration with sync failure of the MCG), RAN2 can support fast recovery to a LTM candidate cell by LTM execution, if configured.

In Rel-16, upon failure, the UE will perform CHO if the selected cell is CHO candidate cell and it is the first time of recovery and if the network allows CHO based recovery. An indicator (i.e. attemptCondReconfig-r16) is included in the Rel-16 IE ConditionalReconfiguration, which indicates whether the UE shall perform conditional reconfiguration if selected cell is a target candidate cell and it is the first cell selection after failure.

Similar to Rel-16 CHO based recovery, LTM based recovery can also only be performed for the first time of recovery and an indicator can be introduced in the LTM-Config to identify whether UE can perform LTM execution for failure recovery.

Proposal 10: Similar to CHO based recovery, upon failure, UE can only execute MCG LTM if the selected cell is a LTM candidate cell and it is the first time of recovery and if the network allows LTM based recovery.

Proposal 11：An indicator provided by network can be introduced in the LTM-Config to identify whether UE can perform LTM execution for failure recovery.

  - Coexistence of LTM and CHO/CPAC

In Rel-18, only the intra-CU LTM is supported. CHO and CPAC can be configured for the pre-configured inter-CU mobility, and UE can autonomously execute PCell change and/or PSCell change/addition.

In RAN2#121bis-e [2], RAN2 has agreed “while configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network.” RAN2 can also discuss whether LTM and CHO/CPAC can be configured simultaneously.

Proposal 12: RAN2 can discuss whether LTM and CHO/CPAC can be configured simultaneously.

If LTM and CHO/CPAC can be configured simultaneously, when LTM or CPAC is triggered, UE behaviours can be discussed.

- When CHO/CPAC is triggered, UE will not autonomously release the LTM configurations and the RRCReconfiguration as the CHO/CPAC candidate configuration can modified/released the LTM configurations, which is align with P7 provided in the contribution.

- When UE performs LTM cell switch procedure, how to handle CHO/CPAC configuration can be discussed.

Proposal 13: If LTM and CHO/CPAC can be configured simultaneously, whether LTM Cell switch can trigger the removal of CHO/CPAC can be discussed.

Conclusions

According to the analysis given above, we have the following observations and proposal:

Scenarios

Observation 1: Some principle for MCG LTM also can be directly applied to SCG LTM in Rel-18.

Proposal 1：RAN2 can prioritize MCG LTM but not remove SCG LTM for now.

Proposal 2: One LTM candidate configuration can contain either the MCG LTM configuration or the SCG LTM configuration (not both).

Proposal 3：The subsequent LTM cell switch should be a default behaviour for Rel-18 LTM. And a subsequent LTM cell switch does not need to be explicitly indicated to the UE.

Proposal 4: If the network wants to use the initial source cell as a LTM candidate cell, the network could explicitly configure a LTM candidate configuration for the initial source cell.

Compliance check

Observation 2: As captured in the running 38.331 CR, it is up to UE implementation whether the compliance check for an RRCReconfiguration message received as part of an LTM-Config IE is performed upon the reception of the message or during an LTM cell switch procedure.

Proposal 5: If an early compliance check fails, UE should follow the legacy UE behaviour and trigger the RRC re-establishment.

Proposal 6: For LTM, UE performs the compliance check for the complete LTM candidate cell configurations.

Release of LTM configurations

Observation 3: For the UE in RRC connected mode, the network can explicitly release the invalid LTM configurations via Rel-18 ltm-CandidateToReleaseList IE, if needed.

Proposal 7: In RRC_CONNECTED, the LTM configurations can only be modified/released by the network.

Proposal 8: Upon going to RRC_IDLE or RRC_INACTIVE, UE should release all LTM configurations.

Failure recovery by LTM

Proposal 9: At handover failure (i.e. re-configuration with sync failure of the MCG), RAN2 can support fast recovery to a LTM candidate cell by LTM execution, if configured.

Proposal 10: Similar to CHO based recovery, upon failure, UE can only execute MCG LTM if the selected cell is a LTM candidate cell and it is the first time of recovery and if the network allows LTM based recovery.

Proposal 11：An indicator provided by network can be introduced in the LTM-Config to identify whether UE can perform LTM execution for failure recovery.

Coexistence of LTM and CHO/CPAC

Proposal 12: RAN2 can discuss whether LTM and CHO/CPAC can be configured simultaneously.

Proposal 13: If LTM and CHO/CPAC can be configured simultaneously, whether LTM Cell switch can trigger the removal of CHO/CPAC can be discussed.

Reference

[1] RP-231475, MediaTek Inc., “Revised WID on Further NR mobility enhancements”

[2] R2-2306553, “Report of 3GPP TSG RAN WG2 meeting #121bis-e”

[3] R2-230xxxx, “RRC running CR for LTM”

[4] R2-2211101, “Report of 3GPP TSG RAN WG2 meeting #119bis-e”