3GPP TSG-RAN WG2 #123bis	R2-2310647

Xiamen, China,  October 9th – 13th 2023

Source:		NEC

Agenda item:		7.4.3	Subsequent CPAC

Title:		Discussion on subsequent CPAC

Document for:		Discussion and decision

# 1. Introduction

At RAN2 #123 meeting, it was agreed that it is also candidate SN generates execution conditions for subsequent CPC in case of MN-initiated subsequent CPAC:

Proposal 3 (option2): For MN-initiated subsequent CPAC, the execution condition configuration is provided as following:

- MN generates the execution conditions (A4 event) for initial CPAC execution, and the measID refers to the measurement configuration associated with MCG;

- candidate SN generates the execution conditions (A3/A5 event)  for subsequent CPC execution, and the measID refers to the measurement configuration associated with SCG.

And RAN2 also confirmed the support of SA3 sk-counter list solution for secondary key update during subsequent CPAC:

Will support the SA3 solution, i.e. update of Sk-counter at inter-SN-mobility, based on pre-configured multiple Sk-counter. UE need to know when Sk counter need to change.

In this paper, we provide our view on remaining issue of subsequent CPAC.

# 2. Discussion

## 2.1. Security for subsequent CPAC

During last RAN2 meeting, RAN2 confirmed the support of the SA3 solution for security, i.e., update of sk-counter at inter-SN-mobility based on pre-configured multiple sk-counter.

To determine whether the subsequent CPC is inter-SN or intra-SN, one group-ID is associated with each candidate cell. If the UE identifies that the target cell belongs to a different group than the serving cell, the UE determines that the subsequent CPC is inter-SN, and selects the first unused entry in the sk-counter list that corresponds to the group-ID of the target cell to derive updated secondary key.

For detailed mechanism, SA3 provided three options in their CR [2], and the final decision would be made by collaboration with RAN2.

Option 1: The MN derives the corresponding KSN as described in Annex A.16 based on the corresponding SN Counter values. The KSN keys and the corresponding SN Counter values are sent to the SN from the MN. The SN shall store the KSN keys and the corresponding SN Counter values in its security context. The UE shall inform the MN which SN Counter value it used for KSN derivation when accessing the SCG of the target SN. The MN signals the received SN Counter value to the SN. The SN decides the KSN based on the received SN Counter value. Option 2: After the derivation of sequence of multiple distinct SN Counter values for each candidate SN, the SN Counter values are sent to the UE from the MN. The MN does not need to store the SN Counter values for each SNs after sending them. [Alt 1] When the UE accesses an SCG of a target SN, the MN receives the SN Counter value from the UE. The MN derives KSN using the received SN Counter value. The KSN is sent to the target SN from the MN. [Alt 2] When the UE accesses an SCG of a target SN, the target SN receives the SN Counter value from the UE. The target SN uses the SN Counter value to request the Ksn derivation from MN. Option 3: The MN derives the corresponding KSN as described in Annex A.16 based on the corresponding SN Counters values. The KSN keys are sent to the SN from the MN. The SN shall store the KSN keys in its security context. The SN chooses the first unused Ksn key for that UE to establish the security with the UE.

In our understanding, Option 1 and Option 3 are similar, where the SN choses one key from a list of security keys pre-provided to the SN from MN, and their difference is that SN choses the key based on sk-counter value from SN or just chose the next unused key. One drawback of Option1 and Option 3 is that if the UE does not access to candidate PSCells of one SN very often, some keys provided to SN may be unused in the end.

For Option 2, SN is provided with only one security key for each subsequent CPAC procedure, which would not have the issue of Option 1 and Option 3. And Alt 2 of Option 2 should be excluded because if the sk-counter value in SN RRCReconfigurationComplete message, since the RRC message may be ciphered, the SN is unable to obtain the sk-counter value, so this alternative does not work. Based on the above analysis, Option 2 Alt 1 is best option from our point of view. But during email discussion [Post123][046], it is controversial whether there is the necessity to report the sk-counter value to NW. Some companies think that this is not needed, while some other companies think that it is safer to report [3]. This can be pending for further RAN2 discussion.

Proposal 1: Among the options for security of subsequent CPAC provided by SA3, option2 Alt 1 with or without the UE reporting sk-counter value related part should be selected.

## 2.2. L2 handling for subsequent CPAC

There is the fundamental assumption that the handling of the reference configuration agreed for the LTM is to be reused for the subsequent CPAC basically. For the LTM, it was agreed that “Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration” in RAN2#121bis-e [4]. In addition, it was also confirmed that “only the replacement procedure (the “full config without L2 reset”) is supported for Execution of LTM cell switch” [4]. However, there are some differences between the LTM and the subsequent CPAC. For example, the subsequent CPAC involves the PDCP re-establishment at inter-SN CPC or the PDCP recovery at intra-SN CPC. Further discussions on how the LTM agreements can be reused for the subsequent CPAC are necessary. We understand that due to this situation, the current running CR for 38.331 captures the following editor’s notes:

Editor’s Note: FFS on whether to rely on the full configuration procedure as specified in 5.3.5.11 or new complete configuration procedure when the UE applies a complete configuration. Editor’s Note: FFS whether to restrict full configuration flag for subsequent CPAC candidate configuration if complete configuration procedure is used.

The simplest way is to reuse the existing full configuration procedure for the subsequent CPAC (i.e. just apply the complete configuration as new configuration). This can work well. However, there is possibility of lossy mobility at every PSCell change. Alternative way is to apply the radio resource configurations of the complete configuration, while the radio bearer configuration is handled like a delta configuration. There may be some additional optimization when the intra-SN CPC is performed based on the assumption that whether it is inter-SN CPC or intra-SN CPC can be distinguished by the group ID as discussed in 2.1. We suggest having further discussions on this issue.

Proposal 2: RAN2 to discuss how exactly the subsequent CPAC is performed based on the complete configuration.

The following part and section 2.3 are depending on how the subsequent CPAC is performed.

Assume that the delta configuration is applied to the complete candidate configuration for SCPAC, in case of inter-SN CPC, , the updated secondary key would trigger some L2 handing for SN terminated DRBs and SRBs. For example, RRC should configure the PDCP entity of the RBs with key for ciphering or integrity associated with the master key, and PDCP re-establishment and RLC reestablishment should be performed for those radio bearers. These behavior should be performed by the UE without RRC IE.

Proposal 3: Upon determination the subsequent CPC procedure is inter-SN CPCl, the UE needs to perform the following behavior:

- Update secondary key

- configure the PDCP entity of the SN-terminated DRBs and SRBs with key for ciphering or integrity associated with the updated secondary key

- PDCP re-establishment for the SN-terminated DRBs and SRBs

- RLC re-establishment for the SN-terminated DRBs and SRBs

## 2.3. Bearer termination point change during subsequent CPAC

In legacy (conditional or non-conditional) PSCell change/addition, bearer type of one DRB may be changed, i.e., from MN-terminated DRB to SN-terminated DRB, or vice visa. And whether one DRB is MN-terminated or SN-terminated is indicated by “keyToUse” configured for each DRB. More specifically, if the “keyToUse” is set as master, the DRB is MN terminated; if the “keyToUse” is set as secondary, the DRB is SN-terminated. The change of bearer termination point usually happens in case of inter-SN CPC, while it does not support in the legacy intra-SN CPC without MN involvement. On the other hand, during the subsequent CPAC, it may also happen in case of CPC within the same SN.

For one DRB, different L2 handling shall be performed in case of with and without a security key change due to a change or not change of bearer termination point. For example, in case of bearer termination point change, PDCP re-establishment and RLC re-establishment should be performed for the corresponding DRB. And in legacy (conditional) PSCell change/addition, this is indicated explicitly in RRCReconfiguration message.

However, for subsequent CPAC, whether the termination point of DRB is changed depends on UE’s moving trajectory. For example, one DRB is SN-terminated in candidate Cell #1 and Cell #2, and is MN-terminated in candidate Cell #3. In case of subsequent CPC from Cell #1 to Cell #2, the termination point of the DRB is not changed. But in case of subsequent CPC from Cell #3 to Cell #2, the termination point of the DRB is changed. Assume that the LTM solution for application of complete candidate configuration is reused for SCPAC,  it is impossible to configure L2 handling IE (e.g., resetablishPDCP) properly in RRCReconfiguration of the conditional reconfiguration associated with the candidate cell. Instead, the UE needs to determine whether the termination point of one DRB is changed or not based on “keyToUse”, and perform different L2 handling based on the determination result.

Proposal 4: During subsequent CPAC, the UE determines whether termination point of one DRB is changed or not, and performs different L2 handling for the cases of termination point changed and unchanged.

## 2.4. Co-existence and reference configurations

In the email discussion [Post123][054], the co-existence of MN-initiated subsequent CPAC, SN-initiated subsequent CPAC and intra-SN subsequent CPAC was discussed. The conclusion is to confirm the support of such co-existence [6]. On the other hand, there is one issue on the meaning of “only one reference configuration” for the co-existence scenario. The following two understandings are on the table and should be discussed further. Note that the understanding 2 is the majority view.

  - Understanding1: There is only one reference configuration for MN/SN initiated inter-SN SCPAC and SN-initiated intra-SN SCPAC.

  - Understanding2: There is only one reference configuration for each scenario, i.e., one reference configuration for MN/SN initiated inter-SN SCPAC and one reference configuration for SN-initiated intra-SN SCPAC.

The understanding 1 can save some signalling overhead and UE complexity to avoid two different reference configurations at a time. However, this needs some coordination between the procedures. On the other hand, the understanding 2 is simpler, as there is no need of coordination between the MN/SN-initiated inter-SN SCPAC and intra-SN SCPAC at least from the reference configuration perspective. Indeed, there is the trade-off between the signalling overhead and the flexibility (simplicity). If there is only one reference configuration among procedure as per the understanding 1, we rather think it is simple not to support the co-existence of inter-SN SCPAC and intra-SN SCPAC.

Proposal 5: RAN2 to agree with the understanding 2, i.e. there is only one reference configuration for each scenario (one for MN/SN initiated inter-SN SCPAC and the other for SN-initiated intra-SN SCPAC.

Related to the above issue, there is one more issue about which node (i.e. the MN or the source SN) is responsible for the reference configuration generation for SN initiated intra-SN SCPAC. Given that the understanding 2 is confirmed, it is natural that the source SN is responsible for the reference configuration generation for the intra-SN SCPAC.

Proposal 6: RAN2 to agree that the source SN generates the reference configuration for the intra-SN SCPAC

# 3. Conclusion

In this paper, we provide some consideration on remaining issues of subsequent CPAC, and the following proposals are given.

Proposal 1: Among the options for security of subsequent CPAC provided by SA3, option2 Alt 1 with or without the UE reporting sk-counter value related part should be selected.

Proposal 2: RAN2 to discuss how exactly the subsequent CPAC is performed based on the complete configuration.

Proposal 3: Upon determination the subsequent CPC procedure is inter-SN CPC, the UE needs to perform the following behavior:

- Update second key

- configure the PDCP entity of the SN-terminated DRBs and SRBs with key for ciphering or integrity associated with the updated secondary key

- PDCP re-establishment for the SN-terminated DRBs and SRBs

- RLC re-establishment for the SN-terminated DRBs and SRBs

Proposal 4: During subsequent CPAC, the UE determines whether termination point of one DRB is changed or not, and performs different L2 handling for the cases of termination point changed and unchanged.

Proposal 5: RAN2 to agree with the understanding 2, i.e. there is only one reference configuration for each scenario (one for MN/SN initiated inter-SN SCPAC and the other for SN-initiated intra-SN SCPAC.

Proposal 6: RAN2 to agree that the source SN generates the reference configuration for the intra-SN SCPAC.

# Reference

[1] Meeting report of RAN2 #123.

[2] S3-234164, Security for Selective SCG Activation

[3] R2-230xxxx, [Post123][046][feMob] subsequent CPAC security (Nokia)

[4] Meeting report of RAN2 #121bis-e

[5] R2-2309336, RRC running CR for subsequent CPAC in NR-DC

[6] R2-230xxxx, [Post123][054][feMob] Discussion on stage-2 signalling open issues