3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310888

Xiamen, China, 9th – 13th October 2023

Agenda Item:	7.4.2.1

Source:	Ericsson

Title:	Early sync and L1 measurements

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we address the remaining issues for what concern the early sync procedures and the L1 measurements for LTM.

# 2	Discussion

## 2.1	Remaining issues on early sync

### 2.1.1	Capturing early sync in 38.300

In the endorsed TS 38.300 in R2-2309335 [1] as outcome of the email discussion [Post123][053][feMob] Running CR 38300, the early sync procedures have been captured as separate procedures (one for the early UL sync and one for the early DL sync) each one in a different subsection. However, according to the inputs provided by the companies, is still unclear is these procedures will be captured separately or tight to the LTM feature.

The understanding is that these procedures are standalone, and they also may be used for other features that are not LTM. According to this, it would be beneficial is RAN2 can confirm this.

RAN2 to confirm that the early sync procedures are captures in a dedicated section (which is not a subsection of LTM) in TS 38.300.

Another concern that arises from the email discussion, was the terminology used for the early DL sync. This is because RAN1 does not really call this procedure as “early DL sync” but rather “early TCI activation”. According to this, a proposal would be to rename this section as “early TCI activation”.

The section on the early DL sync is renamed as “Early TCI activation”.

### 2.1.1	TA validity and time alignment timer(s)

In a legacy RA procedure, the UE transmits a preamble and receives the TA value in the RAR, based on which the UE starts a Time Alignment timer with a value configured in RRC. However, in LTM, when RAR is not configured for TA establishment, the TA value is received at the S-DU for LTM candidate cell and is only to be provided to the UE when the LTM cell switch is triggered. Thus, as the UE does not receive the TA value at the moment or close to the moment the TA value is calculated, it makes sense to leave the monitoring of its validity to the network.

In fact, RAN2 also already agreed that there is no need for the UE to check the validity of a TA value as this one is monitored by the network. Nevertheless, even if the UE does not need to monitor the validity of the TA value, still there could be a possible misalignment between the length of the Time Alignment timer and the actual validity of the TA value, as this one is calculated before an LTM cell switch procedure is triggered.

In fact, upon reception of the LTM cell switch command, the UE applies the TA value. As the UE is switching to another cell, it would in principle need to start the time alignment timer for the new serving cell. However, the timer value configured in RRC typically considers the time from when the UE transmitted the PRACH preamble until the time the TA is considered not valid anymore. For LTM however, the PRACH preamble transmission could have been much earlier, so starting the timer at the time of receiving the LTM cell switch command could lead to a situation where the UE considers the TA as valid for too long time in the target cell. To address this issue, there could be at least two options for how to solve it with specifications:

- Option a) An indication of the Time alignment timer (e.g., remaining or elapsed time alignment time, delta, or something equivalent) is included in the LTM cell switch command;

- Option b) The UE monitors the time in which transmits the PRACH preamble and the time in which it receives the TA value in the LTM cell switch command, so that it reduces that difference form the time alignment timer value configured in RRC.

However, another approach is that the TAT is started upon LTM switch. If the gNB judges that it is important to have exact knowledge of when the TAT will expire in the UE, the gNB sends a TAC MAC CE to the UE after the LTM switch. Note, the target will anyway eventually do this as it is normal gNB behaviour to issue TAC MAC CEs to update the TA value for the UE, or just send periodic deltas = 0 to restart the TAT (if used).

- Option c) The UE starts the TAT at LTM switch. Leave to gNB implementation to send a TAC MAC CE to get aligned with the UE in the TAT duration.

RAN2 to discuss solutions for the setting of the Time Alignment timer when the UE receives the TA value in the LTM cell switch command:

  - Time alignment timer indication (or something equivalent e.g., delta) included in the LTM cell switch command.

  - UE monitors the time difference between PRACH preamble transmission and reception of TA value in the LTM cell switch command, and this difference is subtracted form the time alignment timer value configured in RRC.

  - Leave to gNB implementation, e.g., the target sends a TAC MAC CE after the LTM switch.

## 2.2	Remaining issues on L1 measurements

About the L1 measurements, still one open issue that have not been discussed so far is which node generates the LTM CSI resource configuration. So far the following agreement have been reached about the LTM CSI measurements, but only clarify how the RS configurations are acquired by the CU.

The RS configuration is provided to the UE per LTM candidate cell.

RAN2 assumes that Each candidate DU needs to know the RS configuration of each candidate DUs in order to provide the LTM candidate configuration.

RAN2 assumes that The CU transmits to each C-DU the RS configuration of S-DU (if this is an LTM candidate cell) and/or other C-DUs, to generate the corresponding L1 configuration for LTM.

RAN2 assumes C-DU generates the RS configuration and send to the CU. The CU transmits to the Source DU the RS configuration per LTM candidate cell and the associated LTM candidate (when the CU receives LTM candidate configuration(s) from the C-DU). It is up to RAN3 whether the RS configuration is sent before (or at the same time of) the C-DU creates the LTM candidate configuration (and whether is semi-statis or UE associated).

When it comes to the generation of the LTM CSI resource configuration, a straightforward solution would for the CU to generate such configuration. This is because the CU has already the knowledge of all the RS configurations collected from the serving DU and the LTM candidate DUs. Also, by doing this we avoid having too much signalling overhead on the network since the interaction between CU and DU is simplified. Therefore, we propose:

The LTM CSI resource configuration is generated by the CU.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm that the early sync procedures are captures in a dedicated section (which is not a subsection of LTM) in TS 38.300.

Proposal 2	The section on the early DL sync is renamed as “Early TCI activation”.

Proposal 3	RAN2 to discuss solutions for the setting of the Time Alignment timer when the UE receives the TA value in the LTM cell switch command:

a.	Time alignment timer indication (or something equivalent e.g., delta) included in the LTM cell switch command.

b.	UE monitors the time difference between PRACH preamble transmission and reception of TA value in the LTM cell switch command, and this difference is subtracted form the time alignment timer value configured in RRC.

c.	Leave to gNB implementation, e.g., the target sends a TAC MAC CE after the LTM switch.

Proposal 4	The LTM CSI resource configuration is generated by the CU.

# 4	References

R2-2309335, 38.300 running CR for introduction of NR further mobility enhancements, MediaTek, Vivo, 3GPP TSG-RAN WG2 Meeting #123, Toulouse, France, August 2023.