3GPP TSG-RAN WG2 Meeting #123bis	R2-2311273

Xiamen, China, Oct 9-13, 2023

Agenda item:	8.1

Source: 	Vice Chairman (ZTE Corporation)

Title:	Report from Break-out session on NR-NTN and IoT-NTN

Document for:	Approval

Organizational

- All organization emails and notes will be shared over the following email discussion throughout the meeting:

[AT123bis][300] Organizational – NR-NTN and IoT-NTN session

Scope:

    - Share plans for the meeting and list of ongoing email discussions for the sessions related to NR-NTN and IoT-NTN

    - Share meetings notes and agreements for review and endorsement

Schedule/Plan

|  | Main room | Brk 1 room | Brk 2 room | Brk 3 room |
| --- | --- | --- | --- | --- |
| Monday October 9 |  |  |  |  |
| 09:00 – 10:30 | [1], [2], [3], [7.0] R18 common (Diana) NR18 Network Energy Saving [1] (Diana) | Breakout to start after coffee break | Breakout to start after formal opening of meeting in main room: NRLTE1516 Pos (Nathan) NR17 Pos (Nathan) NR17 SL Relay if time (Nathan) |  |
| 11:00 – 13:00 |  | MUSIM [1] (Erlin) |  |  |
| 14:30 – 16:30 | NR151617 (Mattias) | 14:30-15:00 - NR18 MIMO evo [0.75] NR151617 UP (Diana) NR18 Closed WIs early items NR18 MT-SDT(Diana) IDC (Yi) (email discussion only) NCR(Sasha) (email discussion only) | NRLTE1516 V2X/SL (Kyeongin) NR17 SL (Kyeongin) |  |
| 17:00 – 19:00 | NR151617 (Mattias) | NR18 eQoE [1] (Dawid) NR18 fCovEnh [0.5] (Eswar) | NR18 SL evolution [1] (Kyeongin) |  |
| Tuesday October 10 |  |  |  |  |
| 08:30 – 10:30 | NR18 feMob [2] (Johan) | NR 18 MBS [0.5] (Dawid) MBS TEI 18 | NR18 SL Relay [1.5] (Nathan) - 7.9.1 Organizational - 7.9.2 UE-to-UE - 7.9.3 Service continuity if time |  |
| 11:00 – 13:00 | NR18 Mobile IAB [0.5] (Johan) NR18 LP WUS [0.5] (Johan) | NR18 UAV [1] (Diana) | NR17 SL Relay (Nathan) |  |
| 14:30 -16:30 | NR18 XR [2] (Diana) | NR17 NTN Maint (Sergio) - 4.2 - 6.3 NTN Self evaluation (Sergio) - 7.25.4: Report of [Post123][102] NR18 NTN enh [1] (Sergio) - 7.7.1 - 7.7.2 (also depending on progress in the common session) - 7.7.3 | NR18 Pos [2] (Nathan) - 7.2.1 Organizational - 7.2.3 RAT-dependent integrity - 7.2.4 LPHAP - 7.2.5 RAN1-led objectives if time |  |
| 17:00– 19:00 | NR18 Other [2] Diana | NR18 NTN enh [1] (Sergio) - 7.7.4.1.1 - 7.7.4.1.2 - 7.7.4.2 | NR18 Pos [2] (Nathan) - 7.2.5 RAN1-led objectives - 7.2.2 SL positioning |  |
| Wednesday October 11 |  |  |  |  |
| 08:30 – 10:30 | NR18 feMob [2] (Johan) | NR18 eQoE [1] (Dawid) 17:00-17:30 EUTRA17+ | NR18 SL relay [1.5] (Nathan) - 7.9.3 Service continuity - 7.9.4 Multi-path TEI Relay/POS (Nathan) - Relay cause value issues (R2-2309684 and R2-2309795) - Positioning for remote UEs (R2-2310544, R2-2310855) |  |
| 11:00 – 13:00 | NR18 URLLC [0.5] (Diana) NR18 Network Energy Saving [1] (Diana) | NR17 (Mattias) | NR17 SONMDT (HuNan) |  |
| 14:30 – 16:30 | NR18 XR [2] (Diana) | R18 IoT-NTN [1] (Sergio) - 7.6.1 - 7.6.2.1 - 7.6.2.2 - 7.6.3.1 - 7.6.3.2 - 7.6.4 | NR18 SONMDT [1] (HuNan) |  |
| 17:00 – 19:00 | NR18 AIML [1] (Diana) | NR18 RedCap [1] (Mattias) | NR18 MIMO evo [0.75] (Erlin) |  |
| Thursday October 12 |  |  |  |  |
| 08:30 – 10:30 | CB NR151617 (Mattias) | CB Sergio NR18 NTN Enh - TBD | CB Kyeongin |  |
| 11:00 – 13:00 | NR18 TEI [1] (Diana) | CB Erlin (change this) MU-SIM MIMO | CB Kyeongin |  |
| 14:30 – 16:30 | NR18 Other [2] (Diana) | CB Dawid MBS QoE | CB Nathan |  |
| 17:00 – 19:00 | CB Diana XR UAV NES | CB Johan feMob mIAB | CB Nathan |  |
| Friday October 13 |  |  |  |  |
| 08:30 – 10:30 | CB Mattias R17 | CB Eswar Cov. Enhc. CB Johan | CB Nathan, Kyeongin TBD |  |
| 11:00 – 13:00 | CB Diana | CB Sergio R18 IoT-NTN - TBD | CB Hunan |  |
| 14:30 – 16:00 |  |  |  |  |
| 16:00 – 17:00 | [8] Reports from parallel sessions CB and conclusion (Diana) |  |  |  |

List and status of offline discussions

NOTE:  No offline email discussions will be kicked off before Monday Oct 9th, 09:00 local time

[AT123bis][301][IoT-NTN] Koffset handling during handover (Huawei)

Scope: discuss Stage 2 CRs

Intended outcome: 36.300 and 38.300 CRs

Deadline for rapporteur's CRs (in R2-2311311 and R2-2311312):  Friday 2023-10-13 08:00

[AT123bis][302][NR-NTN] Notes in the RRC release procedure (Google)

Scope: discuss the need and possible wording for the second note

Intended outcome: revised CR

Deadline for rapporteur's CR (in R2-2311313):  Friday 2023-10-13 08:00

[AT123bis][303][NTN Self Ev] CPUP latency (Ericsson)

Scope: continue the discussion on the TP also based on input papers

Intended outcome: updated TP and draft LS to RAN1

F2F schedule: FFS

Deadline for updated TP and LS (in R2-2311314 and R2-2311315):  Friday 2023-10-13 08:00

[AT123bis][304][NR-NTN Enh] NW verified UE location failure during cell change (Qualcomm)

Scope: discuss NW verified UE location failure occurred during change of cell

Intended outcome: offline summary

F2F schedule: FFS

Deadline for rapporteur's summary (in R2-2311316):  Thursday 2023-10-12 08:00

[AT123bis][305][NR-NTN Enh] Support of NTN neighbor cell info in TN cell (Ericsson)

Scope: discuss the details of how broadcast NTN neighbor cell info in a TN cell

Intended outcome: offline summary

F2F schedule: FFS

Deadline for rapporteur's summary (in R2-2311317):  Friday 2023-10-14 08:00

[AT123bis][306][NR-NTN Enh] RACH-less HO (Interdigital)

Scope: continue the discussion on RACH-less HO

Intended outcome: offline summary

No formal F2F offline is expected: the rapporteur will check offline with as many interested companies as possible and prepare an updated list of proposals for online discussion in the Thursday CB session

Deadline for rapporteur's summary (in R2-2311318):  Thursday 2023-10-12 08:00

[AT123bis][307][NR-NTN Enh] Unchanged PCI (Apple)

Scope: continue the discussion on unchanged PCI aspects (marked for offline discussion in 307)

Intended outcome: offline summary

F2F schedule: Wednesday 2023-10-11 10:30-11:00 Brk3

Deadline for rapporteur's summary (in R2-2311319):  Thursday 2023-10-12 08:00

## 4.2	NB-IoT and eMTC support for NTN Rel-17

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

Tdoc Limitation: 1 tdocs

This Agenda Item is treated in the Breakout session that includes NTN

A single CR per TS with miscelaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.  Big open issues can be discussed with contributions with CR in the appendix of the contribution

R2-2310835	Clarification on ul-SyncValidityDuration in SIB31	ZTE Corporation, Sanechips	CR	Rel-17	36.331	17.6.0	4962	-	F	LTE_NBIOT_eMTC_NTN-Core

- ZTE indicated that “ntn-UlSyncValidityDuration” needs to be updated to the actual name for IoT-NTN in the field description

- Vivo thinks the version in the coversheet should be 17.6.0

Revised in R2-2311310

R2-2311310	Clarification on ul-SyncValidityDuration in SIB31	ZTE Corporation, Sanechips	CR	Rel-17	36.331	17.6.0	4962	1	F	LTE_NBIOT_eMTC_NTN-Core

Agreed in principle

R2-2310716	Koffset handling during handover	Huawei, HiSilicon	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

Proposal 1: Clarify that UE specific Koffset should be cleared during MAC reset.

-	Oppo agrees with the problem and thinks it should be fixed

-	LG thinks the same applies to NR

-	Lenovo does not think this is an essential issue (and it also applies to NR NTN)

-	QC don’t think this is not needed and if we clarify something we should not refer to MAC reset but refer to HO. Samsung agrees

-	Ericsson thinks it would be a strange UE implementation to use this after HO, but is ok to clarify that that differential Koffset should be cell specific

Introduce a clarification in Stage2 for both IoT NTN and NR NTN that UE specific Koffset should be cleared during HO (exact wording FFS)

Draft corresponding CRs for 36.300 and 38.300

[AT123bis][301][IoT-NTN] Koffset handling during handover (Huawei)

Scope: discuss Stage 2 CRs

Intended outcome: 36.300 and 38.300 CRs

Deadline for rapporteur's CRs (in R2-2311311 and R2-2311312):  Friday 2023-10-13 08:00

## 6.3	NR Non-Terrestrial Networks (NTN)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

Tdoc Limitation: 1 tdocs

A single CR per TS with miscellaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.  Big open issues can be discussed with contributions with CR in the appendix of the contribution

R2-2309533	Correction on Event D1	OPPO	CR	Rel-17	38.331	17.6.0	4306	-	F	NR_NTN_solutions-Core

- MTK and QC don’t think the CR is needed

- HW thinks the UE will report the latest information and think the existing text is ok

Not pursued

R2-2309652	Correction on the Capability of TA Reporting	vivo	CR	Rel-17	38.306	17.6.0	0931	1	F	NR_NTN_solutions-Core	R2-2307113

- QC thinks this is not needed. Ericsson agrees with QC. MTK also agrees

Not pursued

R2-2310757	Notes in the RRC release procedure for NR-NTN	Google Inc.	CR	Rel-17	38.331	17.6.0	4351	-	F	NR_NTN_solutions-Core

- Samsung that the first note was suggested in Rel-15 and was not agreed at that time

- QC thinks the notes reflect the expected UE behaviour.

- Ericsson thinks the notes are not needed as this the expected UE behaviour anyway and if we add these notes for these cases we would need to add other similar notes as well

- ZTE thinks the first note is common for NR, not for NR NTN. The second note mention HARQ disabled but this is not suitable for RRC spec

- Intel thinks both notes are not needed. Apple agrees

- Ericsson think the IoT NTN case is different, due to repetitions, for NR NTN is a corner case we don’t need to clarify

Discuss offline if (a revision of) the second note can be agreeable

[AT123bis][302][NR-NTN] Notes in the RRC release procedure (Google)

Scope: discuss the need and possible wording for the second note

Intended outcome: revised CR

Deadline for rapporteur's CR (in R2-2311313):  Friday 2023-10-13 08:00

R2-2310715	Triggering of TA Report during handover	Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

Observation 1: The current spec is not clear about UE behaviour when offsetThresholdTA is included in handover command while ta-report is not.

Proposal 1: RAN2 to discuss whether and when TA report is triggered to the target cell, if offsetThresholdTA is included in handover command while ta-report is not.

- Lenovo wonders if this is a valid case. Nokia agrees

- Vivo thinks the current MAC spec is clear and no clarification is needed.

No need to further discuss this, it should be up to NW implementation

R2-2311241	UTC reference point in NR NTN R17	Ericsson	discussion	Rel-17	NR_NTN_solutions

Observation 1	Usefulness of the broadcasted/dedicated signalled UTC is much less in NR where UEs are assumed to have simultaneous capability to transmit/receive and measure GNSS.

Proposal 1	Align the IoT NTN UTC reference point and the NR NTN reference point.

- Nokia agrees with observation 1 so is not sure whether we need p1 now, we can add it in the future if needed

- QC agrees with p1

- Vivo agrees with the intention but think that Redcap for NTN is not supported in this release and we should postpone this. QC does not agrees with this statement : Redcap can be supported in NTN

- Sequans agrees with QC and supports p1

- Apple supports the observation but thinks we can have this in the next release.

- QC thinks that if we only have it in R18 there would be inter-operability issues. Oppo thinks this is not the case

Come back to this in the next meeting (for possible inclusion also in Rel-17)

## 7.6	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-223519)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

### 7.6.1	Organizational

LSs, rapporteur inputs and other organizational documents. Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Incoming LSs

R2-2309416	LS on Rel-18 RAN1 UE features list for LTE after RAN1#114 (R1-2308520; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-18	IoT_NTN_enh	To:RAN2	Cc:RAN4

Running CRs

R2-2310180	Running CR for TS 36.306 for Rel-18 IoT NTN	Qualcomm Incorporated	draftCR	Rel-18	36.306	17.4.0	B	IoT_NTN_enh-Core

R2-2310410	36331 running CR for IOT NTN	Huawei, HiSilicon	draftCR	Rel-18	36.331	17.6.0	B	IoT_NTN_enh-Core

R2-2311070	Stage-3 running CR for TS 36.321 for Rel-18 IoT-NTN	MediaTek Inc.	draftCR	Rel-18	36.321	17.6.0	F	IoT_NTN_enh-Core

R2-2311194	36304 Running CR for IoT-NTN	Nokia Solutions & Networks (I)	draftCR	Rel-18	36.304	17.4.0	IoT_NTN_enh-Core

R2-2311244	Introduction of IoT NTN enhancements	Ericsson	CR	Rel-18	36.300	17.5.0	1387	-	B	IoT_NTN_enh-Core

R2-2311245	Rapporteur input on 36.300	Ericsson	draftCR	Rel-18	36.300	17.5.0	B	IoT_NTN_enh-Core

Other

R2-2309532	Discussion on R18 IoT NTN UE capabilities	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

### 7.6.2	Performance Enhancements

#### 7.6.2.1	HARQ enhancements

R2-2309527	Discussion on HARQ enhancement for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1	For both DCI-based direct indication and DCI-based overridden indication for enabling/disabling HARQ feedback, all the TBs scheduled by a single DCI are configured with HARQ feedback enabled or HARQ feedback disabled, which is simple for DCI design.

Observation 2	For RRC-based enabling/disabling HARQ feedback configuration (without DCI-based overridden indication), it would be more flexible to support mixed HARQ feedback enabled/disabled configuration.

Observation 3	For UL, only RRC-based HARQ mode configuration is supported.

Proposal 1	For NB-IoT UEs configured with two HARQ processes and at least one of them is configured with HARQ feedback disabled, RAN2 does not change the operation on drx-InactivityTimer for single-TB scheduling case.

Proposal 2	For a HARQ process configured as HARQ feedback disabled by RRC and further reversed to HARQ feedback enabled by DCI, UE behaviour on DRX follows the case when HARQ feedback is disabled.

Proposal 3	For multiple UL TBs scheduling, it is up to network implementation to configure multiple TBs using HARQ processes in the same or different HARQ modes.

Proposal 4	If a NB-IoT UE receives a PDCCH indicating the transmission for multiple DL TBs, UE stop drx-InactivityTimer as legacy, regardless of the enabling/disabling HARQ feedback configuration for each of the multiple scheduled TB.

Proposal 5	For a NB-IoT UE configured with two HARQ processes, if PDCCH indicates the transmission is for multiple TBs and if at least one DL HARQ process is configured with disabled HARQ feedback, UE starts drx-InactivityTimer in the subframe containing the last repetition of the PDSCH corresponding to the last scheduled TB plus 12 subframes plus deltaPDCCH.

Proposal 6	If a NB-IoT UE receives a PDCCH indicating the transmission for multiple UL TBs, UE stop drx-InactivityTimer as legacy, regardless of the HARQ mode configuration for each of the multiple scheduled TB.

Proposal 7	For a NB-IoT UE configured with two HARQ processes, if PDCCH indicating the transmission is for multiple TBs and if at least one HARQ process is configured with HARQ mode B, UE starts drx-InactivityTimer in the subframe containing the last repetition of the PUSCH corresponding to the last scheduled TB plus 1 subframe plus deltaPDCCH.

Proposal 8	For multiple TB scheduling with the same HARQ feedback enabled/disabled configuration, HARQ RTT Timer is calculated as legacy.

Proposal 9	For multiple TB scheduling with mixed HARQ feedback enabled/disabled configuration, if HARQ-ACK bundling is configured, HARQ RTT Timer is calculated as legacy.

Proposal 10	For multiple TB scheduling with mixed HARQ feedback enabled/disabled configuration, if HARQ-ACK bundling is not configured, HARQ RTT Timer is calculated based on the number of scheduled TBs with DL HARQ feedback enabled.

Proposal 11	Don’t introduce HARQ mode configuration for PUR in IoT NTN.

R2-2309657	Remaining Issues on HARQ Enhancement for IoT NTN	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2309701	Remaining issues of HARQ enhancement	Huawei, Turkcell, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

R2-2309752	Discussion on HARQ enhancements in IoT NTN	CATT	discussion	Rel-18

R2-2309758	Discussion on HARQ enhancement	Xiaomi	discussion	Rel-18

R2-2309781	On Disabling HARQ Feedback in IoT-NTN	MediaTek Inc.	discussion

R2-2309956	Views on some remaining issues for HARQ in IoT NTN	Lenovo	discussion	Rel-18

R2-2310181	DCI-based HARQ feedback overriding solution	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310269	Discussion on the HARQ enhancement for IoT-NTN	CMCC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310651	Further discussion on HARQ enhancements for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310805	Disabling HARQ feedback for IoT-NTN	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310820	Remaining issues of HARQ enhancements	ZTE Corporation, Sanechips	discussion	Rel-18	IoT_NTN_enh-Core

R2-2311243	R18 IoT NTN HARQ enhancements	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

#### 7.6.2.2	GNSS operation enhancements

R2-2309782	Enhancements on GNSS operation	MediaTek Inc.	discussion

Proposal 1a: If GNSS measurement is completed within the measurement gap, upon measurement completion GNSS validity duration report MAC CE may trigger SR.

Proposal 1b: If GNSS measurement is completed after the measurement gap, upon measurement completion GNSS validity duration report MAC CE will trigger SR.

Proposal 2a: For UE autonomous GNSS measurement trigger, if GNSS measurement is completed within the measurement timer, upon measurement completion GNSS validity duration report MAC CE may trigger SR.

Proposal 2b: For UE autonomous GNSS measurement trigger, if GNSS measurement is completed after the measurement timer, upon measurement completion GNSS validity duration report MAC CE will trigger SR.

Proposal 3: For UE autonomously trigger during the DRX inactivity time, upon GNSS measurement completion GNSS validity duration report MAC CE may trigger SR.

Proposal 4: RAN2 will introduce a new duration D after the measurement gap/time. If UE cannot complete random access before the end of duration D after measurement gap/timer, UE moves to idle. FFS for the value of duration D.

Proposal 5: GNSS Measurement Command MAC CE contains 4-bit GNSS measurement gap with component values: [1,2,3,4,5,6,7,13,19,25,31].

Proposal 6: A new RRC timer T3xx is configured by RRC signaling, where UE can re-acquire the GNSS position fix autonomously.

Proposal 7: The component values of the RRC timer T3xx configuration are: [1,2,3,4,5,6,7,13,19,25,31].

Proposal 8a: The UE AS operation should be resumed upon completion of UE’s GNSS measurement, at least within the measurement gap.

Proposal 8b: The UE AS operation should be suspended when UE is performing GNSS measurement during the GNSS measurement timer.

Proposal 8c: The UE AS operation should be resumed upon UE completing GNSS measurement, at least within the GNSS measurement timer.

Proposal 9: A new RRC parameter is introduced in dedicated RRC signalling to enable/disable duration X.

Proposal 10: A new RRC parameter is introduced in dedicated RRC signalling to configure duration Y when timeAlignmentTimer is infinity.

Proposal 11: UE does not report GNSS position fix time duration when the remaining GNSS validity duration is infinity.

R2-2310821	Remaining issues of GNSS enhancements	ZTE Corporation, Sanechips	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: Even the GNSS position fix time duration can kept unchanged during the whole connection, as there are some large values in the value range of GNSS position fix time duration, it still can be assumed that the reported GNSS position fix time duration and also the configured gap may be possibly aggressive, e.g., (much) longer than the time period that the UE actually needs to perform the GNSS measurement. Then it may be highly possible for UE to finish GNSS measurement earlier than the end of measurement gap.

Proposal 1a: If a remaining GNSS validity duration report is triggered and there are no available UL-SCH resources, contention based Random Access procedure can be initiated by the UE to request UL resource to send this report.

Proposal 1b: No SR is triggered for the remaining GNSS validity duration report.

Proposal 2a: After successful GNSS measurement, UE should finish the remaining GNSS validity duration report before the end of a duration D after the end of the measurement gap (aperiodic GNSS measurement gap or the autonomous GNSS measurement timer).

Proposal 2b: UE should go to IDLE or trigger RLF if the remaining GNSS validity duration is not reported before end of duration D.

Proposal 2c: The duration D is configured by eNB.

Proposal 3a: If timeAlignmentTimer is infinity, a duration X can be configured via RRC signaling, e.g., in Msg4 for IoT UE using CP solution or in Msg4/RRC reconfiguration message for IoT UE using UP solution.

Proposal 3b: Upon receiving the indication that the GNSS position becomes out-of-date, UE would apply duration X (if configured), e.g., to keep UL transmission without GNSS re-acquisition.

Proposal 4: Only one common measurement gap is configured via RRC, e.g., Msg4. And this common measurement gap can be applied to both NW-triggered GNSS measurement and autonomous GNSS measurement.

Proposal 5: RAN2 needs to introduce finer values, e.g., in unit of milliseconds, for the value range of UE remaining GNSS validation duration report during connected mode.

Proposal 6: The eNB can enable autonomous GNSS reacquisition in UE side via Msg4. The following disable or (re)enable configuration can be provided via RRC reconfiguration message for UE using UP solution.

Proposal 7: RAN2 further discuss whether other features related UE location, e.g., location-based connected mode mobility, can still be feasible in duration X.

R2-2309531	Discussion on GNSS operation for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

R2-2309658	Discussion on GNSS Operation for IoT NTN	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2309702	Discussion on the impact of GNSS measurement	Huawei, Turkcell, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

R2-2309751	Discussion on GNSS operation enhancements	CATT	discussion	Rel-18

R2-2309757	Discussion on GNSS operation enhancement	Xiaomi	discussion	Rel-18

R2-2309957	Views on some remaining issues for GNSS operations in IoT NTN	Lenovo	discussion	Rel-18

R2-2309997	Remaining issues on the GNSS operation	Google Inc.	discussion	Rel-18

R2-2310184	GNSS fix in RRC_CONNECTED	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310270	Discussion on GNSS enhancement for IoT-NTN	CMCC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310323	Concluding critical issues in improved GNSS operation	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2310650	GNSS Validity duration Reporting	Nordic Semiconductor ASA	discussion

R2-2310652	Further discussion on GNSS operation enhancement in Rel-18 IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310806	GNSS acquisition and reporting for IoT NTN	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2311036	On improved GNSS operation for IoT NTN	Samsung Electronics Polska	discussion	Rel-18	IoT_NTN_enh

R2-2311086	Discussion of GNSS operation enhancements	SHARP Corporation	discussion	R2-2308617	Late

R2-2311242	R18 IoT NTN GNSS operation enhancements	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

### 7.6.3	Mobility Enhancements

R2-2309958	Views on some remaining issues for mobility in IoT NTN	Lenovo	discussion	Rel-18

#### 7.6.3.1	Enhancements for neighbour cell measurements

R2-2310628	On enhancements for neighbour cell measurements	Samsung Electronics Polska	discussion	Rel-18	IoT_NTN_enh

Observation 1: To integrate NTN with a terrestrial network, an inter or intra-frequency should be able to be NTN, TN or both NTN and TN.

Observation 2: Without specified rules on how to acquire T318, there will be restrictions on how network can schedule SIBxx.

Proposal 1: t-ServiceStartNeigh is per neighbour cell.

Proposal 2: To indicate an intra or inter-frequency (cell) having the same ephemeris as serving cell, either explicit indication in SIB3 and SIB5, or implicit via pre-defined satelliteId=0 value is introduced.

Proposal 3: Upon absence of satellite IDs for intra-frequency, the UE assumes Rel-17 behavior for intra-frequency measurements, i.e measurement according to UE implementation.

Proposal 4: Upon absence of satellite IDs for inter-frequency, the UE assumes Rel-17 behavior for inter-frequency measurements, i.e measurement according to UE implementation.

Proposal 5: If SIBxx is present, then satellite IDs in either SIB3, SIB5 or both SIB3 and SIB5 shall be present.

Proposal 6: Enable signaling that an inter or intra-frequency is NTN, TN or both TN and NTN.

Proposal 7: Introduce choice structure in inter and intra-frequency signaling to indicate that the frequency is NTN, TN or both NTN and TN.

Proposal 8: Agree TP in appendix A1 and A2.

Proposal 9: If UE is acquiring SIBxx, the T318 is not stopped when SIB31 is succesfully acquired. T318 expiry does not trigger RLF if SIBxx is being acquired (as in Appendix A3).

Proposal 10: Current condition on triggering SIBxx acquiry as in Running CR is sufficient.

Proposal 11: RAN2 to discuss how UE acquires target cell neighbour cell assistance information during handovers.

Proposal 12: RAN2 to consider use case-based / traffic-based conditions for not performing neighbour cell measurements.

Proposal 13: Allow sending UE context in advance for faster RLF procedures.

R2-2310411	Remaining issues on mobility enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: Satellite assistance information is provided per frequency, and not associated with PCIs. The satellite IDs for intra-frequency measurements are in SIB3 as in the current running CR (no need to place them into SIB4).

Proposal 2: Introduce satellite ID for serving satellite (in SIB31) as well. RAN2 does not consider implicitly reusing serving satellite assistance information.

Proposal 3：t-ServiceStartNeigh is set as the earliest start time among all neighbour cells across different frequencies, or set per frequency or per satellite, not per neighbour cell.

Proposal 4：RAN2 discuss how to solve the case where T318 is stopped before successful acquisition of SIBxx.

Proposal 5: Separate reference locations are introduced for earth-quasi fixed cells and earth-moving cells.

Proposal 6: A new t-Service IE is introduced in SIB3-NB to indicate RRC_CONNECTED UEs to initiate the neighbour cell measurements before that t-Service (to differentiate from the R17 t-Service to be used for measurements initiation in RRC_IDLE).

Proposal 7: For both RRC_CONNECTED and RRC_IDLE, time/Location based neighbour cell measurement triggering can be configured together with the existing RSRP based triggering. If configured jointly, the UE starts measure neighbour cell when either of the triggering condition is met.

Proposal 8: Regarding Connected mode measurement initiation for eMTC UEs, time-based configuration is in SIB3, location-based configuration is in SIB31, legacy RSRP-based configuration (s-Measure) is in measObject. No signalling optimisation is introduced.

R2-2309528	Discussion on mobility enhancement for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

R2-2309762	Discussion on UE behavior when serving cell t-service expires	Xiaomi	discussion	Rel-18

R2-2309783	Enhancements on neighbor cell measurement	MediaTek Inc.	discussion

R2-2310183	Measurement and Mobility enhancements	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310271	Discussion on mobility enhancements for IoT-NTN	CMCC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310324	Neighbour cell measurements before RLF for eMTC-NTN	Apple, MediaTek Inc.	discussion	Rel-18	IoT_NTN_enh

R2-2310807	Fast RLF and re-establishment in the discontinuous coverage scenario	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310822	Remaining issues of mobility enhancements	ZTE Corporation, Sanechips	discussion	Rel-18	IoT_NTN_enh-Core

R2-2311012	On Remaining issues for IoT-NTN Mobility Enhancements	Nokia, Nokia Shanghai Bell	discussion

R2-2311069	Discussion on gaps for neighbour cell measurements in IoT NTN	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core	R2-2308811

R2-2311240	Discussion on triggering RA for RRC connection re-establishment in IoT NTN	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core	Late

#### 7.6.3.2	Other

R2-2310192	NB-IoT NTN Coarse UE location reporting	Inmarsat, Viasat, Sateliot, Novamint, ESA, Thales	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1:  UE location reporting to the network is a fundamental requirement for NTN to support:

-	Basic Regulatory Compliance

-	Efficient Radio Resource Management and Cell Mapping

-	Efficient Mobility Management

Observation 2: RRC CoarseLocationInfo mechanism was introduced in Release-17 IoT NTN for eMTC but not for NB-IoT NTN, due to lack of AS security and no alternative solution was provided.

Observation 3: In practice, we have to recognize that majority of NB-IoT NTN devices will focus on CP CIoT Optimizations with Data over NAS, thus AS security cannot be assumed to be in place.

Observation 4:  Methods relying on LPP cannot be relied upon, if anything because LPP is not supported by most NB-IoT UE implementations.  This is already affecting Release-17 UE and it’s clear that we must identify a different solution.

Observation 5: It is difficult to justify privacy concerns when reporting Coarse UE location at a level of granularity equivalent to the size of a terrestrial cell (e.g. 5 km or larger), given that such concerns with knowledge of the UE location by an external observer within the granularity of a cell do not exist in terrestrial networks.

Proposal 1: Specify a Release-18 mechanism for UE location reporting as early as possible during the initial access procedure for NB-IoT NTN devices, including those devices with support for CP CIoT EPS Optimization only.

•	At least coarse UE location on the order of 50-100 km shall be supported;

•	Coarse UE location on the order of 5-10 km is desirable.

•	Specify a method to broadcast the request for Coarse UE Location to a group of UE or to all UE within a cell.

•	AS security should not be assumed.  SA3 should consider an exception, if necessary.

•	Consider specifying support for full UE location reporting over NAS with NAS security activated.

R2-2309659	Discussion on CHO Enhancement for IoT NTN	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310325	Mobility enhancement in IoT NTN	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2310629	On other mobility enhancements for IoT NTN	Samsung Electronics Polska	discussion	Rel-18	IoT_NTN_enh

### 7.6.4	Enhancements to discontinuous coverage

R2-2309660	Discussion on Discontinuous Coverage	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2309703	Remaining issues of discontinuous coverage	Huawei, Turkcell, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

R2-2309753	Discussion on discontinuous coverage enhancement	CATT	discussion	Rel-18

R2-2309882	Discussion on TN coverage for discontinuous coverage	ASUSTeK	discussion	Rel-18	IoT_NTN_enh-Core

R2-2309959	Views on some remaining issues for discontinuous coverage in IoT NTN	Lenovo	discussion	Rel-18

R2-2310021	Discussion on enhancement to discontinuous coverage for IoT NTN	Transsion Holdings	discussion	Rel-18

R2-2310071	Considerations on Supporting Discontinuous Coverage	NEC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310182	RRC release procedure in discontinuous coverage	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310808	RRC Release in discontinuous coverage	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2310826	Remaining issues of discontinuous coverage	ZTE Corporation, Sanechips	discussion	Rel-18	IoT_NTN_enh-Core	R2-2307590

R2-2310919	Enhancements to discontinuous coverage 	Samsung Electronics Iberia SA	discussion	Rel-18	IoT_NTN_enh-Core

R2-2311013	Discussion on discontinuous coverage Enhancements	Nokia, Nokia Shanghai Bell	discussion

R2-2311232	Measurement information to assist cell search after a coverage gap	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core	R2-2306466

## 7.7	NR NTN enhancements

(NR_NTN_enh -Core; leading WG: RAN1; REL-18; WID: RP-232669)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

### 7.7.1	Organizational

LSs, rapporteur inputs and other organizational documents. Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Incoming LSs

R2-2309407	LS on the service requirement of restricting satellite access RAT type (C1-236567; contact: Google)	CT1	LS in	Rel-18	5GSAT_Ph2	To:SA1	Cc:SA2, RAN2

Noted

R2-2309421	Reply LS to RAN2 on unchanged PCI (R1-2308566; contact: CATT)	RAN1	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2

Noted

R2-2309422	Reply LS on RACH-less Handover (R1-2308568; contact: Samsung)	RAN1	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2

- CATT wonders if the answer to Q3 means that RAN2 should not discuss power control anymore. Samsung this is the case, unless RAN1 come back to us for this

Noted

R2-2309438	Reply LS on Common Signaling in (C)HO (R3-234664; contact: Qualcomm)	RAN3	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2

- Oppo thinks that RAN2 can still discuss the intra-gNB case

Noted

R2-2309476	Reply LS on time-based trigger condition in NG HO for NR NTN (S2-2310013; contact: Samsung)	SA2	LS in	Rel-18	5GSAT_Ph2, NR_NTN_enh-Core	To:RAN3	Cc:RAN2

Noted

Rapporteurs input

R2-2310084	Remaining Issues on NR Non-Terrestrial Networks (NTN)	THALES	discussion	Rel-18	NR_NTN_enh

Noted

R2-2310085	R18 WI NR-NTN-enh work plan at RAN1, 2 and 3	THALES	Work Plan	Rel-18	NR_NTN_enh

Noted

R2-2310841	MAC open issues in NTN	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

Noted

Running CRs

R2-2310157	UE Capabilities for Rel-18 NR NTN Enh. WI	Intel Corporation	draftCR	Rel-18	38.306	17.6.0	NR_NTN_enh-Core

Endorsed as a basis for further updates

R2-2310158	UE Capabilities for Rel-18 NR NTN Enh. WI	Intel Corporation	draftCR	Rel-18	38.331	17.6.0	NR_NTN_enh-Core

Endorsed as a basis for further updates

R2-2310552	Stage 3 running 38.304 CR for NTN	ZTE Corporation, Sanechips	draftCR	Rel-18	38.304	17.6.0	NR_NTN_enh-Core

Used as a basis for further updates

R2-2310840	Stage 3 NTN running CR for 38.321 - RAN2#123	InterDigital	draftCR	Rel-18	38.321	17.6.0	B	NR_NTN_enh-Core	R2-2309345

Used as a basis for further updates

R2-2311230	Stage 3 Running RRC CR for NR NTN Rel-18	Ericsson	CR	Rel-18	38.331	17.6.0	4387	-	B	NR_NTN_enh-Core	R2-2309341

Endorsed as a basis for further updates

R2-2311231	Rapporteur s input to 38.331 regarding TN area information	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Noted

R2-2311255	Stage-2 running CR for TS 38.300 for Rel-18 NTN enhancements	Thales	draftCR	Rel-18	38.300	17.6.0	NR_NTN_enh-Core	Late

Used as a basis for further updates

### 7.7.2	Coverage Enhancements

*** Intermediate conclusion from F2F offline discussion [002] ***

- Usage of first R bit LCID extension only applied to UL CCCH/CCCH1 controlled by network.

R2-2309529	Discussion on PUCCH enhancement for Msg4 HARQ-ACK in NR NTN	OPPO	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	Indicating request or capability report for PUCCH repetition for Msg4 HARQ-ACK in Msg3 only applies to random access procedure triggered by RRC connection establishment, RRC connection re-establishment or RRC connection resume.

- CATT agrees these cases should be considered but wonders if random access during RRC connected should also be covered. Ericsson thinks this is not needed

- Ericsson also think RAN2 removed the reference to request. Nokia agrees with Ericsson that RAN1 now only consider the capability, not the request anymore

- QC thinks RAN1 is also considering the request case. Vivo agrees

Indicating request or capability report for PUCCH repetition for Msg4 HARQ-ACK in Msg3 only applies to random access procedure triggered by RRC connection establishment, RRC connection re-establishment or RRC connection resume, i.e. to CCCH/CCCH1 (FFS if this also applies to random access during RRC connected)

CB Thursday

R2-2310559	Consideration on coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-18

Proposal 1: Postpone the decision on which higher layer signalling to indicate UE’s capability/request for PUCCH repetition for Msg4 ACK until progress has been made on LCID extension discussion in the main session.

Proposal 2: Provided Msg3 signalling to report its capability/request for PUCCH repetition for Msg4 HARQ-ACK is confirmed, UE decides whether to report its capability/request for PUCCH repetition for Msg4 HARQ-ACK bases on implicit indication (e.g., number of repetition, RSRP configuration in SIB), no explicit indication is needed to enable this behavior.

No explicit NW indication to enable/disable PUCCH repetition for Msg4 HARQ-ACK besides the needed signalling for number of repetition, RSRP configuration in SIB (meaning that if these parameters are signalled, PUCCH repetition for Msg4 HARQ-ACK is enabled)

Proposal 3: An relative RSRP value to rsrp-ThresholdMsg3 is signalled for PUCCH repetition for Msg4 HARQ-ACK.

R2-2310000	Higher layer signalling for PUCCH repetition for Msg4 HARQ-ACK	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

### 7.7.3	Network verified UE location

R2-2310560	Consideration on NW verified UE location	ZTE Corporation, Sanechips	discussion	Rel-18

Observation 1: There are could be different failure causes leading to multi-RTT measurements failure during change of NTN cell, which can be well addressed by legacy failure procedure defined in LPP and NRPPa specs, while a simple indication to indicate failure due to change of NTN cell maybe too ambiguous.

Observation 2: UE Rx-Tx time difference measurement with offset to calibrate the timing drift due to precompensation will be captured in RAN1 specs with no RAN2 impacts.

Observation 3: Newly agreed offset as defined by RAN1 shall be included as part of the UE Rx-Tx time difference measurements in NR-Multi-RTT-SignalMeasurementInformation, also indication from NW is needed to inform UE it is a task for NW verified UE location so that UE can take necessary adaptions.

Observation 4: It is sufficient for NW to provide the satellite ephemeris together with the time information to LMF to derive position of satellite, no need for UE to provide duplicated information to LMF.

Observation 5: The satellite ephemeris information maybe available in OAM or gNB in different NTH deployment, it is up to RAN3 to discuss the details on how NW provide such information to LMF.

Proposal 1: Current failure procedure defined in LPP and NRPPa protocol can be reused to handle NW verified UE location failure occurred during change of cell, no additional specs work is needed.

- QC thinks that especially in the moving cell case this would be a problem. Relying on legacy procedure would add extra delay. RAN needs a way to react quickly and proactively send information to LMF

- Ericsson agrees with QC

Continue in offline 304

[AT123bis][304][NR-NTN Enh] NW verified UE location failure during cell change (Qualcomm)

Scope: discuss NW verified UE location failure occurred during change of cell

Intended outcome: offline summary

F2F schedule: FFS

Deadline for rapporteur's summary (in R2-2311316):  Thursday 2023-10-12 08:00

Proposal 2: Add in NR-Multi-RTT-SignalMeasurementInformation the measurements relevant to RAN1 agreed offset (e.g., the actual index difference between subframe j and subframe i and the DL timing drift due to Doppler over the service link associated with the UE RX-TX time difference measurement period) with detailed definition referred to RAN1 agreements.

Agreed

Proposal 3: Add indication indicating the requested positioning measurements (e.g.,multi-RTT measurement) is for NW verified UE location in the LPP configuration (e.g, RequestLocationInformation) from location server to UE.

- QC is not sure this is needed. Capability should be sufficient

- Ericsson also thinks this is not really needed

Proposal 4: ephemeris and corresponding time information (e.g., epochTime) is provided to LMF only by gNB, with details up to RAN3 (e.g., on whether OAM or gNB to provide such information).

-	Lenovo thinks the UE could use a different ephemeris than the gNB and then it should be reported back by the UE. ZTE thinks this is not common case we need to address here.

Ephemeris and corresponding time information (e.g., epochTime) is not provided by the UE. How this is provided to the LMF is up to RAN3 (can come back to see whether the problem that the UE could use a different ephemeris – and then should report it back to the gNB – is a valid case to consider)

Proposal 5: Only one capability (i.e., FG 44-3) is defined for UE’s capability to support the feature of network verified UE location in NR NTN.

- Ericsson thinks a new capability would be needed.

- QC thinks that FG 44-3 should not be an RRC capability but an LPP one.

RAN2 assumes that FG 44-3 should be an LPP capability to be reported to the LMF (no need for other capabilities)

R2-2309700	Handling of UE location verification during handover	Huawei, Turkcell, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: If handover occurs between two TX-RX difference measurement procedures, there is no impact on the verification of UE location.

Proposal 1: If handover occurs when the Tx-Rx difference is being measured, the gNB should inform LMF of the handover, same as legacy.

Proposal 2: Send an LS to RAN3 if RAN2 agrees to reuse the legacy mechanism to inform LMF of the handover.

R2-2310176	Single satellite Multi-RTT based positioning	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	Add a new LPP capability by extending the current multi-RTT positioning capability NR-Multi-RTT-ProvideCapabilities-r16.

Proposal 2	A new flag in NRPPa message (e.g., POSITIONING INFORMATION UPDATE) is introduced to indicate TN-NTN switch, satellite switch with PCI unchanged, Handover success.

Proposal 3	Source sends the flag in NRPPa message (e.g., POSITIONING INFORMATION UPDATE) to LMF before path switch, i.e., receiving end marker.

Proposal 4	Clarify that LMF will start enquiring UE capability upon handover between TN and NTN and send LS to RAN3 to inform potential change in NRPPa signaling.

Proposal 5	To resolve mirror point, LMF sends indication to gNB asking to configure connected mode measurements of neighbor cells in such a way that can resolve the mirror point. Send LS to RAN3 for possible impact to NRPPa message to carry indication.

- Nokia agrees that sending the LS to RAN3 would be beneficial

- Ericsson thinks this is up to RAN3 and we don’t need an LS for this.

RAN2 understands that to solve the mirror point issue, the measurements reported by RAN should include the information of the cells on the opposite side

R2-2309503	Remaining issues on NW verification of UE location in R18 NR NTN	CATT	discussion

R2-2309989	Views on some remaining issues for network verified UE location	Lenovo Information Technology	discussion	Rel-18	NR_NTN_enh-Core

R2-2309990	Discussion on Network Verified UE Location	TCL	discussion	R2-2308706

R2-2309995	Multiple-RTT positioning in NTN	Quectel	discussion

R2-2310037	Discussion on network verified UE location in NR NTN	THALES	discussion	Rel-18	NR_NTN_enh

R2-2310133	Discussion on network verified UE location	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

R2-2310147	Open issues on Network verified location	Nokia, Nokia Shanghai Bell	discussion	NR_NTN_enh-Core

R2-2310245	Discussion on the remaining issues for network verified UE location	CMCC	discussion	Rel-18	NR_NTN_enh-Core

R2-2310418	Discussion on network verified UE location	Xiaomi	discussion

R2-2310985	UE location verification by Network	NEC Telecom MODUS Ltd.	discussion	R2-2308450

R2-2311009	Network Verified UE Location in NTN	Samsung Electronics Iberia SA	discussion	NR_NTN_enh-Core

### 7.7.4	NTN-TN and NTN-NTN mobility and service continuity enhancements

#### 7.7.4.1	Cell reselection enhancements

R2-2310046	Discussion on mobility enhancements for VSAT	THALES	discussion	Rel-18	NR_NTN_enh

R2-2310246	Discussion on left issues for cell reselection	CMCC	discussion	Rel-18	NR_NTN_enh-Core

##### 7.7.4.1.1	NTN-TN enhancements

Maximum number of TN coverage area IDs

R2-2309862	Remaining issues on NTN-TN cell reselection enhancement	LG Electronics France	discussion	Rel-18	38.331	NR_NTN_enh	R2-2307217

Observation 1	A single TN coverage area information has a size of 64 bits.

Proposal 1	The maximum number of the TN coverage area information is 42. Accordingly, the maximum size of the TN coverage area ID is 6 bits.

The maximum number of the TN coverage area information is 32 (5 bits)

Proposal 2	Do not pursue TN-NTN/NTN-TN cell reselection enhancement in Rel-18.

Observation 2	Area information can be efficiently signalled if NW can describe an area having no TN coverage in case when TN coverage has concave shape.

Proposal 3	Allow NW to configure positive area information (TN coverage area) or negative area information (no TN coverage area).

R2-2310842	NTN-TN mobility and service continuity	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1:	RAN2 considers the largest supported NTN cell (i.e., GEO with 3500km diameter) when determing the maximum number of TN coverage areas.

Proposal 2:	The maximum number of TN coverage areas broadcast within SIBxx is 64.

Proposal 3:	RAN2 will not specify restrictions on TN coverage description (i.e., description of TN coverage is left to NW implementation)

RAN2 will not specify restrictions on TN coverage description (i.e., description of TN coverage is left to NW implementation). The signalled TN coverage can describe areas not currently covered by the satellite cell footprint (FFS how to reflect this in the specification)

Proposal 4:	Confirm TN coverage information can be broadcast by both (quasi)earth-fixed and earth-moving cells

Agreed

Proposal 5:	Confirm the working assumption “We do not introduce new triggers making the UE reacquire the TN coverage information from SI” in Rel-18.

Agreed

R2-2309653	Remaining Issues on Cell Reselection for Power Saving	vivo	discussion	Rel-18	NR_NTN_enh-Core

Proposal 3: Up to 16 TN coverage areas can be broadcast.

Whether the new SIB is an essential SIB / Update of TN coverage info / Broadcast of NTN information in TN cell

R2-2309960	Views on some remaining issues for NTN-TN mobility	Lenovo	discussion	Rel-18

Proposal 1: The new SIB including the TN coverage information is not an essential SIB for NTN. An NTN-capable UE does not need to consider the cell barred if it is unable to acquire the SIB when scheduled.

Agreed

Proposal 2: The change of TN coverage information should neither result in system information change notifications nor in a modification of valueTag in SIB1.

- Samsung wonders how SIB should be updated in case of TN coverage areas change

- QC thinks that a change of information regarding a specific area ID should trigger a change notification, but not the additional/removal of area IDs

- Oppo wonders whether we need to introduce a validity duration for the information, maybe the legacy SI update procedure should be followed. ZTE agrees

- CMCC thinks the information should not change too frequently and then we could rely on legacy procedure

- CATT and Huawei support p2. Xiaomi as well

- Intel thinks we could leave the freedom to the NW to signal whether some changes are critical and then notify the UEs.

Legacy SI update procedure will be used when the network updates the TN coverage information (can further check for moving cell case)

Proposal 3: Broadcast of NTN information in TN cell is not pursued in this release.

R2-2311229	NTN neighbour cell information in TN cells	Ericsson, Thales	discussion	Rel-18	NR_NTN_enh-Core

Observation 1	In a terrestrial cell, neighbour cell information provided in system information (e.g., SIB3/4/5) is insufficient to measure an NTN neighbour cell and secure service continuity between TN and NTN in RRC_IDLE/RRC_INACTIVE modes.

Observation 2	In RRC_IDLE/RRC_INACTIVE mode, service continuity from TN to NTN may be restricted because the network cannot provide UEs with the required satellite assistance information (e.g., ntn-Config-r17).

Proposal 1	In TN cells, satellite assistance information, e.g., NTN-config-r17, for NTN neighbour cells can be provided in System Information.

R2-2310306	NTN-TN cell reselection enhancement	Apple	discussion	Rel-18	NR_NTN_enh-Core

Proposal 7: Support to provide the NTN neighbor cell info in TN cell.

- Huawei and Nokia thinks this is not essential for Rel-18 and we have no time to discuss this at this stage. LG agrees

- Sony supports the proposal

- Ericsson thinks this is a low hanging fruit for a real problem

- MTK/QC support the proposal

- China Telecom supports this proposal. DT also supports this proposal. NTT Docomo supports the proposal

- CATT does not support this

- Thales think this is important for service continuity.

- Samsung also support this.

- IDC thinks there is no convincing arguments not to have this.

Continue in offline 305 to discuss the details of how broadcast NTN neighbor cell info in a TN cell (if we will have an agreement to support this in Rel-18)

[AT123bis][305][NR-NTN Enh] Support of NTN neighbor cell info in TN cell (Ericsson)

Scope: discuss the details of how broadcast NTN neighbor cell info in a TN cell

Intended outcome: offline summary

F2F schedule: FFS

Deadline for rapporteur's summary (in R2-2311317):  Friday 2023-10-14 08:00

Proposal 8: RAN2 to select one of the following two options to provide the NTN neighbor cell info in TN cell:

-	Option 1: SIB19 can be broadcasted in TN cell in order to provide the NTN neighbor cell info;

-	Option 2: NTN neigbhor cell info can be provided in SIB3 and/or SIB4 in TN cell.

R2-2310065	Open issues of cell reselection enhancement	Samsung Research America	discussion	Rel-18	NR_NTN_enh-Core

R2-2310177	TN cell coverage info and measurement relaxation	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2310419	Cell reselection enhancements for NTN-TN mobility	Xiaomi	discussion

R2-2310626	Discussion on TN Measurement Relaxation Issues	FGI	discussion	Rel-18

R2-2310986	On the use of TN coverage signalling to indicate non-TN areas	NEC Telecom MODUS Ltd.	discussion

Withdrawn

R2-2309909	Discussion on TN Coverage Area Information Update Issues	FGI	discussion

##### 7.7.4.1.2	NTN-NTN enhancements

Measurements on neighbour cells during hard cell switch / feeder link switch

R2-2310843	Cell reselection enhancements for Earth moving cell	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1:	For Earth-moving cells, multiple future reference location coordinates and associated timestamp information can be broadcast simultaneously.

Proposal 2:	If multiple future reference location coordinates are supported, timestamp information for each future reference point is provided by broadcasting an offset to epochTime.

Proposal 3:	RAN2 to discuss whether the UE can relax (e.g., not perform) measurements on neighbouring cell(s) originating from the same feeder-link about to be switched.

Proposal 4:	If UE can relax (e.g., not perform) measurements on neighbouring cell(s) originating from the same feeder-link about to be switched, RAN2 to discuss how to distinguish when t-Service is due to feeder-link switch.

R2-2311228	Cell reselection enhancements	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	In case of NTN cell hard switch, UE needs not to start neighbour cell measurements of the new cell before t-service expires.

Proposal 2	The network informs (either implicitly or explicitly) the UE whether the next NTN cell switch is a soft or a hard switch.

CB Thursday

R2-2310413	Discussion on location-based measurement initiation in moving cells	Huawei, HiSilicon, Turkcell	discussion	Rel-18	NR_NTN_enh-Core

#### 7.7.4.2	Handover enhancements

RACH-less HO

R2-2310844	NTN mobility enhancements for RRC_CONNECTED	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: 	For location-based CHO for earth-moving cells, re-use the procedure from cell reselection to derive the candidate cell’s reference location as the cell moves.

For location-based CHO for earth-moving cells, re-use the procedure from cell reselection as baseline to derive the candidate cell’s reference location as the cell moves (FFS on how to signal the needed parameters, e.g. ephemeris and Epoch time)

Proposal 2: 	Upon RACH-less HO failure, UE does not fallback to RACH-based HO.

- Oppo was not in favour of this but can accept it

Upon T304 expiry, the UE does not fallback to RACH-based HO.

Continue in offline discussion 306

Proposal 3: 	NW can send PDCCH/PDSCH addressed to C-RNTI to confirm RACH-less HO completion instead of UE Contention Resolution MAC CE (e.g., if DL data is available).

- Ericsson wonders how many mechanism we need

- Samsung thinks we can wait for the outcome of a similar discussion for mobility

Continue in offline discussion 306

Proposal 4: 	UE starts the PTAG timeAlignmentTimer upon indication from upper layers (i.e., RRC) that UL synchronization is obtained for the target cell during RACH-less HO.

Continue in offline discussion 306

Proposal 5: 	Combination of RACH-less HO with time-based CHO is not supported in Rel-18 NTN.

Continue in offline discussion 306

Proposal 6: 	Preallocated UL grant must be configured with an associated RSRP threshold.

Agreed

[AT123bis][306][NR-NTN Enh] RACH-less HO (Interdigital)

Scope: continue the discussion on RACH-less HO

Intended outcome: offline summary

No formal F2F offline is expected: the rapporteur will check offline with as many interested companies as possible and prepare an updated list of proposals for online discussion in the Thursday CB session

Deadline for rapporteur's summary (in R2-2311318):  Thursday 2023-10-12 08:00

R2-2310178	RACH-less handover for NTN	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	For dynamic grant, network can indicate the associated target cell index, i.e., ssb-IndexTarget-r18 in RACH-less configuration.

Proposal 2	While monitoring target cell PDCCH in RACH-less HO, HO failure timer T304 and TimeAlignmentTimer are sufficient to handle fallback to random access to the target cell.

Proposal 3	Confirm network can ask UE to fallback to RACH by PDCCH order when RACH-less HO is configured.

Proposal 4	Regardless of the pre-allocated grant occasions, HARQ process for pre-allocated UL grant belongs to HARQ mode A if UL HARQ mode is configured.

Proposal 5	Target cell specific Koffset is used to determine the very first pre-allocated UL grant after the reception of handover command.

Proposal 6	If RACH-less configuration is included in time-based CHO command, RACH-less handover configuration is valid only after T1 and is released after T2. TimeAlignmentTimer is started at T1.

R2-2309501	Discussion on RACH-less HO in NR NTN	CATT	discussion

R2-2309655	Remaining Issue on RACH-less for R18 NR NTN	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2309865	Discussion on RACH-less HO	TCL	discussion

R2-2309962	Views on some remaining issues for RACH-less HO in NTN	Lenovo	discussion	Rel-18

R2-2310178	RACH-less handover for NTN	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2310435	Remaining issue on RACH-less HO for NTN	ITL	discussion	Rel-18

R2-2310637	Final View on RACH-less HO in Rel-18 NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

R2-2310662	Remaining Issues of RACH-less (C)HO	NEC	discussion	Rel-18	NR_NTN_enh-Core

R2-2311019	Remaining issues on RACH-less HO for NR NTN	ETRI	discussion	Rel-18

Unchanged PCI

Whether / how to support Rel-17 UEs in unchanged PCI case

R2-2311227	Handover enhancements	Ericsson	discussion	Rel-18	NR_NTN_enh-Core	R2-2308900

Observation 9:	In the unchanged PCI scenario, the use of regular handover to handle Rel-17 RRC_CONNECTED UEs might pose implementation challenges.

Observation 10:	In the unchanged PCI scenario, the only feasible option for legacy UEs not supporting time-based CHO is RLF and Re-establishment which will result in a significant increase of interruption time.

Proposal 10:	Discuss how Rel-17 NTN UEs can use regular handover mechanism in the unchanged PCI service link switch.

We don’t consider the impact on Rel-17 UEs behavior (or Rel-18 UEs not supporting unchanged PCI) when defining the Rel-18 unchanged PCI solution

R2-2311223	“Unchanged PCI” solution vs “PCI change only” solution	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core	R2-2308753

Observation 1: “Reusing PCI after satellite changes” would increase interruption time, contrary to initial motivation

Observation 2: “Reusing PCI after satellite changes” also increases interruption time for Rel-17 UEs

Observation 3: A Rel-17 UE may not correctly resync the target cell in case of same cell handover

Observation 4: The HO message to Rel-17 UE needs to be sent after any source cell SSB to prevent resync on the source cell

Observation 5: The “next NTN-config” of the serving cell to be used by the UE after the service link switch could be broadcasted to reduce IT, but this is useless to Rel-17 UEs

Unchanged PCI details

R2-2310307	Satellite switching with unchanged PCI	Apple	discussion	Rel-18	NR_NTN_enh-Core

< Provision of target satellite information >

Observation 1: In order for UE to synchronize with the target satellite immediately after satellite switching, UE needs to acquire sync information of target satellite in advance.

Observation 2: According to existing RRC design, network can provide the sync information of target satellite in SIB19 only when the target satellite becomes the serving satellite and starts providing the service for the serving cell.

Observation 3: According to existing RRC design, UE can acquire the sync information of target satellite from SIB19 only after satellite switching without serving cell change.

Proposal 1: Network provides the sync information of target satellite in advance to UE before satellite switching via RRC signaling.

Network provides the sync information of target satellite in advance to UE before satellite switching, via broadcast signalling

Proposal 2: Network can provide the multiple satellite information (including serving satellite and the target satellite) for the serving cell in SIB19 and RRC dedicated signaling.

Continue in offline 307 on the details of information in broadcast signalling (e.g. for how many satellites)

< RACH-less satellite switching >

Proposal 3: It’s the optional UE capability to support the RACH-less satellite switching procedure.

Proposal 4: RACH-less satellite switching procedure is configured in SIB19.

Proposal 5: For RACH-less satellite switching, network may indicate the beam info for the dynamic grant reception in target satellite in SIB19, and UE starts monitoring the dynamic UL grant via the indicated beam after performing DL sync in the target satellite.

Proposal 6: For RACH-less satellite switching procedure, network may provide the configured grant and associated to beam info via RRC dedicated signaling, and UE selects the configured grant based on the detected SSB from the target satellite.

Proposal 7: For RACH-less satellite switching procedure, UE fallbacks to RACH-based satellite switching procedure if the beam associated to the UL grant in target satellite has RSRP value lower than a threshold.

Proposal 8: For RACH-less satellite switching procedure, network can set Nta value to 0 or same as source in SIB19.

Proposal 9: For RACH-less satellite switching procedure, UE resumes the UE dedicated transmission/reception via the 1st UL grant towards to target satellite.

< UE operation during satellite switching procedure >

Proposal 10: During satellite switching procedure, UE initiates PHR reporting after satellite switching.

Proposal 11: During satellite switching procedure, UE reinitiates all the serving cell related measurement, e.g. reset L3 filter for serving cell RRM measurement and reset the RLM.

Proposal 12: Introduced timer based failure detection mechanism for satellite switching procedure.

Proposal 13: When the satellite switching failure is detected, UE initiates the UE connection reestablishment procedure.

< Soft satellite switching >

Proposal 14: To support soft satellite switching, NW indicates the SSB for UE to detect the DL sync of target satellite in SIB19 in advance.

Continue in offline 307

Proposal 15: Network should configure a switching duration for the soft satellite switching.

Continue in offline 307

Proposal 16: Within the soft satellite switching duration, UE may keep on the transmission in source satellite till acquiring the target satellite’s DL sync.

Continue in offline 307

Proposal 17: The soft switching duration can be derived based on T-service/T-stop of source satellite and T-start of target satellite.

Continue in offline 307

< Coexistence with CHO/HO procedure>

Proposal 18: After satellite switching scheme is enabled, if UE receives the HO command before the switching period or switching point, UE will initiate the HO procedure immediately.

Proposal 19: Both CHO and satellite switching procedure can be configured simultaneously.

Proposal 20: When both CHO and satellite switching conditions are met, it's up to UE implementation to choose either one.

[AT123bis][307][NR-NTN Enh] Unchanged PCI (Apple)

Scope: continue the discussion on unchanged PCI aspects (marked for offline discussion in 307)

Intended outcome: offline summary

F2F schedule: Wednesday 2023-10-11 10:30-11:00 Brk3

Deadline for rapporteur's summary (in R2-2311319):  Thursday 2023-10-12 08:00

R2-2309502	Discussion on unchanged PCI mechanism	CATT	discussion

R2-2309656	Further Discussion on Service Link Switching with Unchanged PCI	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2309784	Handover Enhancement in LEO NTN: Unchanged PCI	MediaTek Inc.	discussion

R2-2309864	Discussion on unchanged PCI	TCL	discussion

R2-2309884	Discussion on remaining issue for unchanged PCI switch	ASUSTeK	discussion	Rel-18	NR_NTN_enh-Core

R2-2309961	Views on some remaining issues for PCI-unchanged scenario	Lenovo	discussion	Rel-18

R2-2310022	Discussion on remaining issues of PCI unchanged handover	Transsion Holdings	discussion	Rel-18

R2-2310034	Discussion on satellite switch with longer gap in conjunction with unchanged PCI	PANASONIC R&D Center Germany	discussion

R2-2310179	Details on satellite switch with PCI unchange	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2310225	Aquisition of target satellite information with PCI unchanged	China Telecom	discussion	Rel-18	NR_NTN_enh-Core

R2-2310247	Further discussion on PCI unchanged	CMCC	discussion	Rel-18	NR_NTN_enh-Core

R2-2310638	On How To Resolve Remaining Issues for Unchanged PCI (Satellite Switching without L3 Mobility)	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

R2-2310663	Remaining Details of Unchanged PCI Switch	NEC	discussion	Rel-18	NR_NTN_enh-Core

R2-2310696	Remaining issues on the unchanged PCI satellite switch	Google Inc.	discussion	Rel-18

R2-2310697	Discussion on the unchanged PCI scenario with optional random access	ETRI	discussion	Rel-18	NR_NTN_enh-Core

R2-2310845	Satellite switching without PCI change	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

CHO enhancements / Earth-moving cell reference locations

R2-2309654	Further Discussion on CHO Enhancements for NR NTN	vivo	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: For location-based CHO for earth-moving cells, UE derives the candidate cell’s reference location.

Proposal 2: For location-based CHO for earth-moving cells, the following information is included in CHO configuration:

-	Ephemeris and epochTime;

-	referenceLocation1 is reused to indicate the reference location at epochTime for the serving cell;

-	referenceLocation2 is reused to indicate the reference location at epochTime for the corresponding candidate cell.

Proposal 3: Delete the restriction that the network can not configure condEventD1 or condEventT1 independently (i.e., without a jointly configured measurement condition) in the field description of condExecutionCond in TS 38.331.

R2-2309883	Discussion on moving cell reference location for CHO	ASUSTeK	discussion	Rel-18	NR_NTN_enh-Core

R2-2310066	CHO Enhancements for NTN	Samsung Research America	discussion	Rel-18	NR_NTN_enh-Core

Common (C)HO configuration

R2-2310636	On Common HO Signalling for Rel-18 NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: As per the current HO signalling between the target and source cell, it is not possible to simply extract the servingCellConfigCommon and provide it within the source cell using broadcast signalling.

Observation 2: Enabling common signalling for NTN (C)HO requires specification work in RAN3 which, according to R3-234664 will not be pursued in Rel-18.

Proposal 1: Common signalling (e.g. using servingCellConfigCommon) for the purpose of (C)HO in NTN is not supported in Rel-18.

Proposal 2: RAN2 does not respond to R3-234664.

R2-2309500	Discussion on common (C)HO configuration	CATT	discussion

R2-2310768	Common handover signalling for NTN	Sony	discussion	Rel-18	NR_NTN_enh

R2-2310769	Signaling overhead reduction during NTN-NTN HOs	Sony	discussion	Rel-18	NR_NTN_enh

R2-2311212	Common signalling of HO common information	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core	R2-2308755

All aspects

R2-2309537	Discussion on handover enhancement for NR NTN	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2309863	Remaining issues on handover enhancements	LG Electronics France	discussion	Rel-18	38.331	NR_NTN_enh

R2-2310067	Open issues on NTN RACH-less HO and PCI unchanged switch	Samsung Research America	discussion	Rel-18	NR_NTN_enh-Core

R2-2310308	NR NTN specific HO enhancement	Apple	discussion	Rel-18	NR_NTN_enh-Core

R2-2310412	Discussion on handover enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	NR_NTN_enh-Core

R2-2310420	Discussion on handover enhancements for NTN-NTN mobility	Xiaomi	discussion

R2-2310561	Consideration on remaining issues on NTN HO enhancements	ZTE Corporation, Sanechips	discussion	Rel-18

R2-2311144	Discussion on NTN HO enhancements	Sharp	discussion	Rel-18	NR_NTN_enh-Core

### 7.25.4	Self-Evaluation NTN

(FS_IMT-2020_Sat_eval; leading Group: TSG RAN; REL-18; WID: RP-230736)

This will be treated in NTN breakout session (Sergio).

Study on Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology, including both NR NTN and IoT-NTN. Note that the time allocated will be very limited, and this is expected to be mostly an offline activity. Including outcome of [Post123][102][NTN Self Ev] CP/UP latency (Ericsson)

R2-2309714	Report of [Post123][102]NTN Self Ev] CPUP latency (Ericsson)	Ericsson	report	Rel-18

Adopt the following assumptions for the delay calculation of UP and CP latencies.

Agreed (assumption on processing time can be added)

Send an LS to RAN1 to inform about RAN2 progress and check the assumed delay values for UP and CP latency calculations.

Agreed

Adopt the attached TP to TS 37.911 about mobility interruption time.

Endorsed as a basis for further offline discussion during the week, also considering the input papers

[AT123bis][303][NTN Self Ev] ] CPUP latency (Ericsson)

Scope: continue the discussion on the TP also based on input papers

Intended outcome: updated TP and draft LS to RAN1

F2F schedule: FFS

Deadline for updated TP and LS (in R2-2311314 and R2-2311315):  Friday 2023-10-13 08:00

R2-2310086	Discussion on IMT-2020 Satellite self-evaluation for Latency	THALES	discussion	Rel-18	NR_NTN_enh

R2-2311112	CP and UP latency evaluation TP	Qualcomm Technologies Ireland	discussion	Rel-18	FS_IMT2020_SAT_eval

R2-2311215	[Draft] LS to RAN1 on RAN2 progress of NTN Self Evaluation	Huawei, HiSilicon	LS out	Rel-18	FS_IMT2020_SAT_eval	To:RAN1

Used as a basis for the LS to RAN1 (to be discussed in offline 303)

R2-2311233	Satellite IMT-2020 self-evaluation: CP latency	Ericsson	discussion	Rel-18

R2-2311234	Satellite IMT-2020 self-evaluation: UP latency	Ericsson	discussion	Rel-18

# Summary

Agreed CRs

NR-NTN

IoT-NTN

Approved LSs out

[Post123bis] Email discussions

Short

Long