3GPP TSG-RAN WG2 #123bis	R2-2309514

Xiamen, China, October 2023

Agenda Item:	7.25.2

Source:	OPPO

Title:	Left issue on Tx Switching

Document for:	Discussion, Decision

Introduction

This is to discuss the following issues for R18 Tx Switching.

Discussion

Firstly, based on the agreed CR

supportedBandPairListNR-r16, supportedBandPairListNR-v1700, supportedBandPairListNR-v18xy […] A UE supporting R18 dynamic UL Tx switching across up to 4 bands as specified in TS 38.214 [19] and TS 38.101-1 [15], should indicate both of supportedBandPairListNR-v18xy and supportedBandPairListNR-r16, and indicate supportedBandPairListNR-v1700 if the UE support 2Tx-2Tx switching on at least one band pair. The UE shall include all the possible band pairs and list the entries in the same order in supportedBandPairListNR-r16, supportedBandPairListNR-v1700, as well as supportedBandPairListNR-v18xy. For a band pair supporting 2Tx-2Tx switching, the UE should include switchingPeriodFor2T in ULTxSwitchingBandPair-v18xy and uplinkTxSwitchingPeriod2T2T in ULTxSwitchingBandPair-v1700, as well as uplinkTxSwitchingPeriod in ULTxSwitchingBandPair-r16. For a band pair supporting 1Tx-2Tx switching or 1Tx-1Tx switching, the UE should include switchingPeriodFor1T in ULTxSwitchingBandPair-v18xy and uplinkTxSwitchingPeriod in ULTxSwitchingBandPair-r16. For the band pair supporting 2Tx-2Tx switching, the UE always supports 1Tx-2Tx switching and 1Tx-1Tx switching. For the band pair supporting 1Tx-2Tx switching, the UE always support 1Tx-1Tx switching.

In case there are two bands, each supporting 1 MIMO layer, it is infeasible to support R16 1T2T switching, but cannot save the report of “uplinkTxSwitchingPeriod in ULTxSwitchingBandPair-r16”, since the switching period is mandatory field and we need to rely on the bandIndex in this IE anyway.

ULTxSwitchingBandPair-r16 ::=       SEQUENCE {

bandIndexUL1-r16                    INTEGER(1..maxSimultaneousBands),

bandIndexUL2-r16                    INTEGER(1..maxSimultaneousBands),

uplinkTxSwitchingPeriod-r16         ENUMERATED {n35us, n140us, n210us},

uplinkTxSwitching-DL-Interruption-r16 BIT STRING (SIZE(1..maxSimultaneousBands)) OPTIONAL

}

But good to confirm network would not understand UE support 1T2T switching based on this report, given the MIMO layer restriction.

ULTxSwitchingBandPair-r16, ULTxSwitchingBandPair-v1700, ULTxSwitchingBandPair-v18xy Indicates UE supports dynamic UL 1Tx-2Tx switching in case of inter-band CA, SUL, and (NG)EN-DC, and UL 2Tx-2Tx switching in case of inter-band CA and SUL as defined in TS 38.214 [12], TS 38.101-1 [2] and TS 38.101-3 [4]. The capability signalling comprises of the following parameters: - bandIndexUL1-r16 and bandIndexUL2-r16 indicate the band pair on which UE supports dynamic UL Tx switching. bandindexUL1/bandindexUL2 xx refers to the xxth band entry in the band combination. UE shall indicate support for 2-layer UL MIMO capabilities on one of the indicated two bands in each FeatureSet entry supporting UL 1Tx-2Tx switching and indicate support for 2-layer UL MIMO capabilities on both bands in each FeatureSet entry supporting UL 2T-2Tx switching, and only the band where UE supports 2-layer UL MIMO capability can work as carrier2 as defined in TS 38.101-1 [2] and TS 38.101-3 [4].

- R2 confirms the reporting of uplinkTxSwitchingPeriod in ULTxSwitchingBandPair-r16 for 2 bands each supporting 1 MIMO layer only would not be used by the network to configure 1T-2T switching.

Secondly, in 8209, P2, one left issue is as follows (postponed at 123 meeting)

Proposal 2: The NW indicates which BC from the reported UE capability is used for UL Tx switching to address the misunderstanding between UE and network on the UE capability (e.g. switching period) to be used.

The source of this issue is that UE may have different architecture w.r.t band-A/B/C, in case it is configured

- together with band-D, or

- together with band-E

And thus, results in different switching periods, mainly due to the different associations between band and PLL.

So far, we are not quite sure whether this issue is a typical issue, considering the UE architecture w.r.t the association between band and PLL could be fixed, regardless of the parent BC. So seems it is more reasonable to rely on R4 first to confirm the validity of the issue (we observed that R4 in the Toulouse meeting finally failed to send the LS out to R2).

- R2 postpone the discussion on different switching periods reported for different parent BC till receiving R4 LS.

On the other hand, it is not preferred to change UE architecture merely for a single feature, since the UE architecture may be decided by taking into account multiple factors, e.g., power consumption. Thus, an architecture may be adopted for power-saving reasons, even if it would lead to a longer switching period. In light of that, it is preferred to adopt a general solution for this issue (if exist), e.g., the network may assume the maximum value of the different possible switching period.

- If R2 decides to look for solution now, rely on network implementation to derive the switching period (e.g., maximum value between different parent BC:s).

Thirdly, based on R101 conclusion

The scenario {SUL band + corresponding NUL band} + {SUL band + corresponding NUL band} is supported in Rel-18 UL Tx switching framework with following restrictions.

  - For the band pair between {NUL and NUL}, “switched UL” or “dual UL” can be reported/configured.

  - For any other band pair including SUL (between {SUL and another SUL}, between {SUL and corresponding NUL}, and between {SUL and non-corresponding NUL}), only “switched UL” can be reported/configured in Rel-18.

  - Back-to-back transmission (i.e., switching without having switching gap) between {SUL and another SUL} and between {SUL and non-corresponding NUL} are not supported in Rel-18.

  - Note: this does not prevent a study and normative work to support simultaneous transmission between {SUL and another SUL} and/or between {SUL and non-corresponding NUL} as well as back-to-back transmissions between {SUL and another SUL} and/or between {SUL and non-corresponding NUL} in future release

The same restrictions are applied to the scenarios {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s).

  - For band pairs between {SUL and non-corresponding NUL} in this scenario, only “switched UL” can be reported/configured in Rel-18

  - Back-to-back transmission (i.e., switching without having switching gap) between {SUL and non-corresponding NUL} in this scenario is not supported in Rel-18

  - Note: this does not preclude a possible support based on other UE architecture than Rel-18 UL Tx switching

Clarify the restrictions above at least in TS38.101-1 e.g., section 5.5C “Configurations for SUL”

  - Note: TS38.214 may not need to capture the restrictions, and whether/how to capture restrictions in TS38.331/306 can be discussed in Q4 RAN2

It seems beneficial to clarify this in R2 spec.

- R2 captures the R#101 conclusion w.r.t SUL in TS 38.331, 38.306.

Conclusion

We have the following proposals:

Proposal 1	R2 confirms the reporting of uplinkTxSwitchingPeriod in ULTxSwitchingBandPair-r16 for 2 bands each supporting 1 MIMO layer only would not be used by the network to configure 1T-2T switching.

Proposal 2	R2 postpone the discussion on different switching periods reported for different parent BC till receiving R4 LS.

Proposal 3	If R2 decides to look for solution now, rely on network implementation to derive the switching period (e.g., maximum value between different parent BC:s).

Proposal 4	R2 captures the R#101 conclusion w.r.t SUL in TS 38.331, 38.306.