3GPP TSG-RAN WG2 Meeting #123bis		R2-2309754

Xiamen, China, October 9th - 13th, 2023

Source:	CATT

Title:	Discussion on LCID extension

Agenda Item:	7.0.2

Document for:	Discussion and Decision

Introduction

In Rel-18, LCID extension is discussed across multiple Rel-18 WIs. Generally, the intention of LCID extension discussed in these WIs is to transfer specific information in Msg3/MsgA to facilitate early identification for the network. However, considering the space of LCID in the existing Spec may be no more sufficient for future extension, enhancement of the LCID space was regarded as necessary and discussed in previous meetings [1]-[2]. In this document, we discuss the potential solution addressing the issue of LCID space shortage.

Discussion

2.1	Risk of LCID shortage in Rel-18

Several Rel-18 WIs are discussing early identification of related features via the LCID in Msg3/MsgA transmission, and thus considering introducing new LCIDs for these new features, e.g. eRedCap, PUCCH repetition for Msg.4 HARQ-ACK, capability restriction in MUSIM, etc., by occupying the reserved LCIDs in the current Spec. The situations are briefly reflected as follows:

Rel-18 NR NTN

The following agreements were made in [1]

*** possible options for RAN2 discussion *** Option 1: indicate by one R bit in a MAC subheader in Msg3 MAC PDU Option 2: new LCIDs for CCCH and CCCH1 for a non-RedCap UE in Msg3 MAC PDU Option 3: a new MAC CE with fixed size of zero bits in Msg3 MAC PDU Option 4: re-purpose the spare bit currently present in the RRC messages contained in UL CCCH or UL CCCH1. - We no longer consider options 3 and 4 - RAN2 confirms that the request/capability of PUCCH repetition for Msg4 HARQ-ACK via Msg3 higher layer signaling is feasible (can rediscuss if we cannot converge on a specific solution). Proposal 1: RAN2 to discuss how to coordinate the use of the remaining LCIDs between WIs. Proposal 2: RAN2 to discuss solutions to extend the LCID values without increasing the message size. - To be discussed in the common session in the main room

In order to indicate CCCH and CCCH1 separately, 2 LCIDs will be occupied in NTN.

In RAN#101, the following proposal was endorsed [3]

- PUCCH repetition discussed in Rel-18 NR NTN coverage enhancement is supported for the PUCCH transmissions when dedicated PUCCH resource configuration is not provided - No additional RAN1 work to support this behavior is assumed - It is confirmed that DAI field for dynamic indication of repetition factor can indicate '1' when multiple values configured by the gNB for repetition factors include '1' - FG 44-1 is used to indicate the support of this feature

Based on the agreement above, PUCCH repetition can be applied to the common PUCCH resources when dedicated PUCCH resource configuration is not provided. In this case, the PUCCH repetition can be applied for Msg4 HARQ-ACK for CBRA where C-RNTI can be transmitted in Msg3. Therefore, one LCID indicating C-RNTI together with PUCCH repetition for Msg4 HARQ-ACK should also be reserved.

Hence, if LCID in Msg3 is used to indicate PUCCH repetition request/capability in NR NTN, 3 LCIDs should be occupied in Rel-18 NTN.

Re-18 eRedCap

In Rel-18 eRedCap, the following agreement was reached in [2].

- All R18 eRedCap UEs uses the two new LCIDs for Msg3/MsgA PUSCH for CCCH/CCCH1 during Random Access, i.e., both those with peak rate reduction + BB BW reduction, and those with only peak rate reduction.

It can be seen that 2 LCIDs, which is similar as in Rel-17 redCap, will be used to indicate Msg3/MsgA PUSCH for CCCH/CCCH1 for eRedCap UE.

Rel-18 MUSIM enh.

In Rel-18 MUSIM, the following agreement was reached in [1]:

Proposal 1: During RRC setup and RRC resume procedure, UE indicates its capability is temporary restricted by sending CCCH message using dedicated LCID. Proposal 2: During RRC re-establishment procedure, UE indicates its capability is temporary restricted by sending CCCH message using dedicated LCID. - Using LCIDs would avoid any problems for RRC resume procedure. However, there are not many LCIDs left for UL and some other Rel-18 WIs also intend to use them. - FFS whether there is a need to use the LCIDs or whether we can reuse the legacy LCIDs. - Whether we can use the LCIDs (given that multiple WIs may be trying to use them) will be discussed in the main session. How to proceed LCID usage for MUSIM can be discussed in the next meeting based on the main session decision.

Similar to eRedCap, in order to distinguish CCCH and CCCH1 in Rel-18 MUSIM, 2 LCIDs will be occupied.

According to the analysis above, it can be seen that at least 7 LCIDs (3 in NTN, 2 in eRedCap and 2 in MUSIM) may need to be occupied in Rel-18, even without considering any combination of these features or any other new introduced MAC CEs (e.g. SL LBT MAC CE, enhanced PHR in Rel-18 CE, etc). However, there are only 7 LCIDs (37–42 and 47) left as reserved based on current specification [4]. Therefore, the reserved LCID would be completely depleted, if all above features were to be supported by the identification via LCID in Msg3/MsgA, and so the LCID in current specification would become not enough for any future extension (e.g. support of Rel-18 feature combinations, support of new features in Rel-19 on, etc.).

Observation 1: The reserved LCIDs in the current Spec will be depleted, in order to support the Rel-18 introduced features requiring identifications in Msg3/MsgA (including, but not limited to NR-NTN, eRedCap and MUSIM).

Observation 2: It may not be feasible any more to rely on reserved LCID values to support combination of these Rel-18 features or to support new fixed size MAC CE in future releases, based on the current LCID space in NR MAC Spec.

2.2	Solutions

Based on the current situation as in Observation 1/2, we think the extension of LCID space would be the choice to overcome the insufficient LCID space issue. Below the specific solution is discussed.

One of the candidate solutions is to use the R bit in the current MAC subheader to extend the LCID space. Based on the current spec, R bit will be set to zero by UE. Then, if an “R” bit is redefined as an LCID extension indicator (which can be alternatively renamed as e.g. “Ext”) and is set to 1, the corresponding LCID can be re-interpreted into new meanings, and this was discussed in [5].

Since the aforementioned Rel-18 WIs all intend to use LCID in the MAC subheader for early identification of CCCH/CCCH1/C-RNTI MAC CE during Msg3/MsgA transmission, which typically only involve in the MAC subheader format of R/R/LCID, we first focus on how the R bits are used in this MAC subheader format for the LCID extension. Note that the second R bit in the R/R/LCID MAC subheader can’t be used for the LCID extension. Because if it is used for extension, there is no way for the network to distinguish whether the second bit in a MAC subheader should be “F” field used in other MAC subheader formats, or the “Ext” field newly introduced as the LCID extension indicator.

As an example, the new defiend MAC subheader, after first R field redefined as “Ext”, is shown in Figure 1.

Figure 1: Ext/R/LCID MAC subheader

Note also that with this extension, the first R bit of all other MAC subheader formats will be changed to “Ext” as well, not only the Ext/R/LCID format shown in above Figure 1.

Someone had concerns on occupying this scarce R bit, arguing that this may prevent even further extension in future releases.  However, this should not be a concern from our perspective: for the MAC subheader currently with R/R/LCID format, there is still the other R bit kept for future use, even after the first R bit is occupied; whereas for the cases that other MAC subheader formats are applied, eLCID can typically be used for future extension.

Therefore, considering the LCID shortage problem analysed in Observation 1 and Observation 2 above, we propose the following way of LCID extension from this release.

Proposal 1: The first R bit in the MAC subheader is redefined as an LCID extension indicator (e.g. renamed “Ext”) from Release 18. When it sets to “1”, the LCID field refers to a new LCID table.

If the main room agrees to adopt the LCID extension as in Proposal 1, there are then 64 brand new LCID values available for selection, and it is preferable that the new LCID needed by Rel-18 NR NTN/eRedCap/MUSIM should be selected in this new Table. If more LCIDs are needed by Rel-18 WIs, they can also be selected in the new LCID Table. This can be a guideline provided by the Main room to other sessions.

Proposal 2: If Proposal 1 is agreed, guidance is provided to the related WIs (e.g. NR NTN/eRedCap/MUSIM) that the new LCIDs required are selected from the new LCID table.

Conclusion

In this document, we analyse issues for LCID extension accoss multiple WIs, and we find the observations and proposls as following:

Observation 1: The reserved LCIDs in the current Spec will be depleted, in order to support the Rel-18 introduced features requiring identifications in Msg3/MsgA (including, but not limited to NR-NTN, eRedCap and MUSIM).

Observation 2: It may not be feasible any more to rely on reserved LCID values to support combination of these Rel-18 features or to support new fixed size MAC CE in future releases, based on the current LCID space in NR MAC Spec.

Proposal 1: The first R bit in the MAC subheader is redefined as an LCID extension indicator (e.g. renamed “Ext”) from Release 18. When it sets to “1”, the LCID field refers to a new LCID table.

Proposal 2: If Proposal 1 is agreed, guidance is provided to the related WIs (e.g. NR NTN/eRedCap/MUSIM) that the new LCIDs required are selected from the new LCID table.

Reference

- R2-2309401, RAN2 #123 meeting report.

- R2-2307001, RAN2#122 meeting report.

- RAN#101 chairman notes.

- 3GPP TS 38.321 17.5.0, NR; Medium Access Control (MAC) protocol specification, (Release 17).

- R2-2308988, [offline-108] LCID extension, Huawei