3GPP TSG-RAN WG2 Meeting #123bis	R2-2310267

Xiamen, China, October 9th – 13th, 2023

Agenda Item:	7.11.3

Source:	CMCC

Title:	Discussion on shared processing

Document for:	Discussion

Introduction

Rapporteurs listed the open issues to progress the completion of Rel-18 MBS work[1], in this contribution, we’ll discuss the following open issues related to shared processing for broadcast:

FFS whether “location” needs to be also reported and how exactly this is captured in RRC (i.e. which IE is used)

Discussion

In RAN2#119bis meeting, it was agreed to report at least broadcast frequency, subcarrier spacing, and bandwidth [1].

In MBSInterestIndication, for a broadcast service that the UE is receiving or is interested to receive, at least the following information can be signalled: broadcast frequency, subcarrier spacing, and bandwidth. FFS details/exact parameters and other information. FFS in which scenarios the UE reports this information (e.g. intra-PLMN case, inter-PLMN case)

And in last meeting, CFR bandwidth was agreed to be reported, while whether and how to report the location is still FFS [2].

At least CFR bandwidth is reported by the UE in MII. FFS whether “location” needs to be also reported and how exactly this is captured in RRC (i.e. which IE is used).

For Rel-17, MII only reports the frequency and service information to the serving cell since the service interested by the UE is supported in the serving cell, therefore the detail information is not needed. However, for Rel-18 broadcast reception, the broadcast service to be received by UE is from non-serving cell, it’s beneficial to indicate the “location”.

Proposal 1: location of the CFR should be reported in MII for broadcast reception from non-serving cell.

In Rel-17, CFR for broadcast is indicated in SIB 20 CFR-ConfigMCCH-MTCH, locationAndBandwidthBroadcast is actually used to indicate the frequency domain resource used by the broadcast service [3].

CFR-ConfigMCCH-MTCH information element

-- ASN1START

-- TAG-CFR-CONFIGMCCH-MTCH-START

CFR-ConfigMCCH-MTCH-r17 ::= SEQUENCE {

locationAndBandwidthBroadcast-r17          LocationAndBandwidthBroadcast-r17  OPTIONAL,  -- Need S

pdsch-ConfigMCCH-r17                       PDSCH-ConfigBroadcast-r17          OPTIONAL,  -- Need S

commonControlResourceSetExt-r17            ControlResourceSet                 OPTIONAL   -- Cond NotSIB1CommonControlResource

}

LocationAndBandwidthBroadcast-r17 ::= CHOICE {

sameAsSib1ConfiguredLocationAndBW          NULL,

locationAndBandwidth                       INTEGER (0..37949)

}

-- TAG-CFR-CONFIGMCCH-MTCH-STOP

-- ASN1STOP

CFR-ConfigMCCH-MTCH field descriptions

commonControlResourceSetExt An additional common control resource set which may be configured and used for searchSpaceMCCH/searchSpaceMTCH or UE-specific search space in the BWP where searchSpaceMCCH is configured. It is contained in the bandwidth of locationAndBandwidthBroadcast.

locationAndBandwidthBroadcast Indicates starting PRB and the number of PRBs of CFR used for MCCH and MTCH reception. Value sameAsSib1ConfiguredLocationAndBW means the CFR for broadcast has the same location and size as the locationAndBandwidth for initial BWP configured in SIB1. Value locationAndBandwidth is used to configure CFR with bandwidth that is larger than and fully contains the bandwidth for the initial DL BWP and CORESET#0 configured in SIB1. If the field is absent, the CFR for broadcast has the same location and size as CORESET0.

pdsch-ConfigMCCH Indicates PDSCH parameters used for MCCH transmission. If the field is absent, PDSCH paramers used for MCCH are the same as those of PDSCH configuration provided in initialDownlinkBWP in SIB1.

Proposal 2: locationAndBandwidthBroadcast can be used to indicate the bandwidth and location A of CFR used for broadcast reception from non-serving cell.

Conclusions

In this contribution, we analyse the open issue of shared processing, following are proposals.

Proposals:

Proposal 1: location of the CFR should be reported in MII for broadcast reception from non-serving cell.

Proposal 2: locationAndBandwidthBroadcast can be used to indicate the bandwidth and location A of CFR used for broadcast reception from non-serving cell.

References

- R2_119bis_meeing_report;

- R2_123_meeing_report;

- 3GPP TR 38.331 RRC.