3GPP TSG-RAN WG2 #123-bis	R2-2310545

Xiamen, China, Oct 9th - 13th, 2023

Agenda Item:	6.4.1

Source:	ZTE Corporation

Title:	Discussion on LocationMeasurementIndication procedure for positioning

Document for:	Discussion and Decision

1 Introduction

In Rel-16 spec 38.331-gd0 clause 5.5.6.2[1], there is the following description for MG request and stop:

#### 5.5.6.2 Initiation The UE shall: 1> if and only if upper layers indicate to start performing location measurements towards E-UTRA or NR or start subframe and slot timing detection towards E-UTRA, and the UE requires measurement gaps for these operations while measurement gaps are either not configured or not sufficient: 2> initiate the procedure to indicate start; NOTE 1: The UE verifies the measurement gap situation only upon receiving the indication from upper layers. If at this point in time sufficient gaps are available, the UE does not initiate the procedure. Unless it receives a new indication from upper layers, the UE is only allowed to further repeat the procedure in the same PCell once per frequency of the target RAT if the provided measurement gaps are insufficient. 1> if and only if upper layers indicate to stop performing location measurements towards E-UTRA or NR or stop subframe and slot timing detection towards E-UTRA: 2> initiate the procedure to indicate stop. NOTE 2: The UE may initiate the procedure to indicate stop even if it did not previously initiate the procedure to indicate start.

In Rel-17 spec 38.331-h50 clause 5.5.6.2[2], there is the following description for MG request and stop:

#### 5.5.6.2 Initiation The UE shall: 1> if and only if upper layers indicate to start performing location measurements towards E-UTRA or NR or start subframe and slot timing detection towards E-UTRA, and the UE requires measurement gaps for these operations while measurement gaps are either not configured or not sufficient: 2> if preconfigured measurement gaps for positioning and posMG-Request are configured and the UE considers that at least one of the preconfigured measurement gaps for positioning is sufficient for the location measurement when activated: 3> trigger the lower layers to initiate the measurement gap activation request using UL MAC CE as specified in TS 38.321 [3]; 2> else: 3> initiate the procedure to indicate start as specified in clause 5.5.6.3; NOTE 1: The UE verifies the measurement gap situation only upon receiving the indication from upper layers. If at this point in time sufficient gaps are available, the UE does not initiate the procedure. Unless it receives a new indication from upper layers, the UE is only allowed to further repeat the procedure in the same PCell once per frequency of the target RAT if the provided measurement gaps are insufficient. NOTE 1a: When indication is received from upper layers for performing location measurement and there is pre-configured measurement gap configured (not preconfigured measurement gap for positioning), the UE considers this preconfigured measurement gap to be not sufficient if the measurement gap is not considered to be always activated according to clause 9.1.7.2 of TS 38.133 [14]. 1> if and only if upper layers indicate to stop performing location measurements towards E-UTRA or NR or stop subframe and slot timing detection towards E-UTRA: 2> if there is no activated preconfigured measurement gap for positioning: 3> if there is previously triggered UL MAC CE transmission for the measurement gap activation for positioning: 4> indicate to the lower layers to cancel the triggered UL MAC CE transmission for the measurement gap activation as specified in TS 38.321 [3]; 3> else: 4> initiate the procedure to indicate stop as specified in 5.5.6.3. 2> else if there is activated preconfigured measurement gap for positioning: 3> trigger the lower layers to deactivate all the activated measurement gap(s) for positioning as specified in TS 38.321 [3]. NOTE 2: The UE may initiate the procedure to indicate stop even if it did not previously initiate the procedure to indicate start.

This paper discusses the issue that Rel-17 LocationMeasurementIndication procedure does not suit for a scenario where a UE maintains two positioning-use gaps(one of them is pre-configured MG for positioning) simultaneously to perform positioning.

2 Discussion

Measurement gap has the following types:

- Legacy Measurement gap -introduced before Rel-17

- Pre-configured measurement gap -introduced in Rel-17

- Pre-configured measurement gap for positioning -introduced in Rel-17

- MUSIM gap -introduced in Rel-17

- Concurrent measurement gaps -introduced in Rel-17

- NTN gap -introduced in Rel-17

Rel-17 RRC spec has specified that:

posMeasGapPreConfigToAddModList List of preconfigured measurement gap for positioning to add and/or modify. All the gaps configured are associated with the measurement of PRS for RSTD, UE-RxTx Time Difference, PRS-RSRP and PRS-RSRPP as defined in TS 38.215 [9]. In this version of the specification, the network does not configure preconfigured measurement gap for positioning together with concurrent measurement gap or MUSIM gap.

It is specified that UE will not be configured with pre-configured MG for positioning and concurrent measurement gap or MUSIM gap in different FRs.

Furthermore, in Rel-17, the following UE capability for positioning-use gap in different FRs is agreed, see 38.306 section 4.6.9:

| independentGapConfigPRS-r17 Indicates whether the UE supports two independent measurement gap configurations for FR1 and FR2 for PRS measurement, as specified in clause 9.1.2 of TS 38.133 [5]. | UE | No | No | No |
| --- | --- | --- | --- | --- |

Observation 1: The UE, if supports the corresponding capability, can use two positioning-use gaps simultaneously in different FRs, excluding the case that preconfigured measurement gap for positioning to be used together with concurrent measurement gap or MUSIM gap.

The current Rel-17 LocationMeasurementIndication procedure is simplified as follows:

- When UE’s RRC receives UE’s higher layer’s indication that a PRS frequency needs to be measured, UE’s RRC check whether the current MG can also measure this PRS frequency: - If yes, UE uses the current available MG to measure DL PRS; - If no: - if there is no pre-configured MG for positioning can be used, UE sends UL RRC message to the gNB to request for a MG; - if there is pre-configured MG for positioning can be used, UE sends UL MAC CE to activate the pre-configured MG for positioning. - When UE’s RRC receives UE’s higher layer’s indication that to stop performing positioning measurement: - if there is no activated pre-configured MG for positioning: - If there is triggered UL MAC CE but not send yet, UE indicate MAC layer to cancel the triggered UL MAC CE transmission; - If there is no triggered UL MAC CE but not send yet, UE sends the UL RRC message to the gNB to stop measurement; - if there is activated pre-configured MG for positioning, UE sends the UL MAC CE to deactivate the activated pre-configured MG for positioning.

If pre-configured gap for positioning and other measurement gaps (typically legacy MG and pre-configured MG) can work together within different FRs, In this case, if UE wants to stop performing positioning measurement, based on current procedure above:

- If the UE uses a FR1 legacy MG for positioning, and the UE also triggers an UL MAC CE activation request of pre-configured MG for positioning for FR2 but not send yet. So at this time, according to the procedure, UE can only indicate to the lower layers to cancel the triggered UL MAC CE transmission. However, the FR1 legacy MG used for positioning will not be stopped, and NW will not receive the UL RRC message LocationMeasurementIndication with ‘stop’.

- If the UE uses two gaps to measure positioning measurements simultaneously, one is for FR1 legacy MG, the other is for FR2 activated pre-configured MG for positioning. Because UE has an activated pre-configured MG for positioning, UE can only send the UL MAC CE to deactivate it. However, the FR1 legacy MG used for positioning will not be stopped, and NW will not receive the UL RRC message LocationMeasurementIndication with ‘stop’.

It can be seen that the legacy ‘stop’ procedure does not consider when there are two gaps for positioning working simultaneously in different FRs. If NW does not receive the UL RRC message to indicate ‘stop’, NW will not know whether UE still wants to continue performing positioning measurement or not at least in above two cases. This is not the intention of introducing LocationMeasurementIndication at the very first time, and it is also not beneficial for gNB’s control and further scheduling, since it may cause the schedule performance reduction due to considering the redundant measurement gap configuration.

Observation 2：The current Rel-17 LocationMeasurementIndication ‘stop’ procedure does not suit for a case where a UE maintains one pre-configured MG for positioning and other measurement gaps to perform positioning(typically legacy MG and pre-configured MG) simultaneously.

To solve the problem analysed above, we suggest to modify the RRC spec as below:

#### 5.5.6.2 Initiation The UE shall: 1> if and only if upper layers indicate to start performing location measurements towards E-UTRA or NR or start subframe and slot timing detection towards E-UTRA, and the UE requires measurement gaps for these operations while measurement gaps are either not configured or not sufficient: 2> if preconfigured measurement gaps for positioning and posMG-Request are configured and the UE considers that at least one of the preconfigured measurement gaps for positioning is sufficient for the location measurement when activated: 3> trigger the lower layers to initiate the measurement gap activation request using UL MAC CE as specified in TS 38.321 [3]; 2> else: 3> initiate the procedure to indicate start as specified in clause 5.5.6.3; NOTE 1: The UE verifies the measurement gap situation only upon receiving the indication from upper layers. If at this point in time sufficient gaps are available, the UE does not initiate the procedure. Unless it receives a new indication from upper layers, the UE is only allowed to further repeat the procedure in the same PCell once per frequency of the target RAT if the provided measurement gaps are insufficient. NOTE 1a: When indication is received from upper layers for performing location measurement and there is pre-configured measurement gap configured (not preconfigured measurement gap for positioning), the UE considers this preconfigured measurement gap to be not sufficient if the measurement gap is not considered to be always activated according to clause 9.1.7.2 of TS 38.133 [14]. 1> if and only if upper layers indicate to stop performing location measurements towards E-UTRA or NR or stop subframe and slot timing detection towards E-UTRA: 2> if there is no activated preconfigured measurement gap for positioning: 3> if there is previously triggered UL MAC CE transmission for the measurement gap activation for positioning: 4> indicate to the lower layers to cancel the triggered UL MAC CE transmission for the measurement gap activation as specified in TS 38.321 [3]; 2> else if there is activated preconfigured measurement gap for positioning: 3> trigger the lower layers to deactivate all the activated measurement gap(s) for positioning as specified in TS 38.321 [3]. 2> initiate the procedure to indicate stop as specified in 5.5.6.3. NOTE 2: The UE may initiate the procedure to indicate stop even if it did not previously initiate the procedure to indicate start.

With this change, when stoping the positioning measurement, UE should firstly determine whether there exists activated pre-configured MG for positioning:

- If no, UE should trigger low layer to cancel the UL MAC CE activation(if exists), then directly send the UL RRC message LocationMeasurementIndication to indicate ‘stop’, like Rel-15 and Rel-16 legacy behaviour.

- If yes, UE should send UL MAC CE to deactivate it, UE should also indicate network by initiating the ‘stop’ using UL RRC LocationMeasurementIndication.

A draft CR is also provided in R2-2310575 [3].

Proposal 1: Adopt the CR R2-2310575.

3 Conclusion

Based on the discussion above we propose the following proposals:

Observation 1: UE, if supports the corresponding capability, can use two positioning-use gaps simultaneously in different FRs, excluding preconfigured measurement gap for positioning together with concurrent measurement gap or MUSIM gap.

Observation 2：The current Rel-17 LocationMeasurementIndication ‘stop’ procedure does not suit for a case where a UE maintains one pre-configured MG for positioning and other measurement gaps to perform positioning(typically legacy MG and pre-configured MG) simultaneously.

Proposal 1: Adopt the CR R2-2310575.

4 Reference

- TS 38.331-gd0, Radio Resource Control (RRC) protocol specification

- TS 38.331-h50, Radio Resource Control (RRC) protocol specification

[2] R2-2310575 Correction on LocationMeasurementIndication for positioning   ZTE Corporation