3GPP TSG-RAN WG2 Meeting #123bis	R2-2311571

# Xiamen, China, October 9th – 13th, 2023

Source: 	KDDI Corporation

Title: 	Offline discussion summary for Non-collocated signaling

Agenda Item:	7.25.1	RAN4 led items

Document for:	Discussion

- Introduction

This contribution is a summary of the following offline discussion.

Agreements

- RAN2 agree to introduce a new RRC signaling to indicate capability restriction is type 1 or type 2 only for Rel-18 UE and not to introduce for Rel-17/16/15 UE

- Detailed statements capturing detailed RAN4 spec references in RAN2 specs without terminologies “Type1” and “Type2”.

- For EN-DC - Some Rel-18 UE support the new BS signaling and some Rel-18 UE do not support.  A new UE capability will be introduced for this

- Introduce a new UE capability which indicates the support of the new BS signaling per UE. This UE capability is only applicable to the UE indicating “interBandMRDC-WithOverlapDL-Bands-r16”.   The new capability is included in MRDC container

- Introduce a new Per BC UE capability to indicate the supporting TDD-TDD intra-band Non-Collocated NR-CA.

-	UE supporting the new UE capability supports both Type 1 and Type 2 capability requirements, while the UE not supporting the new UE capability supports Type 1 capability requirement only.

- [CB for Thursday] if we introduce a new per CG configuration into TS36.331 and TS38.331 to indicate Type1/Type 2 requirement to apply

- Discussion

During the offline discussion, the following comments were provided.

- Solution should avoid the LTE Specification change and limit the signaling impact to NR specifications.

- Solution should support simultaneous operation non-collocated EN-DC and collocated NR-CA for the future proof.

- For NR-CA indication, "per-CG” signaling is suitable, based on the following assumption

  - Collocation/non-collocation property is always uniform among different CCs of the intra-band TDD-TDD CA. (For example, A1+A2+A3, A1+A2 is collocated, then A2+A3 is also collocated.)

  - Collocation/non-collocation property is always uniform among different intra-band TDD-TDD CA components within a CG. (For example, A1+A2+B1+B2, A1+A2 is collocated, then B1+B2 is also collocated.)

With this, rapporteur propose as follows.

Proposal: RAN2 agree to introduce two separate indications under IE ”CellGroupConfig” in TS38.331, one is for EN-DC operation and the other is for NR-CA.

- 3. Conclusion

Proposal: RAN2 agree to introduce two separate indications under IE ”CellGroupConfig” in TS38.331, one is for EN-DC operation and the other is for NR-CA.