3GPP TSG RAN WG2 #124                                                                         R2-2312241

Chicago, USA, Nov. 13th – 17th, 2023                                    Revision of R2-2310830

Agenda Item:	7.19.2

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues of enhanced eDRX in RRC_INACTIVE

Document for:	Discussion and Decision

Introduction

For R18 eRedCap WID, long eDRX supporting in RRC_INACTIVE has been supported and the running CRs have been progressed. But there are still some Editor's notes to be addressed.

In this contribution, we will discuss the related issues and give our proposals.

Discussions

In the 38331_(Rel-18)_Running CR for eRedCap [1], the new parameter ran-ExtendedPagingCycle-r18 is defined, which further comprise two parameters extendedPagingCycle-r18 and extendedPagingPTW-r18 as follows:

SuspendConfig field descriptions

ncd-SSB-RedCapInitialBWP-SDT Indicates that the UE uses the (e)RedCap-specific initial DL BWP associated with the NCD-SSB for SDT. The network configures this field if an (e)RedCap UE is configured with SDT in the (e)RedCap-specific initial DL BWP not associated with CD-SSB. If configured, the NCD-SSB indicated by this field can only be used during the SDT procedure for CG-SDT or RA-SDT.

extendedPagingCycle The eDRX cycle longer than 10.24 s for RAN-initiated paging to be applied by the UE. Value hf2 corresponds to 2 hyper frames, value hf4 corresponds to 4 hyper frames and so on. Value of the field indicates an eDRX cycle longer than 10.24 s which is shorter than or equal to the IDLE mode eDRX cycle configured for the UE.

extendedPagingPTW The length of paging transmission window for RAN-initiated paging to be applied by the UE as defined in TS 38.304 [20]. Value ms1280 corresponds to 1280 miliseconds, value ms2560 corresponds to 2560 miliseconds and so on.

ran-ExtendedPagingCycle The extended DRX (eDRX) cycle for RAN-initiated paging to be applied by the UE. Value rf256 corresponds to 256 radio frames, value rf512 corresponds to 512 radio frames and so on. Value of the field indicates an eDRX cycle which is shorter or equal to the IDLE mode eDRX cycle configured for the UE.

ran-NotificationAreaInfo Network ensures that the UE in RRC_INACTIVE always has a valid ran-NotificationAreaInfo.

ran-PagingCycle Refers to the UE specific cycle for RAN-initiated paging. Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on.

sl-UEIdentityRemote Indicates the C-RNTI to the L2 U2N Remote UE.

t380 Refers to the timer that triggers the periodic RNAU procedure in UE. Value min5 corresponds to 5 minutes, value min10 corresponds to 10 minutes and so on.

| Conditional Presence | Explanation |
| --- | --- |
| L2RemoteUE | The field is mandatory present for L2 U2N Remote UE's RNAU; otherwise it is absent. |
| RANPaging | This field is optionally present, Need R, if the UE is configured with IDLE eDRX, see TS 24.501 [23]; otherwise the field is not present. |
| Redirection2 | The field is optionally present, Need R, if redirectedCarrierInfo is included; otherwise the field is not present. |

Editor's note: The naming of the following parameters is FFS: extendedPagingCycle, extendedPagingPTW, ran-ExtendedPagingCycle-r18, and ExtendedPagingCycle-Config-r18

Since extendedPagingCycle is included in ran-ExtendedPagingCycle-r18, it can clearly indicate the RAN extended paging cycle when ran-ExtendedPagingCycle-r18 is configured, we think the name of both extendedPagingCycle and ran-ExtendedPagingCycle-r18 are suitable.

Observation 1: The parameter extendedPagingCycle included in ran-ExtendedPagingCycle-r18 can clearly indicate the parameter intention, e.g., to indicate the extended RAN paging cycle. So both extendedPagingCycle and ran-ExtendedPagingCycle-r18 are suitable. Accordingly, the name for the IE structure ExtendedPagingCycle-Config-r18 is also suitable.

Proposal 1: To keep the name of parameters extendedPagingCycle-r18, ran-ExtendedPagingCycle-r18, and ExtendedPagingCycle-Config-r18 unchanged.

Considering that one intention in R18 eRedCap WID is to extend the RAN paging cycle length, not extend paging message and not extend PTW, e.g., the length of the PTW for extended RAN paging cycle still can be same as that for other cases (non-extended RAN paging or CT paging), the parameter name of extendedPagingPTW seems not so suitable. In TS 38.304, it is described as “Paging Time Window (PTW) length (in seconds) configured by RRC” . So, the parameter name extendedPagingPTW is suggested to be pagingPTWLength or just pagingPTW.

Observation 2: The intention in R18 eRedCap WID is mainly to extend the RAN paging cycle length, but not to extend paging message and not to extend PTW.

Proposal 2: To change the name of parameter extendedPagingPTW-r18 to pagingPTWLength-r18 or just pagingPTW-r18.

Conclusions

This contribution presents some of our views on the remaining issue of eDRX enhancements for Rel-18 eRedCap. The following observations and proposals are given:

Observation 1: The parameter extendedPagingCycle included in ran-ExtendedPagingCycle-r18 can clearly indicate the parameter intention, e.g., to indicate the extended RAN paging cycle. So both extendedPagingCycle and ran-ExtendedPagingCycle-r18 are suitable. Accordingly, the name for the IE structure ExtendedPagingCycle-Config-r18 is also suitable.

Observation 2: The intention in R18 eRedCap WID is mainly to extend the RAN paging cycle length, but not to extend paging message and not to extend PTW.

Proposal 1: To keep the name of parameters extendedPagingCycle-r18, ran-ExtendedPagingCycle-r18, and ExtendedPagingCycle-Config-r18 unchanged.

Proposal 2: To change the name of parameter extendedPagingPTW-r18 to pagingPTWLength-r18 or just pagingPTW-r18.

References

- R2-231XXXX_38331_(Rel-18)_Running CR for eRedCap_v13_clean_Rapporteur, November 2023

- draft R2-230xxxx_38304_Running 38.304 CR for eRedCap_v05_Rapp, November 2023