3GPP TSG RAN WG2#124	R2-2311713

Chicago, USA, 13th - 17th November 2023

3GPP TSG-RAN WG1 Meeting #114bis 	R1-2310578

# Xiamen, China, October 9th – October 13th, 2023

Title:	SP-CSI reporting for network energy savings

Response to:

Release:	Rel-18

Work Item:	Netw_Energy_NR-Core

Source:	RAN1

To:	RAN2

Contact Person:

#### Name:	Yi Wang

E-mail Address:	wangyi6@Huawei.com

Attachments:	None

1. Overall Description:

RAN1 has discussed the SP-CSI reporting related issues and made the following agreements

•  From RAN1 perspective, up to 4 CSI report configurations can be configured in a BWP for SP CSI reporting on PUCCH where one or more report configurations can contain a list of sub-configuration(s).

Furthermore, it is agreed that

•  For the max number of sub-configurations Lmax in one CSI report configuration, the maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH

•  For report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration, the maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH.

2. Actions:

To RAN2:

ACTION: 	RAN1 kindly ask RAN2 to take the above into account in their future work related to the MAC-CE design for SP-CSI reporting for Rel-18 NES.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #115 	13 - 17 November 2022	Chicago, US

TSG-RAN WG1 Meeting #116 	26 February – 1 March 2023	Athens, Greece