3GPP TSG-RAN WG2 Meeting #124	R2-2xxxxxx

Chicago, USA, Nov. 13th – 17th, 2023

Agenda Item:	7.9.1

Source:	NEC

Title:	(TP for TS 38.300) on mode 1 resource allocation of inter-DU U2N Remote UE

Document for:	Decision

1	Introduction

During RAN2 #122 meeting, the following agreement is agreed:

Agreement: For Scenario-1, mode-1 scheduling for remote UE is supported at least for intra-DU case, with the SR/BSR and grant sent on the direct path; whether it is supported for inter-DU case is up to R3, but R2 do not intend to make specification changes to support this case, and for specification purposes RAN2 intend to model it as a single MAC entity at the UE.

Thereafter, RAN2 has sent the relevant LS[1] towards RAN3 for informing. Correspondingly, in this meeting, RAN3 has replied the LS[2] to RAN2 to inform that mode 1 resource allocation for U2N remote UE under inter-DU case is not supported.

So in this contribution, we will discuss how to reflect RAN3’s agreement into RAN2 specification.

2	Discussion

Previously, RAN2 has agreed that no matter inter-DU mode 1 for U2N remote UE is supported or not, RAN2 will not purse any spec impact. One understands the intention of this saying is to not make any further enhancement related to inter-DU case at RAN2 side.

However, currently, RAN3 decided not to support the inter-DU case. So next one should consider how to reflect RAN3 agreement at RAN2 side to avoid any further chance of related enhancement in the following in this release.

RAN2 should reflect RAN3’s agreement that not to support mode 1 resource allocation of U2N remote UE under inter-DU multi-path relay case in RAN2’s specification.

In details, such avoidance can be relied on network implementation, so there is no need to have detailed normative text in TS 38.331. Instead, it is enough to use stage 2 spec to carry this restriction since there is already related description captured in the corresponding stage 2 spec sections. So, it is suggested to adopt the corresponding TP in the annex section to reflect RAN3 agreement to get minimum spec impact.

It is suggested to adopt the corresponding TP into stage 2 specification to reflect that U2N Remote UE mode 1 resource allocation of U2N remote UE under inter-DU multi-path relay case is not supported in this release.

3	Conclusion and Proposals

In this contribution, we have discussed the remaining issues of U2N multi-path relay, and the following observations and proposals are given out:

Proposal 1	RAN2 should reflect RAN3’s agreement that not to support mode 1 resource allocation of U2N remote UE under inter-DU multi-path relay case in RAN2’s specification.

Proposal 2	It is suggested to adopt the corresponding TP into stage 2 specification to reflect that U2N Remote UE mode 1 resource allocation of U2N remote UE under inter-DU multi-path relay case is not supported in this release.

4	References

[1]	R2-2306781_LS to RAN3 on mode 1 scheduling in inter-DU multi-path case	NEC

[2]           R2-2311724_ Reply LS to RAN2 on mode 1 scheduling in inter-DU multi-path      NEC

5	Annex

16.X	Multi-path Relay

16.X.1	General

In multi-path relay, a MP Remote UE is connected to a single gNB via one direct path and one indirect path while the MP Remote UE is in RRC_CONNECTED state. For the indirect path, both L2 and L3 MP Relay architectures are supported. The L3 MP Relay architecture is transparent to the serving NG-RAN of the MP Relay UE, except for controlling sidelink resources. In the case of MP Remote UE using PC5 indirect path, mode 1 resource allocation is supported only for intra-DU case, with the SR/BSR and grant sent on the direct path.

In multi-path relay, the interface between MP Remote UE and MP Relay UE can be either PC5 orN3C. When the interface between MP Remote UE and MP Relay UE is N3C interface, the relationship of MP Remote UE and MP Relay UE is pre-configured or static, and it is up to implementation how to pre-configure or make it static.

Multi-path relay supports MP Remote UE and MP Relay UE when they are in the intra/inter-frequency cells, and PCell is always on the direct path.

Multi-path relay is supported in the following cell deployment scenarios:

-  The MP Relay UE and MP Remote UE are served by the same cell.

-  The MPRelay UE and MP Remote UE are served by different intra-frequency cells of the same gNB.

-  The MP Relay UE and MP Remote UE are served by different inter-frequency cells of the same gNB.

Multi-path relay is supported in the following sidelink scenarios:

-  Sidelink TX/RX and Uu link share the same carrier at the MP Remote UE.

-  Sidelink TX/RX and Uu link use different carriers at the MP Remote UE.

-  Sidelink TX/RX and Uu link share the same carrier at the MP Relay UE.

-	Sidelink TX/RX and Uu link use different carriers at the MP Relay UE.