3GPP TSG-RAN WG2 Meeting #124                                                                    R2-2312417

Chicago, USA, November 13th – 17th, 2023

Source:	CATT

Title:	Further Consideration on Service Continuity Enhancements

Agenda Item:	7.9.3

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to discuss the three open issues based on the summary of [POST123bis] [414][Relay] Rel-18 service continuity.

Discussion

Issue1: Details when the target Relay is IDLE/INACTIVE

By the discussion in last meeting, for the inter-gNB x-to-indirect path switch, if the target relay is in idle/inactive mode, it is a risk that the target gNB could not establish Uu RRC connection with this UE and the path switch will be failed, so some optimization, such as RRC state of L2 U2N relay UE included in U2N relay discovery announcement/response message, maybe needed. In our view, it also has some risk that the target gNB will lose its RRC connection with the target relay UE in RRC Connected mode during the path switch, so it is not necessary to indicate the target relay is in idle/inactive or connected mode. Besides, as smart target gNB, it can choose the RRC connected relay UE with high priority as the target relay UE, which is a kind of network implementation. Hence, reusing legacy procedure for inter-gNB direct-to-indirect path switching is feasible when the target Relay is IDLE/INACTIVE, no obvious issue is identified.

Proposal 1: No further enhancement is needed to guarantee the successful RRC connection of Relay UE for the case of inter-gNB direct-to-indirect path switching when target Relay is IDLE/INACTIVE.

Issue2: The long latency of inter-gNB x-to-indirect path switch with IDLE/INACTIVE target relay UE

For inter-gNB x-to-indirect path switch with IDLE/INACTIVE target relay UE, there are lots of CP signaling needs to be exchanged, it seems that the delay of this case is considerably long. However, compared to the intra-gNB path switch, the delay difference is introduced by path switch request/response among source gNB and target gNB, there is no HARQ feedback and RLC retransmission in these two steps, so the delay difference among inter-gNB case and intra-gNB case may be acceptable, furthermore, considering it is ending of R18, no further optimization is needed to short the delay.

Proposal 2: No further enhancement is needed to short the delay for the case of inter-gNB direct-to-indirect path switching when target Relay is IDLE/INACTIVE.

Issue3: ASN.1 details of “whether it is reporting SL-RSRP and SD-RSRP.”

Based on below running CR [1], one enumerated variable is applied to report SL relay measurement result, which could be used to indicate the measurement result is sl-rsrp or sd-rsrp.

SL-MeasResultListRelay-r17 ::= SEQUENCE (SIZE (1..maxNrofRelayMeas-r17)) OF SL-MeasResultRelay-r17 SL-MeasResultRelay-r17 ::= SEQUENCE { cellIdentity-r17 CellAccessRelatedInfo, sl-RelayUE-Identity-r17 SL-SourceIdentity-r17, sl-MeasResult-r17 SL-MeasResult-r16, ..., sl-MeasQuantity-r18 ENUMERATED { sl-rsrp, sd-rsrp } OPTIONAL }

One open issue is whether we could replace it by choice structure as follows.  The proposed choice structure with r17 will have impact to legacy R17 UE, it is NBC issue. If the choice structure is applied from r18, compared with above yellow part, obviously, the enumerated type seems more concise. Hence it is better to follow the current running CR, no further revise for choice structure is needed.

SL-MeasResult-r17 ::= CHOICE { Sl-rsrp SL-MeasResult-r16, Sd-rsrp SL-MeasResult-r16 }

Proposal 3: Follow the running CR and apply the enumerated variable to report SL relay measurement result.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: No further enhancement is needed to guarantee the successful RRC connection of Relay UE for the case of inter-gNB direct-to-indirect path switching when target Relay is IDLE/INACTIVE.

Proposal 2: No further enhancement is needed to short the delay for the case of inter-gNB direct-to-indirect path switching when target Relay is IDLE/INACTIVE.

Proposal 3: Follow the running CR and apply the enumerated variable to report SL relay measurement result.

Reference

- R2-2312499, Introduction of Rel-18 SL relay service continuity, MediaTek, Inc.