3GPP TSG-RAN WG2 Meeting 124			R2-2312666

Chicago, USA, Nov. 13th-17th

Agenda item:	7.14.3

Title:	Remaining issues on QMC in ND-DC

Source:	CMCC

Document for:	Discussion

1. Introduction

In RAN2#123bis [1], companies mainly discuss RVQoE and SCG issues for supporting QMC in NR-DC. Due to the upcoming QoE WI endorsement, suggest to discuss the open issues captured in the post meeting email discussion and formulate agreements.

Also, there are some technical issues captured in RAN3’s agreements and may have further impact on RAN2.

2. Discussion

  - Remaining issues

RVQoE report handling when SRB is not available:

In RAN2#123bis, RAN2 has already agreed that UE is not required to buffer RVQoE report, therefore it would be impossible for UE to do anything other than to discard the RVQoE reports.

Proposal 1: For RVQoE, when the specific SRB is not available, UE can discard RVQoE report(s).

2.2 RAN3 impact

RAN3 has made some progress on QMC in NR-DC and prepare to send an LS to RAN2 [2], the draft LS can be seen in R3-235807, with following impacts on RAN2:

In addition, RAN3 would also like to inform RAN2 about the latest agreements reached during RAN3#121bis, on NR-DC, and inter-RAT mobility, including: For NR-DC - Release all SN configured QoE measurements during SN release For Inter-RAT mobility - For HO from LTE/5GC to NR, there is no impacts to RAN3 - From NR to LTE, the source node decides which one of the QoE configurations to keep RAN3 thinks that the agreements on NR-DC and Inter-RAT mobility may require further RAN2 work, i.e., RAN2 needs to consider how to treat the unsent QoE report by UE during SN release, how UE release the other QoE measurements during the Inter-RAT handover.

RAN3 has already agree that SN configured QoE measurement configuration can be released during SN release. The handling for the corresponding unset QoE report would be an open issue. We think it’s better for NW to indicate UE whether to discard or forward the unsent QoE report and using which SRB (SRB can be unavailable). If not indicated, UE can discard the unsent QoE report.

Proposal 2: NW explicitly indicates UE whether to forward unsend QoE report and using which SRB (if necessary) when SN is released.

3. Conclusion

Proposal 1: For RVQoE, when the specific SRB is not available, UE can discard RVQoE report(s).

Proposal 2: NW explicitly indicates UE whether to forward unsend QoE report and using which SRB (if necessary) when SN is released.

4. References

[1]	R2-2311277, Report from session on MBS, QoE and LTE legacy

[2]	RAN3_121bis_agenda_20231013_EOM