3GPP TSG RAN WG2#124	 R2-2312793

Chicago, United States, 13th -17th November, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	RACH relevant SON open issues

Agenda item:		7.13.1

Document for: 	Discussion and Decision

- Introduction

This report provides an overview on the open issues of the RACH relevant MRO features based on the existing and unresolved FFS(es) in the running CR. A simple analysis on the criticality of the open issues is given and suggest to treat the open issues in the next RAN2 meeting in priority based on whether it is essential for completion of the feature.

- Discussion

SN RACH report

| FFS issues | Necessary for WI completion? | Priority |
| --- | --- | --- |
| For SN RACH report, ffs whether UE report all available NR RACH report based on request from LTE BS or whether new UE variable is needed. | Yes | High |

Above ffs is based on companies’ comments collected during CR review stages, which relevant to detailed UE behavior on how to report available NR RACH report upon request from NW. It is important to be resolved so that SN RACH report feature can be supported in R18, therefore it is suggested to be treated with high priority.

Observation 1: FFS issue on SN RACH report is important to be resolved so that SN RACH report feature can be supported in R18, which shall be treated with high priority.

NR RACH report

- Based on above table, below proposals are given:

| FFS issues |  | Necessary for WI completion? | Priority |
| --- | --- | --- | --- |
| FFS how to set the numberOfPreamblesSentOnSSB-r16 ,numberOfPreamblesSentOnCSI-RS-r16 and the perRAAttemptInfoList. |  | Yes | High |
| RAN2 discuss if raPurposes (including SchedulingRequestFailure and noPUCCHResourceAvailable) require any change when the LBT failure leads to an SR procedure failure or unavailability of the PUCCH resources for the SR in SCell. | Yes |  | High |

- Above ffs issues are on the stage 3 details of NR-U relevant information captured in RACH report, which is important for  CR implementation, which can be treated with high priority.

Observation 2: NR-U relevant ffs stage 3 issues identified in CR review is important for CR implementation , which can be treated with high priority.

| FFS issues | Necessary for WI completion? | Priority |
| --- | --- | --- |
| Proposal 6 RAN2 discuss which of the following information to the logged in the RA report when the SDT triggers an RA procedure a.UE reports the DL RSRP and pending UL data volume at the time of SDT initiation. b.The data volume buffered at UE side upon SDT initiation c.The data volume buffered at UE side when SDT fails | No | Medium |
| Proposal 7 RAN2 firstly discusses what kind of power information for an RA procedure the network actually needs for RA enhancement. Proposal 8 if power ramping information is needed at network, RAN2 considers the following options: Option 1: UE indicates whether notification of suspending power ramping counter has been received from power layer per RA attempt in RA report. Option 2: UE indicates whether power ramping is performed or not per RA attempt in RA report. | No | Medium |
| Proposal 10RAN2 discuss whether UE reports if it has used slicing specific or AI specific RACH parameters for the RA. | No | Low |

- Above issues are remaining proposals summarized in the RACH summary in [3]. Among them, P6 is relevant to ffs issue on SDT details to be stored in RA report in this release and P7-8 is on power information in RA report which is common for all RACH report, which also get some supports for discussion. For all above proposals, they are irrelevant to WI completion which can be treated only if time allows.

- Observation 3: Remaining proposals from R2-2311521[3] that are not critical for WI completion can seen as low priority, and only be treated if time allows.

- Based on above observations, below proposals are given:

- Proposal 1: For RACH relevant MRO, RAN2 prioritize discussion on below ffs issues:

- For SN RACH report, ffs whether UE report all available NR RACH report based on request from LTE BS or whether new UE variable is needed.

- FFS how to set the numberOfPreamblesSentOnSSB-r16 ,numberOfPreamblesSentOnCSI-RS-r16 and the perRAAttemptInfoList.

- RAN2 discuss if raPurposes (including SchedulingRequestFailure and noPUCCHResourceAvailable) require any change when the LBT failure leads to an SR procedure failure or unavailability of the PUCCH resources for the SR in SCell.

- Proposal 2: Below proposals are treated only if time allows:

Proposal 6 RAN2 discuss which of the following information to the logged in the RA report when the SDT triggers an RA procedure

a.UE reports the DL RSRP and pending UL data volume at the time of SDT initiation.

b.The data volume buffered at UE side upon SDT initiation

c.The data volume buffered at UE side when SDT fails

Proposal 7 RAN2 firstly discusses what kind of power information for an RA procedure the network actually needs for RA enhancement.

Proposal 8 if power ramping information is needed at network, RAN2 considers the following options:

Option 1: UE indicates whether notification of suspending power ramping counter has been received from power layer per RA attempt in RA report.

Option 2: UE indicates whether power ramping is performed or not per RA attempt in RA report.

Proposal 10RAN2 discuss whether UE reports if it has used slicing specific or AI specific RACH parameters for the RA.

- Conclusion and proposals

Based on above analysis, we have the following observations and proposals:

Observation 1: FFS issue on SN RACH report is important to be resolved so that SN RACH report feature can be supported in R18, which shall be treated with high priority.

Observation 2: NR-U relevant ffs stage 3 issues identified in CR review is important for CR implementation , which can be treated with high priority.

- Observation 3: Remaining proposals from R2-2311521 that are not critical for WI completion can seen as low priority, and only be treated if time allows.

- Proposal 1: For RACH relevant MRO, RAN2 prioritize discussion on below ffs issues:

- For SN RACH report, ffs whether UE report all available NR RACH report based on request from LTE BS or whether new UE variable is needed.

- FFS how to set the numberOfPreamblesSentOnSSB-r16 ,numberOfPreamblesSentOnCSI-RS-r16 and the perRAAttemptInfoList.

- RAN2 discuss if raPurposes (including SchedulingRequestFailure and noPUCCHResourceAvailable) require any change when the LBT failure leads to an SR procedure failure or unavailability of the PUCCH resources for the SR in SCell.

- Proposal 2: Below proposals are treated only if time allows:

Proposal 6 RAN2 discuss which of the following information to the logged in the RA report when the SDT triggers an RA procedure

a.UE reports the DL RSRP and pending UL data volume at the time of SDT initiation.

b.The data volume buffered at UE side upon SDT initiation

c.The data volume buffered at UE side when SDT fails

Proposal 7 RAN2 firstly discusses what kind of power information for an RA procedure the network actually needs for RA enhancement.

Proposal 8 if power ramping information is needed at network, RAN2 considers the following options:

Option 1: UE indicates whether notification of suspending power ramping counter has been received from power layer per RA attempt in RA report.

Option 2: UE indicates whether power ramping is performed or not per RA attempt in RA report.

Proposal 10RAN2 discuss whether UE reports if it has used slicing specific or AI specific RACH parameters for the RA.

- 4. References

- RR2-2312791	Running 36.331 CR for SN RACH report	ZTE Corporation, Sanechips

- R2-2312792	Running 38.331 CR for RACH report	ZTE Corporation, Sanechips

- R2-2311521	Summary of the AI 7.13.6 RACH optimization (Ericsson)