3GPP TSG RAN WG2 #124	                            				     R2-2312878

Chicago, USA, 13th – 17th November 2023

Agenda Item:	7.6.3.2

Source:	InterDigital, Inc.

Title:	CHO enhancement for earth-moving cells.

Document for:	Discussion

# Introduction

For NR, the following was agreed in RAN2#123bis:

- For location-based CHO for earth-moving cells, re-use the procedure from cell reselection as baseline to derive the candidate cell’s reference location as the cell moves (FFS on how to signal the needed parameters, e.g. ephemeris and Epoch time)

In IoT-NTN, the following has been agreed so far.

RAN2#119-e CHO enhancements for eMTC NTN (i.e. time/timer based solution) are introduced based on the R17 NR NTN solution. FFS on location-based solution RAN2#119bis-e CHO enhancements for eMTC NTN (i.e. time/timer based solution) are introduced based on the R17 NR NTN solution. FFS on location-based solution RAN2#120 CHO enhancements for eMTC NTN (i.e. time/timer based solution) are introduced based on the R17 NR NTN solution. FFS on location-based solution RAN2#121 Location-based connected mode measurement initiation is supported in quasi-Earth-fixed cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for quasi-Earth-fixed cell. FFS on whether the R17 IEs are reused or not. FFS if the same mechanism can also be used in idle (like in NR-NTN) Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris). FFS if the same mechanism can also be used in idle (like in NR-NTN) RAN2#121bis For moving cell, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (like in NR-NTN) For eMTC NTN, for moving cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN). FFS whether to consider solution that does not require UE to update the GNSS for this same as in connected mode RAN2#123 For CHO in NTN (both NR NTN and eMTC NTN, time and location-based trigger conditions may be configured independently (i.e., without a jointly configured measurement condition). We add a description/note saying in which scenarios this is reasonable, e.g. at least hard-switch case where gap is assumed to be zero/negligible RAN2#123bis Separate reference locations are introduced for earth-quasi fixed cells and earth-moving cells.

In this paper we discuss how to enable location based CHO for earth-moving cells, considering the R18 enhancements in NR.

# Discussion

As per the agreements in the previous meetings, location based CHO will be introduced for IoT-NTN based on the solution in NR. Additionally, it was agreed to have separate reference location IEs for the serving cell for fixed and moving cell.

In R18, it has been agreed for NR to enhance location based CHO by deriving the candidate cell’s reference location based on ephemeris and epoch time as can be done for RRC_IDLE.

We think that the same enhancement can be introduced for IoT-NTN, and is particularly useful to have.

Proposal 1: For location-based CHO for earth-moving cells, support deriving the candidate cell’s reference location as the cell moves (as in R18 NR)

In the case of NR, since location based CHO was initially introduced in R18, then an extension is needed to support signaling of whether the candidate cell is moving, and the relevant parameters. A non-critical extension is likely needed, at least for the indication of whether the candidate cell reference location is fixed or moving. The epoch time and ephemeris may be obtained from the candidate cell ntn-Config.

However, for IoT-NTN, since we are introducing support for CHO in R18, there is no need for extension, but rather the support for fixed/moving location in the CHO configuration can be introduced from the beginning.

Proposal 2: Support configuration of fixed or moving reference referenceLocation1 and referenceLocation2 in condEventD1.

In IoT-NTN, the epochTime and ephemerisInfo are not provided as part of the ntn-Config, they are signalled in SIB31 (ServingSatelliteInfo) for the serving cell, and SIBXX (NeighSatelliteInfoList) for neighbour cells. It cannot be assumed that the UE always has the information from SIBXX for a particular candidate cell to use for CHO, therefore epochTime and ephemerisInfo need to be included in the dedicated signalling. This can either be introduced in the event configuration or the ntn-ConfigDedicated for the candidate cell. To align with NR, and allow for potential future use of epochTime and ephemerisInfo for purposes other than event D1, we propose to include in RadioResourceConfigDedicated.

Proposal 3: Introduce epochTime and ephemerisInfo in RadioResourceConfigDedicated so that these can be provided for the CHO candidate cell.

In order to support updating moving cell, the following TP (based on the latest running CR) is provided:

CondReconfigurationTriggerEUTRA-r16 ::= SEQUENCE { condEventId-r16 CHOICE { condEventA3-r16 SEQUENCE { a3-Offset-r16 INTEGER (-30..30), hysteresis-r16 Hysteresis, timeToTrigger-r16 TimeToTrigger }, condEventA5-r16 SEQUENCE { a5-Threshold1-r16 ThresholdEUTRA, a5-Threshold2-r16 ThresholdEUTRA, hysteresis-r16 Hysteresis, timeToTrigger-r16 TimeToTrigger }, ..., [[ condEventA4-r18 SEQUENCE { a4-Threshold-r18 ThresholdEUTRA, hysteresis-r18 Hysteresis, timeToTrigger-r18 TimeToTrigger }, condEventD1-r18 SEQUENCE { distanceThreshFromReference1-r18 INTEGER(0.. 65535), distanceThreshFromReference2-r18 INTEGER(0.. 65535), referenceLocation1-r18 CHOICE { fixedCell-r18 ReferenceLocation-r18, movingCell-r18 ReferenceLocation-r18 }, referenceLocation2-r18 CHOICE { fixedCell-r18 ReferenceLocation-r18, movingCell-r18 ReferenceLocation-r18 }, hysteresisLocation-r18 HysteresisLocation-r18, timeToTrigger-r18 TimeToTrigger }, condEventT1-r18 SEQUENCE { t1-Threshold-r18 INTEGER (0..549755813887), duration-r18 INTEGER (1..6000) } ]] } }

RadioResourceConfigDedicated ::= SEQUENCE { srb-ToAddModList SRB-ToAddModList OPTIONAL, -- Cond HO-Conn drb-ToAddModList DRB-ToAddModList OPTIONAL, -- Cond HO-toEUTRA drb-ToReleaseList DRB-ToReleaseList OPTIONAL, -- Need ON mac-MainConfig CHOICE { explicitValue MAC-MainConfig, defaultValue NULL } OPTIONAL, -- Cond HO-toEUTRA2 sps-Config SPS-Config OPTIONAL, -- Need ON physicalConfigDedicated PhysicalConfigDedicated OPTIONAL, -- Need ON ..., [[ rlf-TimersAndConstants-r9 RLF-TimersAndConstants-r9 OPTIONAL -- Need ON ]], [[ measSubframePatternPCell-r10 MeasSubframePatternPCell-r10 OPTIONAL -- Need ON ]], [[ neighCellsCRS-Info-r11 NeighCellsCRS-Info-r11 OPTIONAL -- Need ON ]], [[ naics-Info-r12 NAICS-AssistanceInfo-r12 OPTIONAL -- Need ON ]], [[ neighCellsCRS-Info-r13 NeighCellsCRS-Info-r13 OPTIONAL, -- Cond CRSIM rlf-TimersAndConstants-r13 RLF-TimersAndConstants-r13 OPTIONAL -- Need ON ]], [[ sps-Config-v1430 SPS-Config-v1430 OPTIONAL -- Cond SPS ]], [[ srb-ToAddModListExt-r15 SRB-ToAddModListExt-r15 OPTIONAL, -- Need ON srb-ToReleaseListExt-r15 INTEGER (4) OPTIONAL, -- Need ON sps-Config-v1530 SPS-Config-v1530 OPTIONAL, -- Need ON crs-IntfMitigConfig-r15 CHOICE { release NULL, setup CHOICE { crs-IntfMitigEnabled NULL, crs-IntfMitigNumPRBs ENUMERATED {n6, n24} } } OPTIONAL, -- Need ON neighCellsCRS-Info-r15 NeighCellsCRS-Info-r15 OPTIONAL, -- Need ON drb-ToAddModList-r15 DRB-ToAddModList-r15 OPTIONAL, -- Need ON drb-ToReleaseList-r15 DRB-ToReleaseList-r15 OPTIONAL, -- Need ON dummy SEQUENCE (SIZE (1..2)) OF INTEGER (1..2) OPTIONAL -- Need ON ]], [[ sps-Config-v1540 SPS-Config-v1540 OPTIONAL -- Need ON ]], [[ rlf-TimersAndConstantsMCG-Failure-r16 RLF-TimersAndConstantsMCG-Failure-r16 OPTIONAL, -- Cond Split-SRB1-SRB3 crs-ChEstMPDCCH-ConfigDedicated-r16 SetupRelease{CRS-ChEstMPDCCH-ConfigDedicated-r16} OPTIONAL, -- Need ON newUE-Identity-r16 C-RNTI OPTIONAL -- Need OP ]], [[ harq-FeedbackEnablingforSPSactive-r18 ENUMERATED {enabled} OPTIONAL, -- Need OR ephemerisInfo-r17 CHOICE { stateVectors EphemerisStateVectors-r17, orbitalParameters EphemerisOrbitalParameters-r17 }, epochTime-r17 SEQUENCE { startSFN-r17 INTEGER (0..1023), startSubFrame-r17 INTEGER (0..9) } ]] }

# Conclusion

In this contribution we discuss the enhancement of location-based CHO for earth moving cells and have the following proposals:

Proposal 1: For location-based CHO for earth-moving cells, support deriving the candidate cell’s reference location as the cell moves (as in R18 NR)

Proposal 2: Support configuration of fixed or moving reference referenceLocation1 and referenceLocation2 in condEventD1.

Proposal 3: Introduce epochTime and ephemerisInfo in RadioResourceConfigDedicated so that these can be provided for the CHO candidate cell.

# References

- RP-223519,  “Revised WID on IoT NTN enhancements