3GPP TSG-RAN WG2 Meeting #124	R2-2313021

# Chicago, U.S.A, 13th-17th November, 2023

Agenda Item:	7.3.6

Source:	InterDigital

Title:	SP CSI reporting on PUCCH Activation MAC CE

Document for:	Discussion, Decision

# Introduction

RAN1 has been working on spatial and power domain adaption for NES, per the WI objective in [1]. For semi-persistent CSI reporting on PUCCH, the MAC CE design agreed by R1 needs to indicate sub-configuration selection, corresponding to the following RAN1 agreements in RAN1 #113:

Agreement

For N>=1 CSI reporting corresponding to N out of L sub-configurations in one reportConfig where each sub-configuration corresponding to an SD adaptation pattern or/[and] a powerControlOffset value,

- For SP-CSI on PUCCH report, support MAC-CE-based triggering

- For MAC-CE based triggering

    - Opt 2: An indication to select to N sub-configurations in a MAC-CE is supported

    - It is up to RAN2 to decide the signaling designs of the MAC-CE (including whether it is a new MAC CE or an existing MAC CE)

    - Only one MAC CE is used for this triggering

Subsequently, an LS [3] was received towards the end of RAN2#123bis:

1. Overall Description: RAN1 has discussed the SP-CSI reporting related issues and made the following agreements • From RAN1 perspective, up to 4 CSI report configurations can be configured in a BWP for SP CSI reporting on PUCCH where one or more report configurations can contain a list of sub-configuration(s). Furthermore, it is agreed that • For the max number of sub-configurations Lmax in one CSI report configuration, the maximum value of Lmax is no larger than 8 for semi-persistent CSI reporting on PUCCH • For report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration, the maximum value of N is no larger than 4 for semi-persistent CSI reporting on PUCCH. 2. Actions: To RAN2: ACTION: RAN1 kindly ask RAN2 to take the above into account in their future work related to the MAC-CE design for SP-CSI reporting for Rel-18 NES.

The following was agreed in RAN2 123bis for the MAC CE design:

Agreements:

- design a new MAC CE for activating/deactivating SP CSI report configurations and selecting N out of L subconfigurations for each CSI reportconfiguration.

- The new MAC CE can be used to activate/deactivate configuration and sub-configuration. One new bit per sub-configuration will be added to activate/deactivate.

During the online discussion, it was suggested to provide a sample for the MAC CE format to avoid having many formats submitted to the next meeting. This tdoc contains the MAC CE implementation that is proposed as a common view as the outcome of the email discussion [Post123bis][022][NES] 38.321 Running CR (Interdigital) [4].

# SP CSI reporting on PUCCH Activation MAC CE

#### 6.1.3.y	Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE

The enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1. It has a variable size and consists of the following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated;

-	Ni,x: this field indicates the activation/deactivation status of the Semi-Persistent CSI report SubConfiguration x within csi-ReportSubConfigList of CSI-ReportConfigId i, as specified in TS 38.331 [5]. If Si set to 1, the octet corresponding to Ni,0 to Ni,7 is present. If Si set to 0, the octet corresponding to Ni,0 to Ni,7 is not present. N0,0 refers to the report SubConfiguration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest csi-ReportSubConfigID within the list, N0,1 to the report SubConfiguration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest csi-ReportSubConfigID and so on. If the number of report SubConfigurations within the list with type set to csi-ReportSubConfigList in the indicated BWP is less than x + 1, MAC entity shall ignore the Ni,x field. The Ni,x field is set to 1 to indicate that the corresponding Semi-Persistent CSI report SubConfiguration x shall be activated. The Ni,x field is set to 0 to indicate that the corresponding Semi-Persistent CSI report SubConfiguration x shall be deactivated;

-	R: Reserved bit, set to 0.

Figure x: Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE

# Conclusion

This contribution contains the MAC CE implementation that is proposed as a common view as the outcome of the email discussion [Post123bis][022][NES] 38.321 Running CR (Interdigital) [4].

# References

RP-223540, “New WID: Network energy savings for NR”, Huawei

TS 38.321, “Medium Access Control (MAC) protocol specification”, v 17.5.0

R1-2310578, “SP-CSI reporting for network energy savings”, RAN1, Huawei

R2-2313020, “Report of [Post123bis][022][NES] 38.321 Running CR (Interdigital)”, InterDigital (Rapporteur)