3GPP TSG-RAN WG2 Meeting #124	R2-2313273

# Chicago, USA, 13 – 17 November, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4327 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on extension of AreaConfiguration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In last meeting we provide the discussion paper and a CR for the extension of R17 AreaConfiguration and reach the agreement as below: R2-2310363 Corrections on extension of R17 AreaConfiguration CATT discussion Rel-17 NR_ENDC_SON_MDT_enh-Core => Noted R2-2310364 Corrections on extension of AreaConfiguration (Option1) CATT CR Rel-17 38.331 17.6.0 4327 - F NR_ENDC_SON_MDT_enh-Core => Agree with the intention and will be further discussed.. So in this CR we use Option 1 in the above discussion paper to correct the extension type of R17 AreaConfiguration to align with the original agreement made in R17: 5: Introduce AreaConfiguration-r17 (including areaConfig-r16 and interFreqTargetList-r16 inside it with both fields being optional) in Rel-17. |
| Summary of change: |  | Revert back to the original agreement “Introduce AreaConfiguration-r17 (including areaConfig-r16 and interFreqTargetList-r16 inside it with both fields being optional) in Rel-17”: - Change the suffix of “areaConfiguration-v1700/AreaConfiguration-v1700” to “-r17”; - Change the suffix of “areaConfig-r17” and “interFreqTargetList-r17” in “AreaConfiguration-r17” to “-r16”; - Add a Cond to the areaConfiguration-v1700 field. Impact analysis Architecture options NR SA, NR-DC, NE-DC Impacted functionality: Logged MDT configuration Inter-operability: - If the UE implements this change but the network does not, the network cannot configure the UE to apply the single interFreqTargetList without areaConfig, which is not aligned with the RAN3 spec; - If the network implements this change but the UE does not, the UE will consider the area configuration as NCE, and the original intention of only configuring interFreqTargetList without areaConfig can not be achieved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is ambiguous whether a CE or a NCE is applied for the areaConfig and interFreqTargetList in areaConfiguration field, the network configuration in TS38.331 can not be alignd with TS38.413, and the UE comprehension/behaviour is indeterminate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

6.2.2	Message definitions

#### –	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::=  SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration-r16      LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16                    OPTIONAL,  --Need R

plmn-IdentityList-r16                       PLMN-IdentityList2-r16                   OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need M

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need M

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need M

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16,

...

},

lateNonCriticalExtension                    OCTET STRING                             OPTIONAL,

nonCriticalExtension                        LoggedMeasurementConfiguration-v1700-IEs OPTIONAL

}

LoggedMeasurementConfiguration-v1700-IEs ::= SEQUENCE {

sigLoggedMeasType-r17                       ENUMERATED {true}                        OPTIONAL, -- Need R

earlyMeasIndication-r17                     ENUMERATED {true}                        OPTIONAL, -- Need R

areaConfiguration-r17                     	AreaConfiguration-r17                  	OPTIONAL,  -- Cond NoAreaConfig

nonCriticalExtension                        SEQUENCE {}                              OPTIONAL

}

LoggedPeriodicalReportConfig-r16 ::=            SEQUENCE {

loggingInterval-r16                             LoggingInterval-r16,

...

}

LoggedEventTriggerConfig-r16 ::=                SEQUENCE {

eventType-r16                                   EventType-r16,

loggingInterval-r16                             LoggingInterval-r16,

...

}

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting either one of the included cell identities or one of the included tracking area codes/ frequencies.

earlyMeasIndication If included, the field indicates the UE is allowed to log measurements on early measurement related frequencies in logged measurements.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs.

sigLoggedMeasType If included, the field indicates a signalling based logged measurement configuration (See TS 37.320 [61]).

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT configuration or Event Triggerd MDT configuration.

| Conditional Presence | Explanation |
| --- | --- |
| NoAreaConfig | The field is optional present, need R, if areaConfiguration-r16 is absent; otherwise this field is absent. |

NEXT CHANGE

6.3.4	Other information elements

#### –	AreaConfiguration

The AreaConfiguration indicates area for which UE is requested to perform measurement logging. If not configured, measurement logging is not restricted to specific cells or tracking areas but applies as long as the RPLMN is contained in plmn-IdentityList stored in VarLogMeasReport.

AreaConfiguration information element

-- ASN1START

-- TAG-AREACONFIGURATION-START

AreaConfiguration-r16 ::=        SEQUENCE {

areaConfig-r16                   AreaConfig-r16,

interFreqTargetList-r16          SEQUENCE(SIZE (1..maxFreq)) OF InterFreqTargetInfo-r16              OPTIONAL  -- Need R

}

AreaConfiguration-r17 ::=      SEQUENCE {

areaConfig-r16                   AreaConfig-r16                                                      OPTIONAL, -- Need R

interFreqTargetList-r16          SEQUENCE(SIZE (1..maxFreq)) OF InterFreqTargetInfo-r16              OPTIONAL  -- Need R

}

AreaConfig-r16 ::=     CHOICE {

cellGlobalIdList-r16             CellGlobalIdList-r16,

trackingAreaCodeList-r16         TrackingAreaCodeList-r16,

trackingAreaIdentityList-r16     TrackingAreaIdentityList-r16

}

InterFreqTargetInfo-r16    ::=   SEQUENCE {

dl-CarrierFreq-r16               ARFCN-ValueNR,

cellList-r16                     SEQUENCE (SIZE (1..32)) OF  PhysCellId  OPTIONAL      -- Need R

}

CellGlobalIdList-r16 ::=         SEQUENCE (SIZE (1..32)) OF CGI-Info-Logging-r16

TrackingAreaCodeList-r16 ::=     SEQUENCE (SIZE (1..8)) OF TrackingAreaCode

TrackingAreaIdentityList-r16 ::= SEQUENCE (SIZE (1..8)) OF TrackingAreaIdentity-r16

TrackingAreaIdentity-r16 ::=     SEQUENCE {

plmn-Identity-r16                PLMN-Identity,

trackingAreaCode-r16             TrackingAreaCode

}

-- TAG-AREACONFIGURATION-STOP

-- ASN1STOP

AreaConfiguration field descriptions

InterFreqTargetInfo If configured, it indicates the neighbouring frequency and cells for which UE is requested to perform measurement logging. It can include sync raster or non-sync raster frequencies.

END OF CHANGE