3GPP TSG-RAN WG2 #124	R2-2313574

Chicago, USA, November 2023

Agenda Item:	5.1.3.2

Source:	OPPO

Title:	Left issues on asyncIntraBandENDC

Document for:	Discussion, Decision

Introduction

This is to discuss the left issue from R4-2317402.

Discussion

Given the RP request to introduce a BS signaling, for TDD-TDD case of inter-band ENDC scenario, to differentiate type-1/2 UE behavior, and now the extension of intra-band ENDC async capability flag to inter-band case and NE-DC case, it seems helpful to summarize all related cases together to have an overview.

The information comes from TS 38.133, and latest CR of R4-2317400 (together with the incoming LSs).

Table 1 UE capability and performance requirement for intra-/inter-band EN/NE-DC and for different duplex combinations

|  |  | EN-DC |  |  |  |  | NE-DC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Question | How to diff 1/3 |  | How to diff 1/2/3 | How to diff 1/2 |  | How to diff 1/2/3 | How to diff 1/2 | How to diff 2/3 |
|  |  | Intra-band, FDD-FDD | Intra-band TDD-TDD | Inter-band FDD-FDD with overlapping frequency | Inter-band FDD-TDD, TDD-FDD, FDD-FDD, TDD-TDD, without overlapping frequency | Inter-band TDD-TDD with overlapping frequency | Inter-band FDD-FDD, with overlapping frequency | FDD-TDD, TDD-FDD, FDD-FDD, TDD-TDD, without overlapping frequency | Inter-band TDD-TDD, with overlapping frequency |
| 1 | Async MTTD: Table 7.5.2-1 Table 7.5.5-1 MRTD Table 7.6.2-1 Table 7.6.5-1 | Capability-based Support | Not support | Capability-based Support | Mandatory support | Not support | Capability-based Support | Mandatory support | Not support |
| 2 | Sync, Type-2 MTTD: Table 7.5.2.1-1 Table 7.5.5.1-1 MRTD: Table 7.6.2.1-1 Table 7.6.5.1-1 | Not Support | Not Support | Capability-based Support | Mandatory support | Capability-based Support | Capability-based Support | Mandatory support | Capability-based Support |
| 3 | Sync, Type-1 MTTD: Table 7.5.3-1 MRTD: Table 7.6.3-1 | Mandatory Support | Mandatory Support |  | Not support |  |  | Not Support |  |

Issue-1: How to extend asyncIntraBandENDC

R4 requested that

The signaling “asyncIntraBandENDC” was introduced for intra-band EN-DC, while it is further extended to cover inter-band EN-DC with overlapping frequency.

[…]

Further, the signaling “asyncIntraBandENDC” now only cover EN-DC UE reporting. RAN4 would like to ask RAN2 to update this signaling description to cover NE-DC reporting.

Currently, asyncIntraBandENDC is for FDD-FDD only, while

asyncIntraBandENDC

Indicates whether the UE supports asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD as specified in clause 7.5 and 7.6 of TS 38.133 [5]. If asynchronous FDD-FDD intra-band (NG)EN-DC is not supported, the UE supports only synchronous FDD-FDD intra-band (NG)EN-DC.

This capability applies to:

[…]

-	Inter-band (NG)EN-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]).

While the interBandMRDC-WithOverlapDL-Bands-r16 covers both FDD-FDD and TDD-TDD

interBandMRDC-WithOverlapDL-Bands-r16

Indicates the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE).

So the firstly since inter-band ENDC, FDD-FDD with overlapping has been somehowcovered for the case that

“where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band”,

but did not mention the case of partial overlapping, e.g., for DC_20A_n28A (included in Table 5.5B.4.1-1 of TS 38.101-3, where the DL of band 20 and n28 is partially overlapping during 791MHz – 803MHz range), yet considering the NOTE that

NOTE 8:	The frequency range in band n28 / 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. This restriction also apply for any band combinations when DC_20_n28/ DC_28_n20/ CA_20-28/ CA_n20-n28 is a subset of a higher order band combination.

It seems not clear whether there is a need to consider this case as “FDD-FDD with overlapping range” (i.e., whether the overlapping is defined in frequency range of band-level or CC-level), and even it is not clear whether the legacy capability bit has already covered that case (if valid).

- For R4-2317402, before capturing in R2 CR, R2 discuss to cover the case of partial overlapping, e.g., DC_20A_n28A for ENDC.

For R4-2317402, before capturing in R2 CR, R2 confirm what R4 requested for EN-DCis to additionally cover inter-band ENDC, FDD-FDD with “partial” overlapping frequency.

Secondly, for NE-DC, the left issue is

- whether the intention is to extend the capability to both TDD-TDD and FDD-FDD, i.e., the cases where overlapping may happen.

- whether the intention is to extend the capability to NEDC for inter-band case only, or intra-band case as well.

- For R4-2317402, before capturing in R2 CR, R2 confirm what R4 requested for NE-DC is to additionally cover  inter-band NEDC, FDD-FDD with overlapping frequency.

Another issue is whether to use separate IE to do the extension, for NE-DC-only BC list, there is no concern, since this IE is per-BC IE, but then for the BC which is capable of both EN-DC and NE-DC, separate IE may be motivated if the UE can support one but not the other. However, it seems not very possible given it is a R4 capability.

- For R4-2317402, before capturing in R2 CR, R2 discuss whether to use a separate capability bit to do the extension for NEDC.

Conclusion

We have the following proposals:

Proposal 1	For R4-2317402, before capturing in R2 CR, R2 discuss to cover the case of partial overlapping, e.g., DC_20A_n28A for ENDC.

Proposal 2	For R4-2317402, before capturing in R2 CR, R2 confirm what R4 requested for NE-DC is to additionally cover  inter-band NEDC, FDD-FDD with overlapping frequency.

Proposal 3	For R4-2317402, before capturing in R2 CR, R2 discuss whether to use a separate capability bit to do the extension for NEDC.