# 3GPP TSG-RAN WG2 Meeting #124	R2-2313981

# Chicago, US, 13 - 17 November, 2023	was R3-237884

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0752 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Network controlled repeater |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (ZTE, China Telecom, Samsung,CATT, Ericsson, Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2023-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add the support for Network-Controlled Repeater. Capture RAN1 agreement on beam info configuration from OAM to gNB and NCR into draft CR. Add step 1 description in section X.Y1, because this step is not covered in section 9.2.1.3, TS 38.300. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following functionalities are introduced for Network Controlled Repeater: - Network Controlled Repeater OAM connectivity - Network Controlled Repeater management Capture RAN1 agreement on beam info configuration from OAM to gNB and NCR into draft CR. Add step 1 description in section X.Y1, because this step is not covered in section 9.2.1.3, TS 38.300. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network Controlled Repeater can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X (new), X.Y (new), X.Y1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 0890 TS 38.401 CR 0274 TS 38.473 CR 1109 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was endorsed by RAN3 in R3-237884. |  |  |  |  |  |  |  |  |

<<<<<< START OF CHANGE >>>>>>

# X	Network controlled repeater

## X.Y	OAM aspects

The transport connection between the NCR-node and its OAM may be provided by the NCR-MT’s PDU session. A NCR may be configured with a list of allowed gNB cell(s) that the NCR-MT is allowed to connect with, and/or a list of forbidden gNB cell(s) that the NCR-MT is not allowed to connect with.

The information on the physical beam(s) used by NCR-Fwd for access link may be provided by OAM to the gNB and the NCR for operation. How to characterize and provide the physical beams of NCR-Fwd is up to implementation.

## X.Y1	Network controlled repeater management

NCR identification is performed in RAN, and NCR authorization is performed in 5GC. The general procedure of the NCR management is illustrated in Figure X.Y-1:

Figure X.Y-1. NCR management

Step 1: The gNB broadcasts the NCR supported information via system information.

Step 6: When a NCR is trying to access the network as a NCR, the NCR indication is sent to the serving gNB.

Step 7: The serving gNB selects an appropriate AMF for the NCR.

Step 9: AMF provides NCR authorization information to the gNB.

Other steps refer to the signalling flow as defined in 9.2.1.3.

<<<<<< END OF CHANGE >>>>>>