3GPP TSG-RAN WG2 Meeting #125	R2-2400132

Athens, Greece, 26th February – 1st March, 2024

Agenda Item:	7.21.3

Source:	Huawei, HiSilicon

Title:	Discussion on open issues on user plane for CE

Document for: Discussion

1	Introduction

During on post RAN2#124 email discussion, the following open issues are summarized for user plane on CE:

| Open issue | Rapporteur's view |
| --- | --- |
| Whether Msg1 repetition is applicable in NTN network? If applicable, when to start RAR window and how to capture it in MAC? | This issue was identified by company during [POST124][852][CE_enh] 38.321 CR, companies are invited to provide tdoc(with TP) to express your view and preferred spec change. |
| For RACH resource selection, current MAC spec is incomplete when both Msg1 repetition and eRedCap are applicable for the RACH procedure. (Note: assume Msg1 repetition is also applicable to eRedCap UEs) | The issue appears when merging the R18 CE and eRedCap CRs, rapporteur will provide updated MAC CR to address it. Companies can also bring tdoc(with TP) if you want. |
| Whether DWS and mTRP can be configured together? Even if the answer is "Yes", RAN2 assumes there is no RAN1 or RAN4 impact. | This was discussed briefly in RAN1#110bis-e and the RAN1 FL suggested to depriotize the discussion. Considering no clear agreement has been made in RAN1/2, companies in RAN2 are invited to show your views in your contribution. |

In this contribution, we further discuss the open issue for CE taking into account the above.

2	Discussion

2.1	Msg1 repetition in NTN network

In NTN, due to long UE-gNB RTT between UE and gNB, normally starting of MAC timer (e.g. ra-ContentionResolutionTimer) takes the long RTT into account to avoid early starting as:

5.1.5 Contention Resolution Once Msg3 is transmitted the MAC entity shall: 1> if the Msg3 transmission (i.e. initial transmission or HARQ retransmission) is scheduled with PUSCH repetition Type A: 2> if Msg3 is transmitted on a non-terrestrial network: 3> start or restart the ra-ContentionResolutionTimer in the first symbol after the end of all repetitions of the Msg3 transmission plus the UE-gNB RTT. 2> else: 3> start or restart the ra-ContentionResolutionTimer in the first symbol after the end of all repetitions of the Msg3 transmission. 1> else if Msg3 transmission (i.e. initial transmission or HARQ retransmission) is transmitted on a non-terrestrial network: 2> start or restart the ra-ContentionResolutionTimer in the first symbol after the end of the Msg3 transmission plus the UE-gNB RTT. 1> else: 2> start or restart the ra-ContentionResolutionTimer in the first symbol after the end of the Msg3 transmission.

The NTN UE is also probably to face a bad coverage issue and to achieve the coverage gains with MSG1 repetition as in TN network, MSG1 repetition should be supported in NTN network. As usual, the starting of RAR window should also take the UE-gNB RTT into account similar as ra-ContentionResolutionTimer, i.e. UE starts the ra-ResponseWindow at the first PDCCH occasion as specified in TS 38.213 [6] from the end of all repetitions of the Random Access Preamble transmission plus the UE-gNB RTT.

- In NTN network, UE starts the ra-ResponseWindow at the first PDCCH occasion as specified in TS 38.213 [6] from the end of all repetitions of the Random Access Preamble transmission plus the UE-gNB RTT.

2.2 eRedcap

On this issue, R18 eRedcap is considered as a separate feature from R17 Redcap based on the TS 38.331.

On the other hand, RAN2 agreed that feature combination between R17 Redcap and MSG1 repetition are supported and in particular for CFRA with MSG1 repetition for Redcap UE, the following text are added in the MAC spec, e.g. Redcap UE selects the set of RA resource that is only configured with Redcap indication and MSG1 repetition indication and associated with the indicated MSG1 repetition number.

1> else if contention-free Random Access Resources with Msg1 repetition have been provided for this Random Access procedure and Msg1 repetition number is indicated in rach-ConfigDedicated, and RedCap is applicable for the current Random Access procedure: 2> select the set of Random Access resources that is only configured with RedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure. 1> else if contention-free Random Access Resources have been provided for this Random Access procedure and RedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with RedCap indication; or 1> else if contention-free Random Access Resources have been provided for this Random Access procedure and eRedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with eRedCap indication; or 1> else if contention-free Random Access Resources have been provided for this Random Access procedure and eRedCap is applicable for the current Random Access procedure and there is no set of Random Access resources available that is only configured with eRedCap indication and there is one set of Random Access resources available that is only configured with RedCap indication: 2> select this set of Random Access resources for this Random Access procedure.

In R18, eRedcap UEs should be also able to get the benefit on MSG1 repetition as Redcap UEs and similar CFRA requirement with MSG1 repetition as Redcap should be added to eRedcap in MAC spec, i.e. UE selects the set of RA resource that is only configured with eRedcap indication and MSG repetition indication and associated with the indicated MSG1 repetition number.

- For eRedcap, UE selects the set of RA resource that is only configured with eRedcap indication and MSG1 repetition indication and associated with the indicated MSG1 repetition number.

2.3 DWS with MTRP

On this issue for DWS with MTRP which raised in email discussion, generally it is a common understand that RAN1 impact is not desirable for mTRP PHR at this stage, however, RAN1 does not have an agreement to support it. In our view, RAN2 should not focus on a scenario which were not concluded by RAN1.

One proposal is to update the two PHR MAC CE to contain an additional assumed PH values for each TRP. But we would like to note that two PHR mode was introduced by RAN1 with the applicable scenario where two TRP are associated with separate UL beam resources, i.e. SRS resource set (see highlight in MAC and RRC spec). Thus, we think any mTRP PHR MAC CE enhancement with assumed PUSCH should be decided by RAN1 and possible RAN4 who are the expertise.

//38.321 The two PHs together with one PCMAX,f,c for the Serving Cell are reported if UE is configured with twoPHRMode with the multiple TRP PUSCH repetition feature is configured. //38.331 twoPHRMode Indicates if the power headroom shall be reported as two PHRs (each PHR associated with a SRS resource set) is enabled or not.

- mTRP PHR MAC CE with assumed PUSCH is not supported.

2.4 DPC

Based on the field description on DPC field in multiple entry PHR MAC CE as below, the network can know whether the criteria to report PDC is met or not from UE side based on the value in DPC field. When PHR is triggered but it is not intended for DPC, the UE set the DPC field to value 0 and hence network understands this PHR MAC CE without measured DPC value info.

- DPCBC: If dpc-Reporting-FR1 is configured, this field indicates the ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC, as specified in TS 38.101-1[14] and TS 38.101-3 [16], this field set to 0 indicates the delta power class for band combination is 0dB, this field set to 1 indicates the delta power class for band combination is same or larger than 3 dB. Otherwise, R bit is present, set to 0; - DPC: If dpc-Reporting-FR1 is configured, and the Serving Cell operates on FR1, this field indicates the ΔPPowerClass, as specified in TS 38.101-1[14] and TS 38.101-3 [16]. This field indicates an index to Table 6.1.3.8-4 and the corresponding measured values of DPC levels in dB are specified in TS 38.133 [11], the DPC field is set to 0 if the criteria to report DPC is not met. The length of the field is 2 bits. If the Serving Cell operates on FR2, R bits or MPE is present instead. Table 6.1.3.8-4: Delta power class for DPC DPC; Measured DPC value. 0; reserved. 1; DPC_00. 2; DPC_03. 3; DPC_06.

On DPCBC, the following is captured in TS 38.101-1, i.e. the UE reports DPCBC only when the criteria to report DPC is met:

NOTE: ∆PPowerClass,CA reporting capability XXX-r18, as defined in TS 38.306, is used to report ∆PPowerClass,CA when the network configures the UE with the reporting and the reporting is triggered only by uplink duty cycle exceedance or by return to the powerClass after the duty cycle exceedance.

However, for DPCBC with 1 bit length, we think it is incomplete to convey all the cases:

- Case 1: PHR is triggered for DPC, and DPCBC field is set to value 0, network may understand the delta power class is 0dB;

- Case 2: PHR is triggered for DPC, and DPCBC field is set to value 1, network may understand the delta power class is same or larger than 3dB.

- Case 3: PHR is NOT triggered for DPC, given that the DPCBC field bit is set to either value 0 and 1, how does the network understand the DPCBC field? In this case, it is not intended for network to interpret DPCBC based on the value.

Thus there is no option to convey the case that this PHR is not triggered for DPC, i.e. DPCBC is not available for a band combination. On this issue, there are some initial discussion over email discussion in which some approaches are raised as below:

- Option 1: the UE should set DPCBC field to "0" when the criteria to report DPC is not met.

- Option 2: the network ignores DPCBC field bit when it determines the criteria to report DPC is not met at UE.

Option 1 is from perspective of UE and we wonder how network understands the value 0 in DPCBC field bit, i.e. consider it as 0dB DPC or DPCBC is not available for a band combination. Thanking to the DPC field which set to value 0, the network can know the criteria to report DPC is not met at UE and network can understand DPCBC is not available for a band combination. Hence it is better to have a link with DPC field when understanding DPCBC field. We prefer to modify this option as “If dpc-Reporting-FR1 is configured and all DPC fields set to a value other than 0, DPCBC field indicates the ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC”.

- For Multiple Entry PHR, RAN2 to discuss which option is supported:

- Option 1: If dpc-Reporting-FR1 is configured and all DPC fields set to a value other than 0, DPCBC field indicates the ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC.

- Option 2: the network shall ignore the DPCBC field in case all the DPC fields are set with value 0.

# 3	Conclusion

This contribution makes the following proposals:

- In NTN network, UE starts the ra-ResponseWindow at the first PDCCH occasion as specified in TS 38.213 [6] from the end of all repetitions of the Random Access Preamble transmission plus the UE-gNB RTT.

- For eRedcap, UE selects the set of RA resource that is only configured with eRedcap indication and MSG1 repetition indication and associated with the indicated MSG1 repetition number.

- mTRP PHR MAC CE with assumed PUSCH is not supported

- For Multiple Entry PHR, RAN2 to discuss which option is supported:

- Option 1: If dpc-Reporting-FR1 is configured and all DPC fields set to a value other than 0, DPCBC field indicates the ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC.

- Option 2: the network shall ignore the DPCBC field in case all the DPC fields are set with value 0.

- TP is provided based on the above proposal.

# 4	MAC TP

MSG1 repetition in NTN

5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted on a non-terrestrial network:

3>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the PDCCH occasion as specified in TS 38.213 [6].

2>	else:

3>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission.

2>	monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running.

1>	else:

2>	if the Random Access Preamble was transmitted on a non-terrestrial network and the Random Access Preamble is transmitted without repetitions:

3>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the PDCCH occasion as specified in TS 38.213 [6].

2>	else if the Random Access Preamble is transmitted with repetitions:

3>	if the Random Access Preamble was transmitted on a non-terrestrial network:

4>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion from the end of all repetitions of the Random Access Preamble transmission as specified in TS 38.213 [6] plus the UE-gNB RTT.

3>	else:

4>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion from the end of all repetitions of the Random Access Preamble transmission as specified in TS 38.213 [6].

2>	else:

3>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission.

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission on the search space indicated by recoverySearchSpaceId is received from lower layers on the Serving Cell where the preamble was transmitted; and

eRedCap RA resource set selection

5.1.1b	Selection of the set of Random Access resources for the Random Access procedure

The MAC entity shall:

[Skip]

1>	else if contention-free Random Access Resources with Msg1 repetition have been provided for this Random Access procedure and Msg1 repetition number is indicated in rach-ConfigDedicated, and RedCap is applicable for the current Random Access procedure:

2>	select the set of Random Access resources that is only configured with RedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

1>	else if contention-free Random Access Resources with Msg1 repetition have been provided for this Random Access procedure and Msg1 repetition number is indicated in rach-ConfigDedicated, and eRedCap is applicable for the current Random Access procedure:

2>	select the set of Random Access resources that is only configured with eRedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure and RedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with RedCap indication; or

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure and eRedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with eRedCap indication; or

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure and eRedCap is applicable for the current Random Access procedure and there is no set of Random Access resources available that is only configured with eRedCap indication and there is one set of Random Access resources available that is only configured with RedCap indication:

2>	select this set of Random Access resources for this Random Access procedure.

DPC

Option 1

6.1.3.9	Multiple Entry PHR MAC CE

-	DPCBC: If dpc-Reporting-FR1 is configured and all the DPC field set to a value other than 0 (i.e. the criteria to report DPC is met), this field indicates the ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC, as specified in TS 38.101-1[14] and TS 38.101-3 [16], this field set to 0 indicates the delta power class for band combination is 0dB, this field set to 1 indicates the delta power class for band combination is same or larger than 3 dB. Otherwise, R bit is present, set to 0;

Option 2

6.1.3.9	Multiple Entry PHR MAC CE

-	DPCBC: If dpc-Reporting-FR1 is configured, this field indicates the ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC, as specified in TS 38.101-1[14] and TS 38.101-3 [16], this field set to 0 indicates the delta power class for band combination is 0dB, this field set to 1 indicates the delta power class for band combination is same or larger than 3 dB. Otherwise, R bit is present, set to 0;

NOTE:	When all the DPC fields are set to 0, the DPCBC field is not applicable and ignored by the network.