3GPP TSG-RAN WG2 Meeting #125	R2-2401337

Athens, Greece, 26th Feb 2024 – 1st Mar 2024

Agenda Item:	7.23.2

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues of EventID and gNB Identity handling

Document for:	Discussion and Decision

Discussion

Based on the agreements achieved for the WID of NR Timing Resiliency and URLLC enhancements, the procedures for acquiring 5GS network timing synchronization status and also the detailed 5G Clock quality information are captured in RRC spec, e.g., in section “5.2.2.4.10 Actions upon reception of SIB9” and “5.7.1.3 Reception of the DLInformationTransfer by the UE”.

After first round ASN.1 review, Rapp think the following change suggestions are agreeable [1][2]:

- Merge VarEventID and VarGnbID to new UE variable VarTSS-info. Add definition for “TSS”

- Replace this note (how the UE calculate the value of the gNB identity) with real procedure text

- Remove the condition “if eventID-TSS is included:” in the DLInformationTransfer.

Moreover, there are two remaining issues:

- Issue#1: For V509, related to the deriving the gNB identity, it is proposed that the UE should store a global ID as the current derived gNB identity is only unique within a PLMN. This was not explicitly discussed in the WI and thus it needs to be determined if the variables are kept when UE changes PLMN and if so, if a global gNB Identity is required. Note that this may impact the signalling sizes if for instance the PLMN Id needs to be maintained.

- Issue#2: For V510, in the current spec, it is specified that UE always calculates the gNB ID from PLMN-IdentityInfo and as such does not cover a NPN-only case in which the UE uses NPN-IdentityInfoList.

Based on the agreeable change suggestions, Rapp gives the following modified spec text [2]:

5.2.2.4.10 Actions upon reception of SIB9 Upon receiving SIB9 with referenceTimeInfo, the UE may perform the related actions in clause 5.7.1.3 except of ignoring all further referenceTimeInfo received in SIB9 . Upon receiving SIB9 with eventID-TSS, the UE shall perform the actions below if requested by upper layers: 1> if VarTSS-info has a storedEventID value: 2> if the storedEventID value within VarTSS-info is different from the value received within SIB9: 3> notify upper layers that clockQualityDetailsLevel may have changed; 3> store the storedEventID value within VarTSS-info with the value received within SIB9; 3> derive the gNB identity from gNB-ID-Length and cellIdentity of the first PLMN-IdentityInfo list entry of PLMN-IdentityInfoList in SIB1, as defined in TS 38.413 [53]; 3> store the storedGnbID value within VarTSS-info with the gNB identity value derived from SIB1; 2> else: 3> derive the gNB identity from gNB-ID-Length and cellIdentity of the first PLMN-IdentityInfo list entry of PLMN-IdentityInfoList in SIB1, as defined in TS 38.413 [53]; 3> if the storedGnbID value within VarTSS-info is different from the gNB identity value derived from SIB1: 4> notify upper layers that clockQualityDetailsLevel may have changed; 4> store the storedGnbID value within VarTSS-info with the gNB identity value derived from SIB1; 1> else: 2> store storedEventID within VarTSS-info with the value of eventID-TSS received within SIB9; 2> derive the gNB identity from gNB-ID-Length and cellIdentity of the first PLMN-IdentityInfo list entry of PLMN-IdentityInfoList in SIB1, as defined in TS 38.413 [53]; 2> store storedGnbID within VarTSS-ingo with the gNB identity value derived from SIB1; 2> notify upper layers that clockQualityDetailsLevel may have changed; 5.7.1.3 Reception of the DLInformationTransfer by the UE / … / 1> if clockQualityDetailsLevel is included: 2> store eventID-TSS value in eventID within VarTSS-info; 2> derive the gNB identity from gNB-ID-Length and cellIdentity of the first PLMN-IdentityInfo list entry of PLMN-IdentityInfoList in SIB1, as defined in TS 38.413 [53]; 2> store the storedGnbID within VarTSS-info with the gNB identity value derived from SIB1. 2> forward clockQualityDetailsLevel to upper layers. / … /

We are generally fine with most of the modified texts proposed by Rapp.

For the remaining issue#1, we understand whether gNB identity should be (and whether it can be) a global ID should be decided by RAN3. But considering the remaining issue#1 and #2 from RAN2 perspective, we think it may be more suitable to derive gNB identity from the PLMN-IdentityInfo containing the selected PLMN, instead of from the first PLMN-IdentityInfo list entry of PLMN-IdentityInfoList in SIB1.

Proposal 1: It’s suggested to derive the gNB identity from the PLMN-IdentityInfo containing the selected PLMN, instead of from the first PLMN-IdentityInfo list entry of PLMN-IdentityInfoList in SIB1. The wording suggestion is as below:

2>	derive the gNB identity from gNB-ID-Length and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN or cellIdentity for the cell as received in the corresponding entry of npn-IdentityInfoList containing the selected PLMN or SNPN, as defined in TS 38.413 [53];

There is another issue for the “if requested by upper layers” part in the description “Upon receiving SIB9 with eventID-TSS, the UE shall perform the actions below if requested by upper layers”. Per our understanding, there is no request from upper layers, e.g., NAS layer, to require UE to process the eventID-TSS in SIB9. The only request from NAS layer is that whether to enter connected mode to acquire 5G Clock quality information if NAS layer gets the indication that clockQualityDetailsLevel may have changed. Therefore, we suggest to remove such “if requested by upper layers”.

Proposal 2: To remove “if requested by upper layers” part from the description “Upon receiving SIB9 with eventID-TSS, the UE shall perform the actions below if requested by upper layers” as there is no related request from upper layers.

Conclusion

In this contribution, the following observations and proposals were made:

Proposal 1: It’s suggested to derive the gNB identity from the PLMN-IdentityInfo containing the selected PLMN, instead of from the first PLMN-IdentityInfo list entry of PLMN-IdentityInfoList in SIB1. The wording suggestion is as below:

2>	derive the gNB identity from gNB-ID-Length and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN or cellIdentity for the cell as received in the corresponding entry of npn-IdentityInfoList containing the selected PLMN or SNPN, as defined in TS 38.413 [53];

Proposal 2: To remove “if requested by upper layers” part from the description “Upon receiving SIB9 with eventID-TSS, the UE shall perform the actions below if requested by upper layers” as there is no related request from upper layers.

References

- RIL-List v171_URLLC.xlsx

- TSS_URLLC_RIL_v02_Len.docx