3GPP TSG-RAN WG2 Meeting #125	R2-2400157

Athens, Greece, Feb 26th – Mar 1st, 2024

Source:	vivo

Title:	Discussion on remaining MAC open issues of SL positioning

Agenda Item:	7.2.6

Document for:	Discussion and Decision

- Introduction

This discussion paper mainly focuses on remaining MAC open issues of SL positioning raised by rapporteur.

| SL#06 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether the MAC layer can determine to select multiple SL-PRS transmission when SL-PRS is triggered either by the peer UE or the UE’s own upper layer. |
| --- | --- | --- |
| SL#07 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether the MAC layer can determine to select single SL-PRS transmission when SL-PRS transmission is triggered by its own upper layer or by peer UE. |
| SL#08 | 5.22.1.1 SL Grant reception and SCI transmission | FFS SL-PRS transmission on SL-PRS shared resource pool when the MAC PDU has been positively acked for resource allocation scheme 1 and scheme 2 |
| SL#10 | 5.22.1.1 SL Grant reception and SCI transmission | FFS how the MAC entity determines the SL-PRS delay budget. |
| SL#11 | 5.22.1.1 SL Grant reception and SCI transmission | FFS minimum time gap requirement on SL-PRS shared resource pool. |
| SL#12 | 5.22.1.1 SL Grant reception and SCI transmission | FFS how the SL-PRS resource is determined based on the list of RRC configured SL-PRS configurations, priority, PHY sensing and MAC layer random resource selection for resource allocation scheme 2. |
| SL#13 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether SL-PRS occasion on SL-PRS shared resource pool can be cleared when the MAC PDU has been positively acked for resource allocation scheme 2. |
| SL#14 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether SL-PRS occasion on SL-PRS shared resource pool can be cleared when the MAC PDU has been positively acked for resource allocation scheme 1. |
| SL#15 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether SL-PRS priority is determined by priority in the peer UE’s SCI or the UE’s own higher layer when the trigger comes from the peer UE’s SCI. |
| SL#16 | 5.22.1.1 SL Grant reception and SCI transmission | FFS how SL-PRS priority is determined when SL-PRS is triggered by the UE’s own higher layer. |
| SL#17 | 5.22.1.2a Re-evaluation and Pre-emption 5.22.1.2b Re-selection for using a received resource conflict indication | The same issue as section 5.22.1.1 for the relationship between remaining PDB and SL-PRS delay budget for resource selection on SL-PRS shared resource pool. |

- Discussion

SL#06&07

| SL#06 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether the MAC layer can determine to select multiple SL-PRS transmission when SL-PRS is triggered either by the peer UE or the UE’s own upper layer. |
| --- | --- | --- |
| SL#07 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether the MAC layer can determine to select single SL-PRS transmission when SL-PRS transmission is triggered by its own upper layer or by peer UE. |

According to the legacy discussion on UE behavior in Mode 2 resource allocation, MAC layer decides to select SL grant highly depends on the transmission format of SL MAC PDU(s). Moreover, the transmission format, i.e. single or multiple SL MAC PDU(s), is indicated by the UE’s own upper layer, with the decision of the transmitting sidelink service type. If it is a periodic service, multiple SL MAC PDUs with similar characteristics can be arranged in a SL grant. If it is a burst service, UE can select SL grant specific for the current SL MAC PDU.

In the realm of sidelink positioning, the SL-PRS transmission is anyway triggered by 1) UE’s own upper layer or 2) peer UE’s upper layer (SLPP), 3) peer UE’s lower layer (SCI for double-sided RTT positioning method).

For the first method, Tx UE happens to be server UE, its SLPP layer can decide for itself to be Tx UE and thus indicate information to its MAC layer. In the second method, Tx UE’s SLPP layer should receive a SLPP message from the server UE or LMF, to inform such UE to become Tx UE in a certain transmission and to provide the corresponding SL-PRS configuration (e.g. bandwidth, priority, etc.). Besides, Tx UE’s PHY layer receives SL-PRS and the attached SCI in the last method. With a special SCI format to trigger UE to transmit the second SL-PRS, it would know the exact SL-PRS configuration from what it receives.

In summary, MAC layer of Tx UE is informed of the SL-PRS characteristics for the current transmission opportunity. The MAC layer thus can decide and select the kind of SL grant for single or multiple SL-PRS transmission(s) according to cross-layer interaction.

Proposal 1: With the cross-layer indication from SLPP or PHY layer, Tx UE’s MAC layer can determine and select to create SL grant for single or multiple SL-PRS transmission(s).

SL#08,13,14

| SL#08 | 5.22.1.1 SL Grant reception and SCI transmission | FFS SL-PRS transmission on SL-PRS shared resource pool when the MAC PDU has been positively acked for resource allocation scheme 1 and scheme 2 |
| --- | --- | --- |
| SL#13 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether SL-PRS occasion on SL-PRS shared resource pool can be cleared when the MAC PDU has been positively acked for resource allocation scheme 2. |
| SL#14 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether SL-PRS occasion on SL-PRS shared resource pool can be cleared when the MAC PDU has been positively acked for resource allocation scheme 1. |

In view of the LS sent to RAN1 in RAN2#124 meeting [1], we believe that the above issues should wait for RAN1’s reply, thus no proposal is made.

In addition, RAN2 would like to ask RAN1 the following questions: - On the maximum number of parallel SL-PRS transmission - What is the maximum total number of parallel SL-PRS transmission on SL-PRS shared/dedicated resource pool? - What is the maximum number of parallel SL-PRS transmission supported on a SL-PRS shared resource pool and SL-PRS dedicated resource pool, respectively? - When SL-PRS is transmitted on a SL-PRS shared resource pool where PSFCH is configured, if the associated PSSCH transmission is positively acknowledged, should the UE continue to perform SL-PRS retransmission?

SL#10

| SL#10 | 5.22.1.1 SL Grant reception and SCI transmission | FFS how the MAC entity determines the SL-PRS delay budget. |
| --- | --- | --- |

Inspired by NR sidelink communication, the packet delay budget is determined by the service, and mapped from the PQI provided by the upper layer. In NR sidelink communication, the quality of positioning service is defined by positioning QoS, which has no ready-made identifier to be mapped from.

In our opinion and based on the previous discussion, there can be three options for this issue.

- Option 1: The straightforward way is to leave it as UE implementation. Based on the instruction from the location server, the MAC layer of Tx UE can decide the SL-PRS delay budget itself.

- Option 2: The location server is able to provide such parameter directly. In positioning procedure, it is the location server to schedule and arrange the whole process, and to provide SL-PRS configuration.

- Option 3: Construct a mapping table from positioning QoS/priority to SL-PRS delay budget. This may in the scope of SA2.

We would like to go with Option 2, which could relax UE’s burden and let the location server to fully manage the SL-PRS transmission.

Proposal 2: SL-PRS delay budget can be provided by location server (LMF or server UE) to the Tx UE when triggering SL-PRS transmission.

SL#11&12

| SL#11 | 5.22.1.1 SL Grant reception and SCI transmission | FFS minimum time gap requirement on SL-PRS shared resource pool. |
| --- | --- | --- |
| SL#12 | 5.22.1.1 SL Grant reception and SCI transmission | FFS how the SL-PRS resource is determined based on the list of RRC configured SL-PRS configurations, priority, PHY sensing and MAC layer random resource selection for resource allocation scheme 2. |

From our understanding, the above two issues are out of RAN2 scope, thus no proposal is made.

SL#15&16

| SL#15 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether SL-PRS priority is determined by priority in the peer UE’s SCI or the UE’s own higher layer when the trigger comes from the peer UE’s SCI. |
| --- | --- | --- |
| SL#16 | 5.22.1.1 SL Grant reception and SCI transmission | FFS how SL-PRS priority is determined when SL-PRS is triggered by the UE’s own higher layer. |

On the basis of the above analysis, when SL-PRS transmission is triggered by peer UE’s SCI, the PHY layer of the UE would receive SL-PRS together with such SCI, to realize it is triggered to transmit another SL-PRS. What is more, SCI-triggered method is only applied for double-sided SL-RTT positioning method, meaning that two SL-PRS is of same importance to realize such positioning procedure. In this sense, it is simple that Tx UE directly follows the priority of SL-PRS that the peer UE transmits initially.

Proposal 3: When SL-PRS transmission is triggered by SCI, SL-PRS priority is equal to the priority in the peer UE’s SCI.

As clarified for SL#06/07, SL-PRS transmission triggered by Tx UE’s own higher layer is applied when Tx UE is the server UE. Such that the SLPP layer of Tx UE itself is capable of deciding SL-PRS priority as a location server in the positioning procedure.

Proposal 4: When SL-PRS transmission is triggered by its own higher layer, SL-PRS priority is determined by SLPP layer.

SL#17

| SL#17 | 5.22.1.2a Re-evaluation and Pre-emption 5.22.1.2b Re-selection for using a received resource conflict indication | The same issue as section 5.22.1.1 for the relationship between remaining PDB and SL-PRS delay budget for resource selection on SL-PRS shared resource pool. |
| --- | --- | --- |

At the RAN2#124 meeting, the following agreement was made and we suppose the principle can be reused.

When there are both SL-PRS and SL-SCH data pending for transmission at resource selection, the resource selection should be within the smaller one of the SL-PRS delay budget of the pending SL-PRSs and PDB of the logical channels.

Proposal 5: When selecting SL grant for both SL data and SL-PRS on the shared resource pool, the resource selection should be within the smaller one of the SL-PRS delay budget and PDB of SL data.

- Conclusion

In this contribution, we made the following proposals regarding the remaining MAC open issues of SL positioning:

SL#06&07

Proposal 1: With the cross-layer indication from SLPP or PHY layer, Tx UE’s MAC layer can determine and select to create SL grant for single or multiple SL-PRS transmission(s).

SL#10

Proposal 2: SL-PRS delay budget can be provided by location server (LMF or server UE) to the Tx UE when triggering SL-PRS transmission.

SL#15&16

Proposal 3: When SL-PRS transmission is triggered by SCI, SL-PRS priority is equal to the priority in the peer UE’s SCI.

Proposal 4: When SL-PRS transmission is triggered by its own higher layer, SL-PRS priority is determined by SLPP layer.

SL#17

Proposal 5: When selecting SL grant for both SL data and SL-PRS on the shared resource pool, the resource selection should be within the smaller one of the SL-PRS delay budget and PDB of SL data.

- Reference

- R2-2313848	LS on MAC agreements for SL positioning