3GPP TSG-RAN WG2 Meeting #125	R2-2400172

Athens, Greece, Feb 26th – Mar 1st, 2024

Source:	vivo

Title:	Discussion on the granularity of UE capability on sidelink A2X service

Agenda Item:	7.8.2

Document for:	Discussion and Decision

- Introduction

Based on Rapporteur’s open issue, this discussion paper mainly focuses on how to address UE capability of sidelink A2X service.

- Discussion

We firstly recur to the latest agreement regarding UE capability on sidelink A2X service [1].

RAN2#121bis meeting RAN2 assumes that BRID and DAA services will be delivered on a frequency designated by regulators RAN2#123bis meeting 1. Configure the separate resource pools one-level higher on SL-BWPPoolConfigCommon/ SL-BWPPoolConfig. Adopt the TP for 38.331. 2. Add additional flag to indicate whether the pool is for BRID, DAA, or both (check if this introduces a problem) RAN2#124 meeting sl-A2X capability, with BRID, DAA, and both granularity, that also means that it supports dedicated A2X pools. FFS if it is per UE or FS (as working assumption for CR we implement per UE)

Besides, the dedicated A2X pools are captured as below [2]:

sl-BWP-PoolConfigA2X-r18 SetupRelease {SL-BWP-PoolConfig-r16} OPTIONAL, -- Need M sl-BWP-PoolConfigCommonA2X-r18 SL-BWP-PoolConfigCommon-r16 OPTIONAL, -- Need R

Seeing from above, the newly introduced sidelink pool configurations are still referred to the legacy pool configuration, with an indication flag on a certain kind of A2X service in a pool configuration, to imply the resource pool is dedicated for BFID, DAA or both. In other word, to support A2X dedicated pool configuration is a common UE capability with no variation in frequency bands or feature sets.

Apart from that, as agreed in 121bis meeting, BRID and DAA services are proceeded on the frequency regulated by the operator, with no further confinement on a finer granularity. Once UE supports legacy sidelink data transmission, it is reasonable that UE further supports A2X service on all the regulated frequencies.

Observation: Sidelink A2X resource pool is directly referred to the legacy sidelink resource pool, with no further difference on frequency band or feature set.

In our analysis, the capability to be configured with dedicated sidelink resource pool for A2X service is a per UE capability.

Proposal: Capture sl-A2X-Service as a per-UE capability in TS 38.306.

- Conclusion

In this contribution, we made the following contribution on UE capability of sidelink 2AX service:

Observation: Sidelink A2X resource pool is directly referred to the legacy sidelink resource pool, with no further difference on frequency band or feature set.

Proposal: Capture sl-A2X-Service as a per-UE capability in TS 38.306.

- Reference

- RP-233385 Status Report for Rel-18 WI on NR Support for UAV (Uncrewed Aerial Vehicles)

- TS38.331-i00	Radio Resource Control (RRC) protocol specification