3GPP TSG-RAN WG2 Meeting #125                                                                    R2-2400271

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Source:	CATT, Fujitsu

Title:	Discussion on RAN1 LS on Activation/Deactivation of SP PUCCH for LTM

Agenda Item:	7.4.1.1

Document for:	Discussion and Decision

1. Introduction

In last meeting, a RAN1 LS [1] on the active/deactivate semi-persistent PUCCH report for LTM was sent to RAN2 as follows:

1. Overall Description: With an independent configuration of LTM CSI reporting which RAN2 has agreed, the legacy MAC CE command cannot be used for LTM semi-persistent reporting on PUCCH because the MAC CE is used for the legacy CSI report configuration. It is not clear how the activation/deactivation of semi-persistent PUCCH report for LTM CSI reporting can be supported. 2. Actions: To RAN2 group. ACTION: RAN1 respectfully asks RAN2 to take the issue into account.

As the LS mentioned, the legacy MAC CE command cannot be used for LTM semi-persistent reporting on PUCCH due to it is used for serving cell CSI report.  RAN2 should address the issue of the MAC CE for activation/deactivation SP CSI reporting for LTM. In this contribution, solutions and corresponding spec impact are provided for discussion.

2. Discussion

Background

The legacy SP CSI reporting on PUCCH Activation/Deactivation MAC CE is specified as below with the fixed size of 16 bits:

Figure 1 legacy SP CSI reporting on PUCCH Activation/Deactivation MAC CE

It could indicate for which serving cell and BWP the MAC CE applies via the fields of Serving Cell ID and the BWP ID. The field Si indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList. The report configuration which includes PUCCH resources for SP CSI reporting(with type set to semiPersistentOnPUCCH) in the indicated BWP is mapping with the Si in order.

According to the current spec,ltm-CSI-ReportConfigToAddModList can contain the SP CSI reporting on PUCCHs for LTM, but there is no way to activate/re-deactivate these SP PUCCH configurations since the Si fields only refer to csi-ReportConfigToAddModList and do not refer to ltm-CSI-ReportConfigToAddModList. So a solution in RAN2 for the activation/deactivation is needed.

To address the issue in RAN2, there are two possible solutions (i.e., to extend the legacy MAC CE or define a new MAC CE),

Observation 1: To support the activation/deactivation of SP CSI reporting on PUCCH for LTM, there are two possible solutions,

- Solution 1: extend the legacy MAC CE to support LTM case

- Solution 2: define a new MAC CE

As seen in the Figure 1,S0~S3 in legacy MAC CE can be used to activate/deactivate at most 4 SP PUCCHs, which means the maximum number of SP PUCCHs that can be configured for CSI reporting is 4.。

The limitation is from NR feature list from RAN1 and it is reflected in the UE capabilities as below,

CSI-ReportFramework ::= SEQUENCE { maxNumberPeriodicCSI-PerBWP-ForCSI-Report INTEGER (1..4), maxNumberAperiodicCSI-PerBWP-ForCSI-Report INTEGER (1..4), maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report INTEGER (0..4), maxNumberPeriodicCSI-PerBWP-ForBeamReport INTEGER (1..4), maxNumberAperiodicCSI-PerBWP-ForBeamReport INTEGER (1..4), maxNumberAperiodicCSI-triggeringStatePerCC ENUMERATED {n3, n7, n15, n31, n63, n128}, maxNumberSemiPersistentCSI-PerBWP-ForBeamReport INTEGER (0..4), simultaneousCSI-ReportsPerCC INTEGER (1..8) }

In last RAN1 meeting, a new LS [2] of the UE features list for Rel-18 NR was sent to RAN2. In the LS, the candidate values of LTM CSI report setting for Semi-persistent type are {0,1,2,3,4}, which are same as the legacy.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. NR_Mob_enh2 | 45-1 | Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement 4. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 5. Maximum number of LTM CSI report configs | 2-21 or 2-22 or 2-23 or 2-23a | Yes | No | UE does not support intra-frequency L1 measurement and reports for Rel-18 LTM operation | [Per band/BC] | No | No | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M  L: {1,2,3,4, 6, 8, 9, 12, 16} Component 5 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4} | Optional with capability signalling |

Observation 2: According to RAN1 LS, the maximum number of SP PUCCHs for LTM CSI reporting is 4, which is same as the legacy.

Based on above observation 2, in the remaining of this section, we provide further analysis of the possible solutions.

Solution 1: extend the legacy MAC CE to support LTM

Due to the maximum number of SP PUCCHs for LTM CSI reporting is still four as legacy, the simple solution is to enhance the legacy SP CSI reporting on PUCCH Activation/Deactivation MAC CE , i.e., a R bit is used for differentiating between legacy SP CSI reporting on PUCCH Activation/Deactivation and that for LTM.

Figure 2 Example for enhancing legacy MAC CE for LTM

As illustrated above, the field T indicates the type of the CSI report, if the field set to 0, it indicates the MAC CE is used to activate/deactivate the SP CSI reporting on PUCCH for the serving cell as legacy, and the Semi-Persistent CSI report configuration  refers to the csi-ReportConfigToAddModList. If the field is set to 1, it indicates the MAC CE is used to activate/deactivate the SP CSI reporting on PUCCH for the LTM and the Semi-Persistent CSI report configuration  refers to the ltm-CSI-ReportConfigToAddModList.

The corretion introduced by this solution is only used one R bit to distiguish the different Semi-Persistent CSI report configuration. obviously it is simple.

Observation 3: If legacy SP CSI reporting on PUCCH Activation/Deactivation MAC CE is extended, there will be small spec impacts as below:

- One R bit is used to indicate the MAC CE is used for LTM or not

Solution 2: define a new MAC CE

Another solution is to define a new MAC CE with new eLCID to support the activation/deactivation of the SP CSI reporting on PUCCH for LTM.  With the observation that the maximum number of  the semi-Persistent CSI report configuration within ltm-CSI-ReportConfigToAddModList which includes PUCCH resources for SP CSI reporting in one BWP is four as legacy. One possibile MAC CE format is as following:

Figure 4 Example for new MAC CE for LTM assuming 4 Si fields

The MAC CE format is same as the legacy MAC CE. Different with the legacy MAC CE, the Si refers to the Semi-Persistent CSI report configuration within ltm-CSI-ReportConfigToAddModList. A new eLCID should be used for the new MAC CE.

With a separate MAC CE, it is more flexible for extension for further enhancement in the future considering the feature of the SP CSI report for serving cell in legacy and for LTM may extend respectively. However, it needs more spec changes, e.g., new clauses should be added for the new MAC CE.

Observation 4: A new MAC CE can be defined to support the activation/deactivation of SP CSI reporting on PUCCH for LTM, with more spec impacts:

- an eLCID is to be used

- a new MAC CE is to be defined

- separate clauses related to the new MAC CE are needed

We also provide the needed spec changes that are provided in annexes (i.e., Annex 1 for solution 1, Annex 2 for solution 2) based on TS 38.321 v i.0.0. Since both solutions are workable but it is clear that solution 1 is with less spec impact, solution 1(i.e., to extend the legacy MAC CE) is preferred.

Proposal 1: To support the activation/deactivation of SP CSI reporting on PUCCH for LTM, the legacy MAC CE (i.e., SP CSI reporting on PUCCH activation/deactivation MAC CE) is extended. The TP in annex 1 is adopted.

Since RAN2 solution is related to how to activate/deactivate the layer 1 configuration, it is better to inform the RAN2 solution to RAN1. A draft of the reply LS is provided in Annex 3.

Proposal 2: Send the reply LS to RAN1 to indicate the RAN2 agreement. The draft of the reply LS in annex 3 is adopted.

3. Conclusion

In this contribution, we have discussion on the solution of supporting activation/deactivation of SP CSI reporting on PUCCH for LTM. The observations and proposals are as following,

Observation 1: To support the activation/deactivation of SP CSI reporting on PUCCH for LTM, there are two possible solutions,

- Solution 1: extend the legacy MAC CE to support LTM case

- Solution 2: define a new MAC CE

Observation 2: According to RAN1 LS, the maximum number of SP PUCCHs for LTM CSI reporting is 4, which is same as the legacy.

Observation 3: If legacy SP CSI reporting on PUCCH Activation/Deactivation MAC CE is extended, there will be small spec impacts as below:

- One R bit is used to indicate the MAC CE is used for LTM or not

Observation 4: A new MAC CE can be defined to support the activation/deactivation of SP CSI reporting on PUCCH for LTM, with more spec impacts:

- an eLCID is to be used

- a new MAC CE is to be defined

- separate clauses related to the new MAC CE are needed

Proposal 1: To support the activation/deactivation of SP CSI reporting on PUCCH for LTM, the legacy MAC CE (i.e., SP CSI reporting on PUCCH activation/deactivation MAC CE) is extended. The TP in annex 1 is adopted.

Proposal 2: Send the reply LS to RAN1 to indicate the RAN2 agreement. The draft of the reply LS in annex 3 is adopted.

4. References

- R1-2312642, LS on MAC CE to activate/deactivate semi-persistent PUCCH report for LTM

- R1-2312707, LS on updates to the Rel-18 RAN1 UE features list for NR after RAN1#115

- R1-2312613, Session Notes of AI 8.16.7      Ad-Hoc Chair (AT&T)

- TS 38.321, NR;Medium Access Control (MAC) protocol specificationvi.0.0

Annex 1: TP for solution 1 (extend the legacy SP CSI reporting on PUCCH Activation/Deactivation MAC CE)

### 5.18.6	Activation/Deactivation of Semi-persistent CSI reporting on PUCCH

The network may activate and deactivate the configured Semi-persistent CSI reporting on PUCCH of a Serving Cell by sending the SP CSI reporting on PUCCH Activation/Deactivation MAC CE described in clause 6.1.3.16 or the Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE described in clause 6.1.3.80. The network may activate and deactivate at least one of the configured sub configuration of a configured Semi-Persistent CSI reporting on PUCCH of a Serving Cell by sending the Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE described in clause 6.1.3.80. For each Semi-persistent CSI reporting on PUCCH configuration configured with csi-ReportSubConfigList, the network may deactivate all configured sub configurations by sending the SP CSI reporting on PUCCH Activation/Deactivation MAC CE. The configured Semi-persistent CSI reporting on PUCCH is initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

The network may activate and deactivate the configured Semi-persistent CSI reporting on PUCCH of a Serving Cell for LTM by sending the SP CSI reporting on PUCCH Activation/Deactivation MAC CE described in clause 6.1.3.16. The configured Semi-persistent CSI reporting on PUCCH for LTM is initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

The MAC entity shall:

1>	if the MAC entity receives an SP CSI reporting on PUCCH Activation/Deactivation MAC CE or an Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the SP CSI reporting on PUCCH Activation/Deactivation MAC CE or the Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE.

#### 6.1.3.16	SP CSI reporting on PUCCH Activation/Deactivation MAC CE

The SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	T: If LTM candidate cell(s) is configured for the UE within the cell group, this field indicates the type of the SP CSI reporting. If this field is set to 1,the SP CSI reporting is for LTM, otherwise the SP CSI reporting is for the serving cell. If LTM candidate cell(s) is not configured for the UE within the cell group, the R bit is present instead;

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within ltm-CSI-ReportConfigToAddModList if T is present and set to 1, otherwise this field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest LTM-CSI-ReportConfigId /CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest LTM-CSI-ReportConfigId /CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated. If the Semi-Persistent CSI report configuration i is configured with csi-ReportSubConfigList, the Si field is set to 0 to additionally indicate that all SubConfigurations within csi-ReportSubConfigList shall be deactivated;

-	R: Reserved bit, set to 0.

NOTE:	If a Semi-Persistent CSI report configuration i is configured with csi-ReportSubConfigList, the corresponding Si field is not set to 1.

Figure 6.1.3.16-1: SP CSI reporting on PUCCH Activation/Deactivation MAC CE

Annex 2: TP for solution 2 (define new SP CSI reporting on PUCCH Activation/Deactivation MAC CE for LTM)

### 5.18.x	Activation/Deactivation of Semi-persistent CSI reporting on PUCCH for LTM

The network may activate and deactivate the configured Semi-persistent CSI reporting on PUCCH of a Serving Cell for LTM by sending the SP CSI reporting on PUCCH Activation/Deactivation MAC CE for LTM described in clause 6.1.3.x. The configured Semi-persistent CSI reporting on PUCCH for LTM is initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

The MAC entity shall:

1>	if the MAC entity receives an SP CSI reporting on PUCCH Activation/Deactivation MAC CE for LTM on a Serving Cell:

2>	indicate to lower layers the information regarding the SP CSI reporting on PUCCH Activation/Deactivation MAC CE for LTM.

#### 6.1.3.x	SP CSI reporting on PUCCH Activation/Deactivation MAC CE for LTM

The SP CSI reporting on PUCCH Activation/Deactivation MAC CE for LTM is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within ltm-CSI-ReportConfigToAddModList , as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest LTM-CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest LTM-CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated;

-	R: Reserved bit, set to 0.

Figure 6.1.3.X-1: SP CSI reporting on PUCCH Activation/Deactivation MAC CE for LTM

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

Table 6.2.1-1b: Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 215 | 64 to 279 | Reserved |
| 216 | 280 | SP CSI reporting on PUCCH Activation/Deactivation MAC CE for LTM |
| 217 | 281 | Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE |
| 218 | 282 | Cross-RRH TCI State Indication for UE-specific PDCCH MAC CE |
| 219 | 283 | LTM Cell Switch Command |
| 220 | 284 | Candidate Cell TCI States Activation/Deactivation |
| 221 | 285 | PSI-Based SDU Discard Activation/Deactivation MAC CE |
| 222 | 286 | Enhanced Unified TCI state Activation/Deactivation MAC CE for Joint TCI States |
| 223 | 287 | Enhanced Unified TCI state Activation/Deactivation MAC CE for Separate TCI States |
| 224 | 288 | NCR Access Link Beam Indication MAC CE |
| 225 | 289 | NCR Downlink Backhaul Link Beam Indication MAC CE |
| 226 | 290 | NCR Uplink Backhaul Link Beam Indication MAC CE |
| 227 | 291 | Serving Cell Set based SRS TCI State Indication MAC CE |
| 228 | 292 | SP/AP SRS TCI State Indication MAC CE |
| 229 | 293 | BFD-RS Indication MAC CE |
| 230 | 294 | Differential Koffset |
| 231 | 295 | Enhanced SCell Activation/Deactivation MAC CE with one octet Ci field |
| 232 | 296 | Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field |
| 233 | 297 | Unified TCI States Activation/Deactivation MAC CE |
| 234 | 298 | PUCCH Power Control Set Update for multiple TRP PUCCH repetition MAC CE |
| 235 | 299 | PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE |
| 236 | 300 | Enhanced TCI States Indication for UE-specific PDCCH |
| 237 | 301 | Positioning Measurement Gap Activation/Deactivation Command |
| 238 | 302 | PPW Activation/Deactivation Command |
| 239 | 303 | DL Tx Power Adjustment |
| 240 | 304 | Timing Case Indication |
| 241 | 305 | Child IAB-DU Restricted Beam Indication |
| 242 | 306 | Case-7 Timing advance offset |
| 243 | 307 | Provided Guard Symbols for Case-6 timing |
| 244 | 308 | Provided Guard Symbols for Case-7 timing |
| 245 | 309 | Serving Cell Set based SRS Spatial Relation Indication |
| 246 | 310 | PUSCH Pathloss Reference RS Update |
| 247 | 311 | SRS Pathloss Reference RS Update |
| 248 | 312 | Enhanced SP/AP SRS Spatial Relation Indication |
| 249 | 313 | Enhanced PUCCH Spatial Relation Activation/Deactivation |
| 250 | 314 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 251 | 315 | Duplication RLC Activation/Deactivation |
| 252 | 316 | Absolute Timing Advance Command |
| 253 | 317 | SP Positioning SRS Activation/Deactivation |
| 254 | 318 | Provided Guard Symbols |
| 255 | 319 | Timing Delta |

Annex 3: Draft reply RAN1 LS

1. Overall Description:

RAN2 thanks RAN1 for their LS on MAC CE to activate/deactivate semi-persistent PUCCH report for LTM. Based on the agreement in RAN2, RAN2 would like to provide the RAN2 conclusion as follows:

For the activation/deactivation of SP CSI reporting on PUCCH for LTM, it is supported by extending the legacy SP CSI reporting on PUCCH Activation/Deactivation MAC CE.

2. Actions

To RAN1 group

ACTION: 	RAN2 kindly asks RAN1 to take the above feedback into consideration.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #125bis    		15 April – 19 April 2024		                                 China

TSG RAN WG2 Meeting #126		20 May – 24 May 2024		                                 Japan