3GPP TSG-RAN WG2 Meeting #125	R2-2400583

Athens, Greece, 26 February – 01 March 2024

Agenda item:	7.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issues on SLPP

WID/SID:	NR_pos_enh2

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we provide maintenance proposals on open issues in Rel-18 NR sidelink positioning as listed in the summary document [POST124][POS] [TS 38.355] Open Issue list.

# 2	Discussion

[Issue Rapp005]

RAN1 made the following agreement in RAN#115:

Agreement Update previous agreement on synchronization information exchange with the following modification: To mitigate the impact of synchronization errors between anchor UEs for SL-PRS based measurement, the exchanged synchronization information of anchor UEs between a UE and LMF or another UE includes the following: - The synchronization source type (GNSS, gNB/eNB, and UE) of anchor UEs, - [If the synchronization source of an anchor UE is SyncRef UE, the anchor UE can optionally indicate the coverage status and synchronization connection status (whether the SyncRef UE is directly or indirectly synchronized to GNSS/gNB, or other SyncRef UE) of the SyncRef UE] - If the synchronization source of an anchor UE is gNB/eNB, the anchor UE can further provide cell identity information - [Synchronization quality/accuracy information] - The RTD between anchor UEs.

The same is captured in Clause 8.4.4. of TS 38.214 as below:

<omitted text> The UE may report synchronization source type via [sync-Info-for-SL-TDOA-TOA] and/or relative time difference with the associated quality metric, via [higher layer parameter(s)]. For the SL RSTD measurement, the UE may report a reference UE information. <omitted text>

In the current TS 38.355, the IE SL-RTD-Info provides time synchronization information of anchor UEs between a UE and LMF or another UE. An open issue “Rapp005” is whether ‘syncSourceType’ needs to be added under the IE ‘RTD-InfoListPerTxUE’.

In our view, the current IE SL-RTD-Info reflects well the synchronization information related aforementioned RAN1 agreement. Note that an anchor UE can report ‘The RTD between anchor UEs’ via IE RTD-InfoList and its own 'the synchronization source type’ information via as part of ReferenceRTD-Info with IE syncSourceType in the SL-RTD-Info. In other words, as every anchor UE will report its synchronization source type information, there is no need for anchor UE to repetitively report the synchronization source type of other anchor UEs.

Proposal 1: Do not support addition of syncSourceType under the IE RTD-InfoListPerTxUE.

[Issue Q003]

We generally understand the idea of specifying “core” measurements to be always reported for each specific method, and leaving any other measurements optional.

On the other hand, we see a benefit in leaving all reported parameters as optional as this would allow more flexible signalling. For example, a UE may (be asked to) report only NLOS indicators without any accompanying “core” measurement to validate its usability of the anchor UE for LOS-based positioning, eg at the start of a positioning session, as part of periodic status updates or when resuming a session. The absence of any mandatory core parameters would in such a case trigger an error.

So in order to ensure simple yet flexible signalling with guaranteed core reports, we propose to have an implementation convention by which

- all measurement report IEs are by default optional

- an IE is reported if the associated measurement is requested from the UE and supported by the UE

Clearly, core measurements are assumed to be always supported.

Proposal 2: By implementation, it is assumed that a UE always reports an IE in SL-[method]-ProvideLocationInformation if

- such IE was requested in the associated SL-[method]-RequestLocationInformation message

- the UE has a prior knowledge of the requested information or is capable of determining from measurements.

The following correction may be added to the specification to reflect the above proposal:

#### ProvideLocationInformation

The ProvideLocationInformation message body in an SLPP message is used by Endpoint A to provide positioning measurements or position estimates to Endpoint B.

Note: A UE always reports an IE in SL-AoA/RTT/TDOA/TOA-ProvideLocationInformation if

-	such IE was requested in the associated SL-AoA/RTT/TDOA/TOA-RequestLocationInformation message

-	the UE has a prior knowledge of the requested information or is capable of determining it from measurements.

[Issue Q004]

RAN1 has made the following related agreements:

Agreement Confirm the following working assumption with update: Working assumption Support to indicate to UE(s) with higher layer signaling to report multiple Rx-Tx measurements for the same SL PRS transmission (resp. reception) and different SL PRS receptions (resp. transmissions) for the same pair of UE(s). - FFS: whether the different SL PRS receptions correspond to the same or different SL PRS resources - Note: reporting a single Rx-Tx measurement is also supported - Note: The indicated Rx-Tx time difference measurement is based on actual Tx time.. Agreement Support to indicate to UE(s) with higher layer signaling to report multiple Rx-Tx measurements for the same SL PRS transmission (resp. reception) and up to N different SL PRS receptions (resp. transmissions) for the same pair of UE(s). - FFS: value range of N Agreement For the indicated number N of different SL PRS receptions (resp. transmissions) associated with the same SL PRS transmission (resp. reception), the value range of N is {2, 3, 4}.

As per the RAN1 agreements, an indication to report multiple Rx-Tx measurements as well as reporting of multiple Rx-Tx measurements are to be supported. For example, two SL-RTT measurements (one at each UE) are required for single-sided RTT and three or four SL-RTT measurements are required for double-sided RTT. Here, the measurements are reported for the same SL PRS transmission (resp. reception) and up to N different SL PRS receptions (resp. transmissions) for the same pair of UE(s).

While the double-sided (two-way) RTT method may offer robust timing measurements in the presence of uncompensated clock offsets, this introduces additional signalling overhead and latency in positioning. So, it should be restrictively used under strict conditions and the single-side RTT should be a default mode.

In this regard, it can be left to UEs to determine whether or not 2nd stage SL-RTT round (additional SL PRS exchange) is to be performed and report more measurements. In other words, for same SL PRS transmission (resp. reception), the UEs can be configured to report measurements for up to N different SL PRS receptions (resp. transmissions) for the same pair of UE(s).

Proposal 3: SL-RTT-ProvideLocationInformation to permit specify up to N additional measurements in response to multipleSL-PRS-RxTxTimeDiffRequest in SL-RTT-RequestLocationInformation.

[Issue V003]

RAN1 made the following agreement:

Agreement - The following working assumption is confirmed without the FFS bullet as below: - For SL PRS sequence generation, the parameter nID,seqSL-PRS is defined as below: - nID,seqSL-PRS is provided by higher layers to a Tx UE - Details on higher layers, including consideration of Tx UE’s own higher layer, are up to RAN2 - The higher layer parameter is provided to an Rx UE via LPP/SLPP. - FFS: If (pre-)configured for a resource pool and use of SL PRS for sensing is supported, nID,seqSL-PRS is based on 12 LSB bits CRC of PSCCH associated with the SL PRS - Otherwise (i.e., if not provided by higher layers), nID,seqSL-PRS is based on 12 LSB bits CRC of PSCCH associated with the SL PRS

Regarding the higher layer signalling, the server may currently configure the SL PRS sequence ID at both the TX and RX UEs. Alternatively, the sequence ID may be generated by the TX UE and sent to the server UE who then shares it with the RX UE.

To avoid unnecessary server overhead, it would be desirable to permit the TX UE to share its generated SL PRS sequence ID directly with the RX UE.

Proposal 4: RAN2 to agree that TX UE may share SL PRS sequence ID directly with RX UE.

In TS 38.305, the following note may be added for the Assistance Data transfer procedure of SL-RTT, SL AoA, and SL-TOA positioning procedures:

NOTE: Dependent on the scenario, Endpoint A may be a SL-PRS receiving (Rx) UE and Endpoint B may be a SL-PRS receiving (Tx) UE.

[Issue E006]

Some comments are verbatim copy of the parameter name and as such bring no value. Description of the IEs can already convey information on the IEs.

Proposal 5: Avoid comments that are verbatim copies of the IE name, or differ only slightly (e.g., missing or extra ‘-‘).

Examples:

SL-AoA-AssistanceData ::= SEQUENCE {

applicationLayerID                           OCTET STRING,

expectedSL-AzimuthAoA-AndUncertainty         INTEGER(0..3599),                  -- expected-SL-AoA-and-Uncertainty

expectedSL-ZenithAoA-AndUncertainty          INTEGER(0..1799)        OPTIONAL,  -- expected-SL-AoA-and-Uncertainty

...

}

SL-AoA-MeasElement ::= SEQUENCE {

applicationLayerID                    OCTET STRING,

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-AngleQuality                       MeasurementAngleQuality   OPTIONAL,  -- sl-AngleQuality

sl-AoA-AdditionalPathList             SL-AoA-AdditionalPathList OPTIONAL,

sl-AzimuthAoA-FirstPathResult         INTEGER (0..3599)         OPTIONAL,  -- sl-PRS-AoA

sl-AzimuthAoA-LCS-GCS-Translation     LCS-GCS-Translation       OPTIONAL,  -- sl-LCS-to-GCS-translation

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-FirstPathRSRPP-Result          INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-TimeStamp                          SL-TimeStamp              OPTIONAL,  -- sl-Timestamp

sl-ZenithAoA-FirstPathResult          INTEGER (0..1799)         OPTIONAL,  -- sl-PRS-AoA

sl-ZenithAoA-LCS-GCS-Translation      LCS-GCS-Translation       OPTIONAL,  -- sl-LCS-to-GCS-translation

...

}

SL-PRS-AssistanceData ::= SEQUENCE {

applicationLayerID        OCTET STRING,

sl-PRS-SequenceID         INTEGER(0..4095)    OPTIONAL,  -- SL PRS sequence generation

sl-POS-ARP-ID-Tx          INTEGER (1..4)      OPTIONAL,  -- sl-pos-arpID-Tx

sl-PRS-ResourceId         INTEGER (0..16)     OPTIONAL,  -- sl-PRS-ResourceId

tx-TimeStamp              SL-TimeStamp        OPTIONAL,  -- Tx TimeStamp

...

}

[Issue H0015]

RAN1 made the following agreements:

Agreement Support per ARP based measurement in sidelink positioning. The ARP related information can be reported along with the SL measurement. FFS on details of ARP related information, including whether TEG ID can be reused for such purpose. Agreement For per ARP measurement - The ARP ID of an ARP used for reception can be reported along with SL positioning measurement in measurement report.The ARP ID is used to uniquely identify an ARP associated with a UE - FFS: UE can indicate whether different ARPs for Rx and Tx are used for UE Rx-Tx time difference, if the UE optionally reports the Tx time information - FFS: ARP ID of an ARP used for transmission, and details if supported Agreement For location calculation, the ARP ID of SL PRS transmission can be informed to another UE or LMF by Tx UE informing the association between ARP ID and the already transmitted SL PRS resource(s) as assistance data. Agreement Regarding the association information report between ARP ID and the already transmited SL PRS resource(s): - The association information includes {ARP ID, Tx time stamp, SL PRS resource ID (optional)}.

We observe that even though both TX and RX UEs can have multiple ARPs (and consequently ARP IDs), but only one TX ARP is associated with an SL PRS instance. Existing ASN.1 is thus acceptable and there is no need for any change.

Proposal 6: SL-PRS-AssistanceData specifies SL PRS measurements for a single TX ARP ID.

[Issue  Rapp001]

In current TS 38.355, it is implicitly assumed that an anchor UE can only be static, or whether an anchor UE is static or not cannot be known since in the anchorUE-LocationInformation of SL-PositionCalculationAssistance under CommonSL-PRS-MethodsIEsProvideAssistanceData, only the location coordinates of an anchor UE can be shared and not its velocity information:

#### –	CommonSL-PRS-MethodsIEsProvideAssistanceData

-- ASN1START

-- TAG-COMMONSL-PRS-METHODSIESPROVIDEASSISTANCEDDATA-START

CommonSL-PRS-MethodsIEsProvideAssistanceData ::= SEQUENCE {

sl-PRS-AssistanceDataInfo                        SEQUENCE (SIZE (1..maxNrOfSLTxUEs)) OF SL-PRS-AssistanceData                OPTIONAL,

sl-PositionCalculationAssistanceInfo             SEQUENCE (SIZE (1..maxNrOfSLTxUEs)) OF SL-PositionCalculationAssistance     OPTIONAL,

...

}

. . .

SL-PositionCalculationAssistance ::= SEQUENCE {

anchorUE-LocationInformation         LocationCoordinates           OPTIONAL,

applicationLayerID                   OCTET STRING,

sl-ARP-LocationInfoPerTxUE           SL-ARP-LocationInfoPerTxUE    OPTIONAL,  -- sl-ARP-LocationInfo

...

}

However, especially in V2X or IIoT scenarios, it could be expected that an anchor becomes mobile, i.e., non-static, which would impact the accuracy of the location estimate. It is therefore important to know whether an anchor UE is mobile or not before utilizing the anchor. This information can be signalled by adding also the velocity information of the UE in addition to its location information in SL-PositionCalculationAssistance.

Proposal 7: In addition to location coordinates, SL-PositionCalculationAssistance contains also the velocity information of an anchor UE.

[Issues H016, ZTE004, A006]

Capabilities indication and transfer serve for exchange of information related to UE capability on SL positioning.

In addition to inter-UE retrieval of UE capabilities on a one-to-one basis using unicast connections, there is also the need to support LMF to both push / pull capability reports to / from the AMF. More specifically, Section 4.3.7 of [1] specifies that

    - AMF stores UE Positioning Capability received from an LMF and send the UE Positioning Capability along with the received location request to an LMF, and

    - LMF supports of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

In this context, it is noteworthy that a UE may collect capability reports on behalf of other UEs and forward this information further in which case tagging of individual capability reports with source UE identity would be crutial.

Current ASN.1 tags UE capability report with its applicationLayerID. For example, the capability report for the AoA method is defined as follows:

#### SL-AoA-ProvideCapabilities

The IE SL-AOA-ProvideCapabilities is used to indicate the support of SL-AOA and to provide SL-AOA positioning capabilities.

-- ASN1START

-- TAG-SL-AOA-PROVIDECAPABILITIES-START

SL-AoA-ProvideCapabilities ::= SEQUENCE {

applicationLayerID              OCTET STRING,

positioningModes                PositioningModes,

tenMsUnitResponseTime           PositioningModes    OPTIONAL,

periodicalReporting             PositioningModes    OPTIONAL,

...

}

We see the need to indicate the UE identifier together with the capability report.

Proposal 8: ApplicationLayerID IE remains used in SL-AoA/RTT/TDOA/TOA-ProvideCapabilities to tag UE capability report with its identity.

However, the ApplicationLayerID IE is used separately in all SL positioning methods. It may be more efficient to specify this IE only once in CommonIEsProvideCapabilities.

Proposal 9: RAN2 to consider specifying ApplicationLayerID in CommonIEsProvideCapabilities.

[Issue H006]

An open question remains when does a UE may release assistance data and related information whereby RAN2 agreed not to use delta signalling.

Proposal 10: Assistance data may be released from memory either at the end of a positioning or when a validity time expires.

[Issue Rapp004]

Agreement(RAN1#112-bis)

For provision of assistance information for absolute SL positioning, the anchor UE location information can be provided to LMF or UE.

FFS: which UEs can receive the anchor UE location information (note: which may be decided by other WGs)

FFS on quality information of anchor UE location information.

Agreement(RAN1#113)

Location information of the target UE based on sidelink positioning measurements can be reported at least to LMF.

- FFS: on whether quality information of location is included, e.g., uncertainty etc

- Up to other WGs to determine whether location information of the target UE can be reported to another UE

- Up to RAN2 for signaling details

FFS: whether and how to report per ARP location information.

With regard to location information sharing, RAN1 made the following agreements:

Agreement(RAN2#123bis)

For absolute sidelink positioning, the locations of the anchor UEs are provided to the entity that does the location calculation.

RAN2 made the following agreements:

To address the above agreements with respect to location information of the anchor UEs, location information of anchor UEs can be provided in assistance data. This can be used, for example, in the case of UE-based position estimation, where the locations of the anchors have to be provided to the target UE. Regarding location information of the target UE, no further agreements on reporting have been made. Providing location information requests only from the server covers all relevant scenarios, including ranging where typically the target or the anchor will take up of the role of the server of the corresponding SL positioning session. Even in ranging scenarios, where the server is implemented on a separate node, it should be the server issuing location information requests to the other involved entities.

Therefore, the field descriptions of locationInformationType and additionalInformation in CommonIEsRequestLocationInformation need not be updated, as location information request may only be issued by the SL positioning session server.

Proposal 11: locationInformationType in CommonIEsRequestLocationInformation need not be updated to allow requests from entities other than the SL positioning session server.

[Misc issues]

In current TS 38.355, the target UE or server is unable to realize if an anchor UE becomes unavailable (e.g., due to power limitations) during an ongoing positioning session. Such scenario is, however, very likely in SL positioning as compared to legacy (e.g., due to limited range of PRS, mobility of anchors, etc.). Similarly, as opposed to legacy positioning where positioning anchors are TRPs with known locations, in SL positioning, a new anchor can be found again to replace the unavailable one, which may not be known to the server beforehand. It would be therefore important to address this issue, which would otherwise abrupt the positioning procedures, and currently no UE behaviour is defined to handle such conditions.

In order to inform the target UE or the server (server UE or LMF), the anchor UE may indicate its unavailability, which could be part of SLPP Assistance Data signaling under CommonSL-PRS-MethodsIEsProvideAssistanceData. The anchor UE may provide further information regarding its unavailability, e.g., time duration, area, etc., so as to allow more efficient reselection of anchors. Alternatively, anchor UE may indicate its availability after becoming available again during an ongoing positioning session.

Proposal 12: RAN2 to add new IE/field under CommonSL-PRS-MethodsIEsProvideAssistanceData for the Anchor UE to indicate its unavailability/availability during an ongoing session. The IE may contain further information regarding unavailability, e.g., time duration, area, etc.

# 3	Conclusion

This paper has made the following proposal and the TP can be found in the annex.

Proposal 1: Do not support addition of syncSourceType under the IE RTD-InfoListPerTxUE.

Proposal 2: By implementation, it is assumed that a UE always reports an IE in SL-[method]-ProvideLocationInformation if

- such IE was requested in the associated SL-[method]-RequestLocationInformation message

- the UE has a prior knowledge of the requested information or is capable of determining from measurements.

Proposal 3: SL-RTT-ProvideLocationInformation to permit specify up to N additional measurements in response to multipleSL-PRS-RxTxTimeDiffRequest in SL-RTT-RequestLocationInformation.

Proposal 4: RAN2 to agree that TX UE may share SL PRS sequence ID directly with RX UE.

Proposal 5: Avoid comments that are verbatim copies of the IE name, or differ only slightly (e.g., missing or extra ‘-‘).

Proposal 6: SL-PRS-AssistanceData specifies SL PRS measurements for a single TX ARP ID.

Proposal 7: In addition to location coordinates, SL-PositionCalculationAssistance contains also the velocity information of an anchor UE.

Proposal 8: ApplicationLayerID IE remains used in SL-AoA/RTT/TDOA/TOA-ProvideCapabilities to tag UE capability report with its identity.

Proposal 9: RAN2 to consider specifying ApplicationLayerID in CommonIEsProvideCapabilities.

Proposal 10: Assistance data may be released from memory either at the end of a positioning or when a validity time expires.

Proposal 11: locationInformationType in CommonIEsRequestLocationInformation need not be updated to allow requests from entities other than the SL positioning session server.

Proposal 12: RAN2 to add new IE/field under CommonSL-PRS-MethodsIEsProvideAssistanceData for the Anchor UE to indicate its unavailability/availability during an ongoing session. The IE may contain further information regarding unavailability, e.g., time duration, area, etc.