3GPP TSG-RAN WG2 Meeting #125                                                                R2-2400687

Athens, Greece, February 26 – March 1, 2024

Title: 	Discussion on remaining issues on multi-path relay

Source: 	ZTE, Sanechips

Agenda item:	7.9.3

Document for:	Discussion and Approval

Introduction

In this paper, we will focus on the remaining issues for multi path. Our point of view are presented.

Discussion

RRC state of L2 U2N relay UE

Issue 1.1: Editor’s Note: FFS whether there is other condition to include the new indication, e.g. explicit NW indication, or RRC state of the L2 U2N Relay UE.

In previous RAN2 meeting, if the SRB1 is not configured or suspended on indirect path, PC5-RRC signaling trigger will be used for bring RRC_IDLE/INACTIVE relay UE to RRC connected state. However, the remaining issue related to PC5-RRC signaling trigger mechanism is how does remote UE learn the RRC state of relay UE.

Following two options are considered as candidate solutions:

a)  gNB indicates the RRC state of target relay UE in RRCReconfiguration (i.e., as part of the indirect path configuration)

b)  RRC state is enclosed in PC5 Relay Discovery message sent by the relay UE.

Firstly, we think both of the two options are feasible. Secondly, according to current specification, if multiple suitable NR sidelink U2N Relay UEs are available, it is up to Remote UE implementation to choose one NR sidelink U2N Relay UE. Then option2 provides one criteria for remote UE to do relay selection, e.g. select the relay UE in RRC connected state.

- Support RRC state is enclosed in PC5 Relay Discovery message sent by the relay UE.

Furthermore, some companies also prefer to Rely on NW indication to remote UE to decide whether PC5-RRC trigger is used or not. From our view, we do not see the benefit for network to send the indication explicitly, if network want remote UE to use PC5-RRC trigger signaling, SRB1 without duplication or suspend SRB1 over indirect path can be configured to remote UE. Otherwise, SRB1 with duplication can be configured to remote UE, which means that the legacy triggering mechanism can be re-used. Corresponding behaviour can be left to gNB and remote UE implementation.

- Do not support NW indication to remote UE to indicate whether PC5-RRC trigger is used or not.

Stop condition of T421

Issue 1.2: Editor’s Note: FFS the stop condition for other cases, i.e. PC5-RRC trigger, CONNECTED relay UE

For stop condition of T421, current specification only capture the case that “if SRB1 is configured as split SRB and pdcp-Duplication is configured”. How to handle the case that direct SRB1 is configured or indirect path of split SRB1 is suspended is not clear.

For direct SRB1 and split SRB1 without duplication, RRCReconfigurationComplete message is transmitted via only direct path, then legacy condition(PC5 RLC acknowledgement is received from target L2 U2N Relay UE) can not be used again. In this case, we think the stop condition of T421 for indirect path addition/change should be upon establishing PC5 RRC connection with relay UE.

- Regarding direct SRB1 and split SRB1 without duplication, the stop condition of T421 for indirect path addition/change is upon establishing PC5 RRC connection with relay UE.

Path release/maintain

| Issue 1.3: Editor’s Note: FFS whether/how to indicate PC5 release/maintain for direct path add/modify/release. And for direct path release, FFS whether to include an explicit “directPathRelease” flag in the reconfiguration procedure so that the UE can apply a simpler behaviour. | The baseline procedure is that the MP remote UE releases source path(s) before accessing the target path(s) which is the same as the current MR-DC mobility procedures. But companies think the procedure should be further optimized to avoid some unnecessary UE behavior. Proponent companies can provide TP to show spec impact, based on which RAN2 can make final decision. |
| --- | --- |

It’s FFS on whether PC5 release/maintain for indirect path and direct path release indication should be included in RRC reconfiguration message.

Regarding PC5 link, current sidelink design allow UE to maintain more than one PC5 unicast links with peer UE. In case indirect path is changed or released, whether the unused PC5 link is released can be totally up to UE implementation. And there is no harm to maintain the PC5 connection with released indirect path peer UE.

- Up to UE implementation to maintain or release PC5 connection with relay UE upon indirect path modification/release.

Another issues related to indirect path add/modify/release is security key change issue. According to current specification, if UE needs to handover to a new cell, gNB will send reconfiguration with sync to UE. At the same time, UE can further check whether masterKeyUpdate is included in reconfigurationwithsync message. If included, UE needs to perform security key update and derive the new key. But for each PDCP entity, UE can only use the derived new key to cipher the packet if reestablishPDCP is set. In other words, network will update the key along with PDCP reestablishment, the packet with old key buffered in PDCP entity will be re-transmitted using new key.

Regarding the packet buffered in RLC entity, as shown in following, network can re-configure the RLC entity, and corresponding packet ciphered by old key will be dropped, which may cause a service interruption.

NOTE 3: When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity.

For uplink, network can first establish the new RLC entity, but not release the old RLC entity. With such operation, the packet buffered in the old RLC entity can still be transmitted to gNB. Then it’s up to gNB’s PDCP entity to handle the packet with old key. For downlink, before updating the key, gNB ensures that all packets buffered in RLC entity are transmitted to UE and no PDCP packet with old key will be delivered to RLC entity. Then after all packets buffered in RLC entity are transmitted to UE, gNB update the key and transmit the packet with new key.

When it comes into MP, the packets may be transmitted to network/UE via a relay UE, except RLC entity in remote UE and gNB, more packets ciphered with old key will be buffered in relay UE. Similar with Uu interface, network can re-configure RLC entity in remote UE/relay UE to discard the packet ciphered with old key. And, as mentioned by rapporteur, UE can also release the PC5 connection with relay UE to discard the packet ciphered with old key.

However, release operation of RLC entity/PC5 connection may cause service interruption. And such service interruption may becomes more serious in MP, especially for downlink. Since for downlink in PC5 interface, gNB does not know when the packet is transmitted to remote UE by relay UE, it is possible that there are still remaining packet with old key is buffered in relay UE and gNB update key at remote UE.

To solve this issue, it is suggested to discuss following two options:

Option1. support gNB and UE exchange PDCP switch indication to indicate transmission of packet ciphered with old key is stopped.

Option2. It is up to gNB implementation to configure RLC re-establish for relay UE and remote UE. Upon RLC re-establishment, remote UE re-transmits UL packets that have not been acknowledged by PDCP status report, irrespective of whether RLC-ACK of the packet is received or not. For downlink packet, up to gNB to determine how to re-transmit the packet.

By adopting option1, with this PDCP switch indication, RX PDCP entities in gNB/UE can identifies when the packet is ciphered with new key. By adopting option2, no new indication is needed, if data ciphered with old key is deciphered with new key, the deciphering of the packet will fail and the data packet will not be acknowledged by PDCP status report. Then remote UE or gNB can re-transmit the packets according to PDCP status report.

We think both option1 and option2 are feasible, RAN2 is suggested to discuss option1 and option2.

- Regarding security key update during direct path addition/change, it is suggested to discuss following two options:

Option 1: Support gNB and UE exchanging PDCP switch indication to indicate when the packet is ciphered with new key.

Option 2: It is up to gNB implementation to configure RLC re-establish for relay UE and remote UE. Upon RLC re-establishment, remote UE re-transmits UL packets that have not been acknowledged by PDCP status report, irrespective of whether RLC-ACK of the packet is received or not. For downlink packet, it is up to gNB to determine how to re-transmit the packets.

Regarding Uu link, during CR discussion phase, companies think the direct path release can be implemented using a direct to indirect service continuity procedure from Rel17.

However, as shown in following, in MP scenario, steps highlighted in yellow are not necessary for direct path release, since the indirect path and SL-RLC1 are already established. Steps highlighted in green are necessary, but they need to be modified to implement direct path release(i.e. change “in source side/cell” to “in direct path”).

1> if sl-PathSwitchConfig is included: 2> consider the target L2 U2N Relay UE to be the one indicated by the targetRelayUE-Identity in the sl-PathSwitchConfig; 2> start timer T420 for the corresponding target L2 U2N Relay UE with the timer value set to T420, as included in the sl-PathSwitchConfig; 2> apply the value of the newUE-Identity as the C-RNTI; 2> indicate to upper layer (to trigger the PC5 unicast link establishment) with the target L2 U2N Relay UE indicated by the targetRelayUE-Identity; 2> apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1; - if sl-PathSwitchConfig was included in reconfigurationWithSync included in spCellConfig of an MCG, and when successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE); or, ….. 2> stop timer T304 for that cell group if running; 2> if sl-PathSwitchConfig was included in reconfigurationWithSync: 3> stop timer T420; 3> release all radio resources, including release of the RLC entities and the MAC configuration at the source side; 3> reset MAC used in the source cell;

Additionally, Relying on direct-to-indirect path change to implement direct path release requires the network to provide redundant information in the reconfiguration message (e.g., relay UE ID, new C-RNTI).

Therefore it is suggested to introduce a “direct path release” flag to apply simple UE behavior.

- It is suggested to introduce a “direct path release” flag to apply simple UE behavior.

Indirect path failure type report

| Issue 1.4: Editor’s Note: FFS whether the detailed report types other than indirectPathAddChangeFailure, path failure, Uu-RLF, Uu failure, PC5-RLF can be included. | The EN can be removed. Companies can bring TP to justify new failure type if any. Then RAN2 can discuss new failure types based on companies’ contribution. |
| --- | --- |

It’s FFS whether relay UE handover is included in indirect path failure.

According to last RAN2 meeting’s agreement, the network determines when the indirect path is released at remote UE for relay UE’s HO.  No special-case handling at the relay UE for the case that the relay UE hands over before the indirect path is released.  The MP remote UE does not trigger reestablishment when it receives the notification message for relay HO.

With this agreement, we think the “relayUE-HO” shall not be detected by MP remote UE as a cause for indirect path failure. So, this EN should be removed from the specification.

- “Relay UE Handover” shall not be included as a failure type in IndirectPathFailureInformation.

T421 for scenario 2

| Issue 2.1: Editor’s Note: whether T421 is applicable to scenario 2. | The EN can be removed, since there is no clear benefit to introduce similar timer to scenario 2. |
| --- | --- |

It’s FFS whether T421 is applicable to scenario 2.

Regarding scenario1, T421 timer is started upon receiving the indirect path command, and stopped upon receiving the PC5 RLC ACK for RRCReconfigurationComplete message. Upon T421 timer expiry, UE considers the indirect path addition/change fails, and initiates indirect path failure reporting procedure.

When it comes to scenario2, the link between remote UE and relay UE is a ideal link. From 3GPP perspective, we think remote UE does not need to perform a discovery procedure and link establishment procedure via non-3GPP link. And according to previous agreement, only connected relay UE will be reported to network within scenario2. Therefore, no need to introduce T421 timer to control  indirect path addition/change procedure.

- The link between remote UE and relay UE is an ideal link from 3GPP perspective, we can consider remote UE does not need to perform a discovery procedure and link establishment procedure, and there is no latency to perform indirect path change.

- For Scenario 2, no need to introduce T421 timer to control indirect path addition/change procedure.

Indirect path addition/change

| Issue 1.5: To confirm that upon detecting radio link failure of the direct path while indirect path change or addition is ongoing, RRC reestablishment is triggered. | This issue was raised by company to align the MR-DC handling “upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing, RRC reestablishment is triggered in 5.3.7.2”. Companies can submit RIL or bring TP on this. |
| --- | --- |

One remaining issue related to indirect path addition/change is whether “upon detecting radio link failure of the direct path while indirect path change or addition is ongoing, RRC reestablishment is triggered” is needed.

Regarding path addition, path addition is not completed, we think UE is still a single direct path UE. Legacy RRC re-establishment of Uu interface can handle this issue.

Regarding path change, according to current specification, remote UE will release connection with source relay UE, UE is a single direct path UE during path change, legacy RRC re-establishment of Uu interface can also be reused to handle this issue.

- Legacy RRC reestablishment condition and procedure are reused to handle the case that radio link failure of the direct path is detected while indirect path change or addition is ongoing. No optimization is needed.

Conclusion

In this contribution, following proposals are given:

- Support RRC state is enclosed in PC5 Relay Discovery message sent by the relay UE.

- Do not support NW indication to remote UE to indicate whether PC5-RRC trigger is used or not.

- Regarding direct SRB1 and split SRB1 without duplication, the stop condition of T421 for indirect path addition/change is upon establishing PC5 RRC connection with relay UE.

- Up to UE implementation to maintain or release PC5 connection with relay UE upon indirect path modification/release.

- Regarding security key update during direct path addition/change, it is suggested to discuss following two options:

Option 1: Support gNB and UE exchanging PDCP switch indication to indicate when the packet is ciphered with new key.

Option 2: It is up to gNB implementation to configure RLC re-establish for relay UE and remote UE. Upon RLC re-establishment, remote UE re-transmits UL packets that have not been acknowledged by PDCP status report, irrespective of whether RLC-ACK of the packet is received or not. For downlink packet, it is up to gNB to determine how to re-transmit the packets.

- It is suggested to introduce a “direct path release” flag to apply simple UE behavior.

- “Relay UE Handover” shall not be included as a failure type in IndirectPathFailureInformation.

- Since the link between remote UE and relay UE is an ideal link from 3GPP perspective, it can be considered that remote UE does not need to perform a discovery procedure and link establishment procedure. That is, there is no latency to perform indirect path change.

- For Scenario 2, no need to introduce T421 timer to control indirect path addition/change procedure.

- Legacy RRC reestablishment condition and procedure are reused to handle the case that radio link failure of the direct path is detected while indirect path change or addition is ongoing. No optimization is needed.