3GPP TSG-RAN WG2 Meeting #125	R2-2400758

# Athens, Greece, February 26 – March 01, 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4553 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Field conditions for MUSIM gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current conditions for MUSIM gap fields are spread between a conditional presence table and field descriptions, which is not consistent. For instance, the field description of musim-Starting-SFN-AndSubframe-r17 states that the field is optional for UE assistance information message: “This field is optionally present for the aperiodic MUSIM gap preference indication.” However, the conditional presence table for this field states that the field is mandatory for RRCReconfiguration message, but otherwise absent, which is not correct, since in case of UE assistance information message the field is optional: “This field is mandatory present in case of aperiodic MUSIM gap configuration. Otherwise it is absent.” Generally, this mix of conditions between field descriptions and conditional presence tables can lead to confusion, specially since MUSIM-GapInfo is used both for UE assistance information message and for RRCReconfiguration message. Hence, the field description should not contain elements of conditional presence, which should be captured in the table for field conditional presence. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: MUSIM gaps Inter-operability: If the network implements the CR and the UE does not, the UE and the NW may have different logic on which fields are mandatory or optional for UE assistance information message and RRCReconfiguration message. If the UE implements the CR and the network does not, the UE and the NW may have different logic on which fields are mandatory or optional for UE assistance information message and RRCReconfiguration message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change field description and conditional presence for MUSIM gap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may lead to confusion whether certain fields are optional or mandatory for MUSIM gap. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### –	MUSIM-GapInfo

The IE MUSIM-GapInfo is used to indicate MUSIM gap parameters.

MUSIM-GapInfo information element

-- ASN1START

-- TAG-MUSIM-GAPINFO-START

MUSIM-GapInfo-r17 ::=               SEQUENCE {

musim-Starting-SFN-AndSubframe-r17  MUSIM-Starting-SFN-AndSubframe-r17             OPTIONAL, -- Cond aperiodic

musim-GapLength-r17                 ENUMERATED {ms3, ms4, ms6, ms10, ms20}         OPTIONAL, -- Cond gapSetup

musim-GapRepetitionAndOffset-r17    CHOICE {

ms20-r17                            INTEGER (0..19),

ms40-r17                            INTEGER (0..39),

ms80-r17                            INTEGER (0..79),

ms160-r17                           INTEGER (0..159),

ms320-r17                           INTEGER (0..319),

ms640-r17                           INTEGER (0..639),

ms1280-r17                          INTEGER (0..1279),

ms2560-r17                          INTEGER (0..2559),

ms5120-r17                          INTEGER (0..5119),

...

}                                                                                  OPTIONAL -- Cond periodic

}

MUSIM-Starting-SFN-AndSubframe-r17 ::= SEQUENCE {

starting-SFN-r17                       INTEGER (0..1023),

startingSubframe-r17                   INTEGER (0..9)

}

-- TAG-MUSIM-GAPINFO-STOP

-- ASN1STOP

MUSIM-GapInfo field descriptions

musim-GapLength Indicates the length of the UE's MUSIM gap as specified in TS 38.133 [14] clause 9.1.10.

musim-GapRepetitionAndOffset Indicates the gap repetition period in ms and gap offset in number of subframes for the periodic MUSIM gap as specified in TS 38.133 [14] clause 9.1.10.

musim-Starting-SFN-AndSubframe Indicates gap starting position for the aperiodic MUSIM gap.

starting-SFN Indicates gap starting SFN number for the aperiodic MUSIM gap.

startingSubframe Indicates gap starting subframe number for the aperiodic MUSIM gap.

| Conditional Presence | Explanation |
| --- | --- |
| aperiodic | For MUSIM gap configuration, this field is mandatory present in case of aperiodic MUSIM gap configuration and absent otherwise. For MUSIM gap preference indication, this field is optionally present in case of aperiodic MUSIM gap preference indication and absent otherwise. |
| gapSetup | For MUSIM gap configuration, this field is mandatory present upon configuration of a new MUSIM gap and optionally present, Need M, otherwise. For MUSIM gap preference indication, this field is mandatory present for both periodic gap and aperiodic gap preference indication. |
| periodic | For MUSIM gap configuration, this field is mandatory present in case of periodic MUSIM gap configuration and absent otherwise. For MUSIM gap preference indication, this field is optionally present in case of periodic MUSIM gap preference indication and absent otherwise. |

END OF CHANGE