3GPP TSG-RAN WG2 Meeting #125                                                      R2-2401100

Athens, Greece, Feb 26th – Mar 1th, 2024

Source:              CATT

Title:	             Discussion on Cell DTX/DRX UE capability

Agenda Item: 	7.3.3

Document for:	Discussion and Decision

Introduction

During RAN2 #123 meeting, we achieve the following agreement.

RAN2#123 agreement:

We focus on the case where DTX in RRC can only be configured when C-DRX is configured.  We will not optimize for the case where C-DRX is not configured.

But in current spec, this agreement is not reflected. So we provide the TP to capture this agreement.

Discussion

In current spec of TS38.306 [1], we have defined a UE capability on cell DTX/DRX as below. UE can report this UE capability to network to indicate whether UE supports cell DTX only or cell DRX only or both.

| nes-CellDTX-DRX-r18 Indicates whether the UE supports cell DTX and/or DRX operation by RRC configuration. The supported number of cell DTX/DRX patterns per cell group is 2, regardless of each pattern is for cell DTX only, cell DRX only, or both. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Besides, in current spec of TS38.306, we have defined the UE capability longDRX-Cycle to indicate UE supports long DRX cycle as below:

| longDRX-Cycle Indicates whether UE supports long DRX cycle as specified in TS 38.321 [8]. | UE | Yes | Yes | No |
| --- | --- | --- | --- | --- |

Long DRX cycle configuration is mandatory configured when C-DRX cycle is configured to a UE. Hence, if a UE indicates it supports long DRX cycle, it means C-DRX can be configured to the UE.

In RAN2#123 meeting, we have agreed that we focus on the case where cell DTX in RRC can only be configured when C-DRX is configured and will not optimize for case where C-DRX is not configured. But this agreement has not been reflected in current UE capability spec. If a UE only indicates support of nes-CellDTX-DRX-r18 per band but does not indicate it supports long DRX cycle, cell DTX cannot be configured to the UE. Therefore, we think we need to add the limitation that a UE supporting cell DTX shall also indicate support of longDRX-Cycle in the field description of UE capability for cell DTX and/or cell DRX. And we provide a TP on TS38.306 in the Appendix.

Proposal 1: Add the limitation that a UE supporting cell DTX shall also indicate support of longDRX-Cycle in the field description of nes-CellDTX-DRX to capture the agreement.

Proposal 2: Adopt the TP for TS38.306 provided in Appendix.

Conclusion

According to the analysis in section 2, it is proposed that:

Proposal 1: Add the limitation that a UE supporting cell DTX shall also indicate support of longDRX-Cycle in the field description of nes-CellDTX-DRX to capture the agreement.

Proposal 2: Adopt the TP for TS38.306 provided in Appendix.

Reference

- TS38.306, User Equipment (UE) radio access capabilities(Release 18), 3GPP

Appendix

5.1 Text proposal for TS38.306

<<<<<<<<<<<<<<<<<<<< Start of change >>>>>>>>>>>>>>>>>>>>

4.2.7.2		BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17 Indicates whether the UE supports DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signalling via DCI format 4_2. A UE supporting this feature shall also indicate support of ack-NACK-FeedbackForMulticast-r17 and dynamicMulticastDCI-Format4-2-r17. | Band | No | N/A | N/A |
| ack-NACK-FeedbackForSPS-MulticastWithDCI-Enabler-r17 Indicates whether the UE supports DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signalling via DCI format 4_2. A UE supporting this feature shall also indicate support of ack-NACK-FeedbackForSPS-Multicast-r17 and sps-MulticastDCI-Format4-2-r17. | Band | No | N/A | N/A |

[…]

| nesBasedCondHandoverWithDCI-r18 Indicates whether the UE supports DCI-based enabling/disabling NES-specific CHO execution condition, i.e. NES-specific CHO execution condition based on source cell NES mode indicated via DCI format 2_9 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| nes-CellDTX-DRX-r18 Indicates whether the UE supports cell DTX and/or DRX operation by RRC configuration. The supported number of cell DTX/DRX patterns per cell group is 2, regardless of each pattern is for cell DTX only, cell DRX only, or both. A UE supporting cell DTX feature shall also indicate support of longDRX-Cycle. | Band | No | N/A | N/A |
| nes-CellDTX-DRX-DCI2-9-r18 Indicates whether the UE supports cell DTX/DRX configuration activation and deactivation via DCI 2_9. A UE supporting this feature shall also indicate support of nes-CellDTX-DRX-r18. | Band | No | N/A | N/A |
| nonGroupSINR-reporting-r16 Indicates N_max L1-SINR values reported when UE supports non-group based L1-SINR reporting. UE indicates support of this feature shall indicate support of ssb-csirs-SINR-measurement-r16. | Band | No | N/A | N/A |

[…]

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>