3GPP TSG-RAN WG2 Meeting #125	R2-2401204

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda item:	7.4.1.4

Source:	Samsung

Title:	Support of L1/L2 controlled LTM CSI reporting

WID/SID:	NR_Mob_enh2-Core

Document for:	Discussion and Decision

Introduction

RAN1 sent LS (R1-2312642 [3]) to ask to support LTM semi-persistent reporting on PUCCH because this is not supported RAN2 perspective in the latest version of the specifications.

With an independent configuration of LTM CSI reporting which RAN2 has agreed, the legacy MAC CE command cannot be used for LTM semi-persistent reporting on PUCCH because the MAC CE is used for the legacy CSI report configuration. It is not clear how the activation/deactivation of semi-persistent PUCCH report for LTM CSI reporting can be supported.

Regarding the LTM CSI reporting, we think there are two aspects need to be further discussed in terms of MAC CE support.

- LTM semi-persistent CSI reporting

- LTM aperiodic CSI reporting

In this contribution, we provide the view to support LTM CSI reporting by MAC CE(s).

Discussion

Introduction

For legacy CSI resource configuration and CSI reporting schemes are various, there are different ways to trigger the CSI report (i.e. periodic, semi-persistent, aperiodic). Below Table 1 describes the triggering/activation of CSI reporting for the possible CSI-RS Configurations.

|  | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS configuration | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command by MAC CE; For reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, activation command by MAC CE; |
| Semi-Persistent CSI-RS configuration | Not Supported | For reporting on PUCCH, the UE receives an activation command by MAC CE; For reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, activation command by MAC CE; |
| Aperiodic CSI-RS configuration | Not Supported | Not Supported | Triggered by DCI; additionally, activation command by MAC CE; |

Table 1. Triggering/Activation of CSI Reporting for the possible CSI-RS Configurations.

Support of LTM CSI reporting

For LTM CSI resource configuration [1], the common LTM CSI resource configuration which is consist of SSB index and LTM candidate ID is located out of configuration for LTM candidate cells. In addition, the LTM CSI report configuration is located in each (candidate) serving cell configuration (i.e. CSI-MeasConfig IE). However the actual RRC parameters to support LTM CSI report are almost same as the legacy CSI reporting based on RAN1 agreements. That is, LTM CSI report supports 1) periodic report, 2) semi-persistent report on PUCCH, 3) semi-persistent report on PUSCH and 4) aperodic report on PUSCH.

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha                                            P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

}

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...

}

Observation 1: Based on the current RRC parameters in LTM-CSI-ReportConfig-r18, 1) periodic report, 2) semi-persistent report on PUCCH, 3) semi-persistent report on PUSCH and 4) aperodic report on PUSCH are supported.

If we follow to support CSI reporting for each reporting method, MAC CE signalling to activate the CSI resources is required at least for following two CSI reporting schemes.

- LTM semi-persistent CSI reporting

- LTM aperiodic CSI reporting

For LTM semi-persistent CSI reporting, as RAN1 requested in LS [3], it is quite clear the new MAC CE is

The basic question would be we can reuse the legacy MAC CEs or the new MAC CEs are necessary for LTM CSI reporting. According to the legacy CSI reporting MAC CE (see section5 Appendix below [2]) formats and the RRC parameters, we can know if the new MAC CE is required or not:

- SP CSI reporting on PUCCH

  - LTM-CSI-ResourceConfigId is associated with LTM-CSI-ReportConfigId, and it is independently configured with the legacy CSI-ReportConfig

  - Si for SP CSI reporting on PUCCH Activation/Deactivation MAC CE

    - indicates the the CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH

Based on the above, the Si field in the legacy MAC CE refers the RRC parameter for legacy CSI reporting and we don’t think reusing the same MAC CE format for the different use cases is efficient. Introducing the new MAC CEs which can be similar format with the legacy, for LTM SP CSI reporting is more simple approach in terms of implementation perspective.

Proposal 1: Introduce the new MAC CEs for LTM semi-persistent report on PUCCH, legacy MAC CE format could be the baseline. See the TP in Appendix.

However, it is different case for LTM AP CSI reporting because the RRC structure/signalling for LTM AP CSI triggering state is provided using the legacy field (i.e. CSI-AperiodicTriggerState) based on RAN1 input.

| CSI-AperiodicTriggerState |  | LTM-associatedReportConfigInfo | new |  | Configures the aperiodic CSI reports of candidate cells. If ltm-associatedReportConfigInfoList is configured the UE ignores the field associatedReportConfigInfoList. | LTM-CSI-ReportConfigId |
| --- | --- | --- | --- | --- | --- | --- |
| CSI-SemiPersistentOnPUSCH-TriggerState |  | LTM-associatedReportConfigInfo | new |  | Configures the semi-persistent CSI reports on PUSCH of candidate cells. If ltm-associatedReportConfigInfo is configured the UE ignores the field associatedReportConfigInfo | LTM-CSI-ReportConfigId |

CSI-MeasConfig ::=                  SEQUENCE {

-- skipped

aperiodicTriggerStateList           SetupRelease { CSI-AperiodicTriggerStateList }                            OPTIONAL, -- Need M

-- skipped

}

CSI-AperiodicTriggerStateList ::=   SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState

CSI-AperiodicTriggerState ::=       SEQUENCE {

associatedReportConfigInfoList      SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo,

...,

[[

ap-CSI-MultiplexingMode-r17         ENUMERATED {enabled}                                          OPTIONAL  -- Need R

]],

[[

ltm-AssociatedReportConfigInfo-r18  LTM-CSI-ReportConfigId-r18                                    OPTIONAL  -- Need R

]]

}

- AP CSI reporting on PUSCH

  - Ti for Aperiodic CSI Trigger State Subselection MAC CE

  - indicates the selection status of the Aperiodic Trigger States configured within aperiodicTriggerStateList

Since aperiodicTriggerStateList is optional, we may create a new aperiodicTriggerStateList for LTM. With this approach, we could save signalling by not sending associatedReportConfigInfoList i.e. maxNrofReportConfigPerAperiodicTrigger is 16 and maxNrOfCSI-AperiodicTriggers is 128.

From our understanding, there are two ways to support LTM AP CSI reporting.

- Option 1 (Current signaling i.e. ltm-AssociatedReportConfigInfo-r18 is signaled under the legacy CSI-AperiodicTriggerState IE)

  - Pros: No need the new MAC CE (i.e. reusing the legacy MAC CE)

  - Cons: Not efficient especially both the legacy AP CSI and LTM AP CSI is configured by NW (size of configuration is reduced)

- Option 2 (i.e. new independent RRC IE for ltm-AssociatedReportConfigInfo-r18)

  - Pros: Separate handling of LTM AP CSI triggering state using the maximum size

  - Cons: Need to change the specification (i.e. complex than the option 1)

Proposal 2: RAN2 discuss/confirm how to support LTM AP CSI report either Option 1 or Option 2.

- Option 1 (Current signaling in the RRC specification)

  - ltm-AssociatedReportConfigInfo-r18 is signaled under the legacy CSI-AperiodicTriggerState IE

  - Reusing the legacy Aperiodic CSI Trigger State Subselection MAC CE

- Option 2

  - New RRC IE for ltm-AssociatedReportConfigInfo-r18

  - Reusing the legacy Aperiodic CSI Trigger State Subselection MAC CE but need to update the referred field for LTM AP CSI reporting case.

Conclusion

Based on the discussion, we have the following proposals.

Observation 1: Based on the current RRC parameters in LTM-CSI-ReportConfig-r18, 1) periodic report, 2) semi-persistent report on PUCCH, 3) semi-persistent report on PUSCH and 4) aperodic report on PUSCH are supported.

Proposal 1: Introduce the new MAC CEs for LTM semi-persistent report on PUCCH, legacy MAC CE format could be the baseline. See the TP in Appendix.

Proposal 2: RAN2 discuss/confirm how to support LTM AP CSI report either Option 1 or Option 2.

- Option 1 (Current signaling in the RRC specification)

  - ltm-AssociatedReportConfigInfo-r18 is signaled under the legacy CSI-AperiodicTriggerState IE

  - Reusing the legacy Aperiodic CSI Trigger State Subselection MAC CE

- Option 2

  - New RRC IE for ltm-AssociatedReportConfigInfo-r18

  - Reusing the legacy Aperiodic CSI Trigger State Subselection MAC CE but need to update the referred field for LTM AP CSI reporting case.

References

[1] 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification (Release 18), version 0.0.

[2] 3GPP TS 38.321 Medium Access Control (MAC) protocol specification (Release 18), version 0.0.

[3] R1-2312642		LS on MAC CE to activate/deactivate semi-persistent PUCCH report for LTM	 RAN1.

Appendix

1) Legacy MAC CEs for CSI reporting

#### 6.1.3.13 Aperiodic CSI Trigger State Subselection MAC CE The Aperiodic CSI Trigger State Subselection MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a variable size consisting of following fields: - Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits; - BWP ID: This field indicates a DL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits; - Ti: This field indicates the selection status of the Aperiodic Trigger States configured within aperiodicTriggerStateList, as specified in TS 38.331 [5]. T0 refers to the first trigger state within the list, T1 to the second one and so on. If the list does not contain entry with index i, MAC entity shall ignore the Ti field. The Ti field is set to 1 to indicate that the Aperiodic Trigger State i shall be mapped to the codepoint of the DCI CSI request field, as specified in TS 38.214 [7]. The codepoint to which the Aperiodic Trigger State is mapped is determined by its ordinal position among all the Aperiodic Trigger States with Ti field set to 1, i.e. the first Aperiodic Trigger State with Ti field set to 1 shall be mapped to the codepoint value 1, second Aperiodic Trigger State with Ti field set to 1 shall be mapped to the codepoint value 2 and so on. The maximum number of mapped Aperiodic Trigger States is 63; - R: Reserved bit, set to 0. Figure 6.1.3.13-1: Aperiodic CSI Trigger State Subselection MAC CE #### 6.1.3.16 SP CSI reporting on PUCCH Activation/Deactivation MAC CE The SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields: - Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits; - BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits; - Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated; - R: Reserved bit, set to 0. Figure 6.1.3.16-1: SP CSI reporting on PUCCH Activation/Deactivation MAC CE

2) TP for TS 38.321

### 5.18.YY	LTM Activation/Deactivation of Semi-persistent CSI reporting on PUCCH

The network may activate and deactivate the configured LTM Semi-persistent CSI reporting on PUCCH of a Serving Cell by sending the LTM SP CSI reporting on PUCCH Activation/Deactivation MAC CE described in clause 6.1.3.YY. The configured LTM Semi-persistent CSI reporting on PUCCH is initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync.

The MAC entity shall:

1>	if the MAC entity receives an LTM SP CSI reporting on PUCCH Activation/Deactivation MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the LTM SP CSI reporting on PUCCH Activation/Deactivation MAC CE.

#### 6.1.3.YY	LTM SP CSI reporting on PUCCH Activation/Deactivation MAC CE

The SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	Si: This field indicates the activation/deactivation status of the LTM Semi-Persistent CSI report configuration within ltm-CSI-ReportConfigToAddModList, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for LTM SP CSI reporting in the indicated BWP and has the lowest LTM-CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH-r18, S1 to the report configuration which includes PUCCH resources for LTM SP CSI reporting in the indicated BWP and has the second lowest ltm-CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH-r18 in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding LTM Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding LTM Semi-Persistent CSI report configuration i shall be deactivated;

-	R: Reserved bit, set to 0.

Figure 6.1.3.16-1: LTM SP CSI reporting on PUCCH Activation/Deactivation MAC CE

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1, 6.2.1-1c and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The size of the LCID field is 6 bits. If the LCID field is set to 34, one additional octet is present in the MAC subheader containing the eLCID field and follow the octet containing LCID field. If the LCID field is set to 33, two additional octets are present in the MAC subheader containing the eLCID field and these two additional octets follow the octet containing LCID field;

NOTE 1:	For MBS broadcast, a logical channel is identified based on G-RNTI and LCID if the same LCID is allocated for logical channels corresponding to different G-RNTIs.

-	eLCID: The extended Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE as described in tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits.

NOTE 2:	The extended Logical Channel ID space using two-octet eLCID and the relevant MAC subheader format is used, only when configured, on the NR backhaul links between IAB nodes or between IAB node and IAB Donor, or for multicast MTCHs.

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel of DCCH, DTCH and multicast MTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35–46 | Reserved |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-1a Values of two-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to (216 – 1) | 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-1b Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 225 | 64 to 289 | Reserved |
| 226 | 290 | LTM SP CSI reporting on PUCCH Activation/Deactivation |
| 227 | 291 | Serving Cell Set based SRS TCI State Indication MAC CE |
| 228 | 292 | SP/AP SRS TCI State Indication MAC CE |
| 229 | 293 | BFD-RS Indication MAC CE |
| 230 | 294 | Differential Koffset |
| 231 | 295 | Enhanced SCell Activation/Deactivation MAC CE with one octet Ci field |
| 232 | 296 | Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field |
| 233 | 297 | Unified TCI States Activation/Deactivation MAC CE |
| 234 | 298 | PUCCH Power Control Set Update for multiple TRP PUCCH repetition MAC CE |
| 235 | 299 | PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE |
| 236 | 300 | Enhanced TCI States Indication for UE-specific PDCCH |
| 237 | 301 | Positioning Measurement Gap Activation/Deactivation Command |
| 238 | 302 | PPW Activation/Deactivation Command |
| 239 | 303 | DL Tx Power Adjustment |
| 240 | 304 | Timing Case Indication |
| 241 | 305 | Child IAB-DU Restricted Beam Indication |
| 242 | 306 | Case-7 Timing advance offset |
| 243 | 307 | Provided Guard Symbols for Case-6 timing |
| 244 | 308 | Provided Guard Symbols for Case-7 timing |
| 245 | 309 | Serving Cell Set based SRS Spatial Relation Indication |
| 246 | 310 | PUSCH Pathloss Reference RS Update |
| 247 | 311 | SRS Pathloss Reference RS Update |
| 248 | 312 | Enhanced SP/AP SRS Spatial Relation Indication |
| 249 | 313 | Enhanced PUCCH Spatial Relation Activation/Deactivation |
| 250 | 314 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 251 | 315 | Duplication RLC Activation/Deactivation |
| 252 | 316 | Absolute Timing Advance Command |
| 253 | 317 | SP Positioning SRS Activation/Deactivation |
| 254 | 318 | Provided Guard Symbols |
| 255 | 319 | Timing Delta |

Table 6.2.1-1c Values of LCID for MBS broadcast on DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | MCCH |
| 1–32 | Identity of the logical channel of broadcast MTCH |
| 33–63 | Reserved |

Table 6.2.1-2 Values of LCID for UL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]), except for a RedCap UE |
| 1–32 | Identity of the logical channel of DCCH and DTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) for a RedCap UE |
| 36 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) for a RedCap UE |
| 37–42 | Reserved |
| 43 | Truncated Enhanced BFR (one octet Ci) |
| 44 | Timing Advance Report |
| 45 | Truncated Sidelink BSR |
| 46 | Sidelink BSR |
| 47 | Reserved |
| 48 | LBT failure (four octets) |
| 49 | LBT failure (one octet) |
| 50 | BFR (one octet Ci) |
| 51 | Truncated BFR (one octet Ci) |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]), except for a RedCap UE |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

Table 6.2.1-2a Values of two-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to (216 – 1) | 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-2b Values of one-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 228 | 64 to 292 | Reserved |
| 229 | 293 | Enhanced Multiple Entry PHR for multiple TRP (four octets Ci) |
| 230 | 294 | Enhanced Multiple Entry PHR for multiple TRP (one octets Ci) |
| 231 | 295 | Enhanced Single Entry PHR for multiple TRP |
| 232 | 296 | Enhanced Multiple Entry PHR (four octets Ci) |
| 233 | 297 | Enhanced Multiple Entry PHR (one octets Ci) |
| 234 | 298 | Enhanced Single Entry PHR |
| 235 | 299 | Enhanced BFR (one octet Ci) |
| 236 | 300 | Enhanced BFR (four octet Ci) |
| 237 | 301 | Truncated Enhanced BFR (four octet Ci) |
| 238 | 302 | Positioning Measurement Gap Activation/Deactivation Request |
| 239 | 303 | IAB-MT Recommended Beam Indication |
| 240 | 304 | Desired IAB-MT PSD range |
| 241 | 305 | Desired DL Tx Power Adjustment |
| 242 | 306 | Case-6 Timing Request |
| 243 | 307 | Desired Guard Symbols for Case 6 timing |
| 244 | 308 | Desired Guard Symbols for Case 7 timing |
| 245 | 309 | Extended Short Truncated BSR |
| 246 | 310 | Extended Long Truncated BSR |
| 247 | 311 | Extended Short BSR |
| 248 | 312 | Extended Long BSR |
| 249 | 313 | Extended Pre-emptive BSR |
| 250 | 314 | BFR (four octets Ci) |
| 251 | 315 | Truncated BFR (four octets Ci) |
| 252 | 316 | Multiple Entry Configured Grant Confirmation |
| 253 | 317 | Sidelink Configured Grant Confirmation |
| 254 | 318 | Desired Guard Symbols |
| 255 | 319 | Pre-emptive BSR |