3GPP TSG-RAN WG2 Meeting #125                                                                        R2-2401212

Athens, Greece, February 26th  – March 1st, 2024

Agenda item:	7.8.2

Source:	China Telecom

Title:	Discussion on Measurement Reporting Enhancements

Document for:	Discussion

- Introduction

In RAN2#124 meeting [1] [2], the following agreement on the case when multiple measurement events associated with the same event type are triggered simultaneously was made:

Agreements from RAN2#124:

1	When multiple events are configured simultaneously, network explicitly configures whether the UAV reports all triggered measurement reports or chooses the MR configuration corresponding to the triggered event with the smallest value between the altitude of the UAV and the altitude threshold.  This flag applies for all events of the same type (Hx and AxHy) and MO (AxHy).  This will be a separate capability

However, the procedure texts in the current specification still has some minor issues to be discussed [3]. This paper mainly discusses the details of simulMultiTriggerSingleMeasReport in measurement reporting.

- Discussion

simulMultiTriggerSingleMeasReport is applied to events that are triggered to send measurement reports. Per TS38.331 v18.0.0 [3], the measurement report triggering procedures for the same event type in section 5.5.4.1, are quoted as follows:

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical:

3>	if the corresponding measObject concerns NR:

4>	if the eventH1 or eventH2 is configured in the corresponding reportConfig:

5>	for all the events of the same type for which simulMultiTriggerSingleMeasReport is set to true and the entry condition applicable for the event has been satisfied:

6>	consider only the event with the smallest value between the altitude of the UE and the corresponding altitude threshold to be applicable;

4>	else if the eventA3H1 or eventA3H2 or eventA4H1 or eventA4H2 or eventA5H1 or eventA5H2 is configured in the corresponding reportConfig:

5>	for all the events of the same type associated with the same measObjectNR for which simulMultiTriggerSingleMeasReport is set to true and the entry conditions applicable for the event has been satisfied:

6>	consider only the event with the smallest value between the altitude of the UE and the corresponding altitude threshold to be applicable;

In the current text procedure and field description, simulMultiTriggerSingleMeasReport is applied to events that satisfy entering conditions. However, the agreement in RAN2#124 is about configuration for measurement reporting, rather than entry conditions. Satisfying the entry condition should not be confused with triggering measurement report. For example, when numberOfTriggeringCells is configured, the event satisfying entering condition may not be triggered. It is not accurate to describe it in terms of entry conditions. Therefore, it is proposed to add the condition as "the measurement report trigger condition of the event has been met".

Proposal 1: it is proposed to add the condition as "the measurement report trigger condition of the event has been met".

- Conclusions

Based on the discussion in the previous sections we propose the following:

Proposal 1: it is proposed to describe the procedure’s condition as "the measurement report trigger condition of the event has been met".

- References

- R2-24xxxxx Draft Report of 3GPP TSG RAN WG2 meeting #124, Chicago, USA

- R2-2312245    Report of [POST123bis][025][UAV] 38.331 Running CR (Qualcomm) including remaining open issues    Qualcomm Incorporated

- TS 38.331-i00 V18.0.0, Radio Resource Control (RRC) protocol specification

Annex TP for 38.331

Start of changes

5.5.4	Measurement report triggering

5.5.4.1	General

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical:

3>	if the corresponding measObject concerns NR:

4>	if the corresponding reportConfig includes measRSSI-ReportConfig:

5>	consider the resource indicated by the rmtc-Config on the associated frequency to be applicable;

4>	if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	if the eventA3 or eventA5 or eventA3H1 or eventA3H2 or eventA5H1 or eventA5H2 is configured in the corresponding reportConfig:

5>	if a serving cell is associated with a measObjectNR and neighbours are associated with another measObjectNR, consider any serving cell associated with the other measObjectNR to be a neighbouring cell as well;

4>	if the eventX2 is configured in the corresponding reportConfig:

5>	consider only the serving L2 U2N Relay UE to be applicable;

4>	if corresponding reportConfig includes reportType set to periodical; or

4>	for measurement events other than eventA1, eventA2, eventD1, eventX2, eventH1 or eventH2:

5>	if useAllowedCellList is set to true:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is included in the allowedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

4>	if the eventH1 or eventH2 is configured in the corresponding reportConfig:

5>	for all the events with the same eventID for which simulMultiTriggerSingleMeasReport is set to true and the measurement report triggering  condition applicable for the event has been satisfied:

6>	consider only the event for which the difference between the altitude of the UE and the corresponding altitude threshold is the smallest to be applicable;

4>	else if the eventA3H1 or eventA3H2 or eventA4H1 or eventA4H2 or eventA5H1 or eventA5H2 is configured in the corresponding reportConfig:

5>	for all the events with the same eventID associated with the same measObjectNR for which simulMultiTriggerSingleMeasReport is set to true and the measurement report triggering conditions applicable for the event has been satisfied:

6>	consider only the eventfor which the difference between the altitude of the UE and the corresponding altitude threshold is the smallest to be applicable;

End of changes