3GPP TSG-RAN WG2 Meeting #125	R2-2401306

Athens, Greece, 26 February – 01 March 2024

Agenda item:	7.20.2

Source:	Nokia, Nokia Shanghai Bell

Title:	CG-SDT TAT and 2TA

WID/SID:	NR_MIMO_evo_DL_UL-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This TDoc discusses if the 2TA framework should account the CG-SDT TAT when UE is resumed from CG-SDT procedure to CONNECTED mode by the network.

# 2	Discussion

In the previous meeting, the following was agreed for 2-step RA:

TAG indication is not included in successRAR

This agreement effectively means that either it is specified the UE autonomously releases the 2 TAG configuration for the PCell upon initiating RRC resume procedure or the NW releases the 2 TAG configuration from the PCell upon releasing the UE connection. Currently, the specification does not support resuming the RRC connection with 2TAG configuration since if the UE used 2-step RACH, there would not be any indication to which TAG the TAC in the successRAR applies.

Observation: Specification does not support resuming the RRC connection with 2 TAG configuration for PCell.

Since this is the case already, it seems that no handling of CG-SDT TAT needs to be considered either but the UE should only have one TAG configuration for the PCell upon the NW resumes the UE connection from CG-SDT procedure.

Proposal: No handling of CG-SDT TAT needs to be specified wrt. 2 TAG configuration upon resumption of UE connection to CONNECTED mode from CG-SDT procedure in INACTIVE mode.

# 3	Conclusion

Observation: Specification does not support resuming the RRC connection with 2 TAG configuration for PCell.

Proposal: No handling of CG-SDT TAT needs to be specified wrt. 2 TAG configuration upon resumption of UE connection to CONNECTED mode from CG-SDT procedure in INACTIVE mode.