3GPP TSG-RAN WG2 Meeting #125	R2-2401324

Athens, Greece, 26th Feb – 1st March 2024

Agenda Item:	7.2.8

Source:	Ericsson

Title:	Addressing SL cell reselection open issues

Document for:	Discussion, Decision

# 1	Introduction

This paper discusses the open issues of TS 38.304 specification and suggest way forward.

# 2	Discussion

## 2.1	Open Issues #1

| SL#02 | 4.5 | Editor's NOTE: FFS the support for SL positioning in limited service state. |
| --- | --- | --- |

In our view this should be same as legacy for the case of SL Positioning in limited service and do not see the need to differentiate.

From TS 38.304

Following exception to these definitions are applicable for UEs:

-	if a UE has an ongoing emergency call, all acceptable cells of that PLMN/SNPN are treated as suitable for the duration of the emergency call.

-	camped on a cell that belongs to a tracking area that is forbidden for regional provision of service; a cell that belongs to a tracking area that is forbidden for regional provision service (TS 23.122 [9], TS 24.501 [14]) is suitable but provides only limited service.

-	if the UE in RRC_IDLE fulfils the conditions to support NR sidelink communication/discovery or V2X sidelink communication in limited service state as specified in TS23.287 [16] clause 5.7, the UE may perform NR sidelink communication/discovery or V2X sidelink communication or NR Sidelink Positioning.

Limited service state for NR Sidelink Positioning is same as for NR sidelink communication/discovery or V2X sidelink communication.

## 2.2	Open Issues #2

| SL#01 | 8.2.1 | FFS the additional reselection process for selecting a cell on non-serving frequency for SL-PRS transmission. |
| --- | --- | --- |

In our view, there is no impact because of SL-PRS for section 8.2.1.

This is also valid for normal communication as there is no impact for DL-PRS for cell reselection procedure.

No special handling is specified for the additional reselection process for selecting a cell on non-serving frequency for SL-PRS transmission.

# Conclusion

Based on the discussion in the previous sections we make the following observations:

Proposal 1	Limited service state for NR Sidelink Positioning is same as for NR sidelink communication/discovery or V2X sidelink communication.

Proposal 2	No special handling is specified for the additional reselection process for selecting a cell on non-serving frequency for SL-PRS transmission.