3GPP TSG-RAN WG2 Meeting #125	R2-2401381

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | DraftCR | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage-2 corrections for Rel-18 mobility enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The stage-2 description of LTM has currently some errors in terminology, inconsistences and as well it it is sometimes hard to read and understand. This CR is an attempt to resolve it in order to maintain the excellent quality of 3GPP specifications also for this one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected name of heading of 9.2.3.5. Corrections in figure 9.2.3.5.2-1 on terminology to use “LTM candidate configuration” and “LTM candidate cell”. In the description of the signalling procedure for LTM in clause 9.2.3.5.2, the following corrections are made: - Corrected terminology to use “LTM candidate cell” - In step 4a early DL synchronization, added that UE may activate TCI states of LTM candidate cell. - In step 4b early UL synchronization, added a short description with similar detail level as the other steps. The old more detailed description is kept (unless we can find a more suitable place in this spec) but moved into separate paragraph before the next step to simplify reading. - In step 6 execution, added a few more details to align wth the aother steps and to provide better understanding of the procedure. - In step 7, random access, corrected clause number in the reference to 38.321. - Corrected reference to figure 9.2.3.5.2-1. In Annex G, the following corrections are made: - Corrected terminoogy to “Mobility interruption time” and “L1/L2 Triggered Mobility”. - Clarified in a few places that the latency is for LTM (and not for mobility in general) - Updated the references for where more details of the latency components can be found. - Updated figures G-2 and G-3 with triggers for the DL and UL early sync. - In figure G-2, the “0ms under certain conditions” is not informative as the conditions are anyway not provided here. It is replaced with “T/F tracking” to align with the other figures and the reference to 38.133 should be used for more details about all the components. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above errors would remain the stage-2 specification which would make it harder to read and understand. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of changes

#### 9.2.3.5	L1/L2 Triggered Mobility

##### 9.2.3.5.1	General

LTM is a procedure in which a gNB receives L1 measurement report(s) from a UE, and on their basis the gNB changes UE serving cell by a cell switch command signalled via a MAC CE. The cell switch command indicates an LTM candidate configuration that the gNB previously prepared and provided to the UE through RRC signalling. Then the UE switches to the target configuration according to the cell switch command. The LTM procedure can be used to reduce the mobility latency as described in Annex G.

When configured by the network, it is possible to activate TCI states of one or multiple cells that are different from the current serving cell. For instance, the TCI states of the LTM candidate cells can be activated in advance before any of those cells become the serving cell. This allows the UE to be DL synchronized with those cells, thereby facilitating a faster cell switch to one of those cells when cell switch is triggered.

When configured by the network, it is possible to initiate UL TA acquisition (called early TA) procedure of one or multiple cells that are different from the current serving cells. If the cell has the same NTA as the current serving cells or NTA=0, early TA acquisition procedure is not required. The network may request the UE to perform early TA acquisition of a candidate cell before a cell switch. The early TA acquisition procedure is triggered by PDCCH order as specified in clause 9.2.6 or realized through UE-based TA measurement as configured by RRC. In the former case, the gNB to which the candidate cell belongs calculates the TA value and sends it to the gNB to which the serving cell belongs. The serving cell sends the TA value in the LTM cell switch command MAC CE when triggering LTM cell switch. In the latter case, the UE performs TA measurement for the candidate cells after being configured by RRC but the exact time the UE performs TA measurement is up to UE implementation. The UE applies the TA value measured by itself and performs RACH-less LTM upon receiving the cell switch command. The network may also send a TA value in the LTM cell switch command MAC CE without early TA acquisition.

Depending on the availability of a valid TA value, the UE performs either a RACH-less LTM or RACH-based LTM cell switch. If the TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network. In the case where UE-based TA measurement is configured, but no TA value is provided in the cell switch command, the UE applies the TA value by itself if available. Meanwhile, the UE performs RACH-less LTM cell switch upon receiving the cell switch command. If no valid TA value is available, the UE performs RACH-based LTM cell switch.

Regardless of whether the UE is configured for UE-based TA measurement for a certain candidate cell, it will still follow the PDCCH order, which includes requesting a random access procedure towards the candidate cells. This also applies to the candidate cells for which the UE is capable of deriving TA values by itself. Additionally, regardless of whether the UE has already performed a random access procedure towards the candidate cells, it will still follow the UE-based measurement configuration if configured by the network.

For RACH-less LTM, the UE accesses the target cell using either a configured grant or a dynamic grant. The configured grant is provided in the LTM candidate configuration, and the UE selects the configured grant occasion associated with the beam indicated in the cell switch command. Upon initiation of LTM cell switch to the target cell, the UE starts to monitor PDCCH on the target cell for dynamic scheduling. Before RACH-less LTM procedure completion, the UE shall not trigger random access procedure if it does not have a valid PUCCH resource for triggered SRs.

The following principles apply to LTM:

-	Security key is maintained upon an LTM cell switch;

-	Subsequent LTM is supported.

LTM supports both intra-gNB-DU and intra-gNB-CU inter-gNB-DU mobility. LTM supports both intra-frequency and inter-frequency mobility, including mobility to inter-frequency cell that is not a current serving cell. LTM is supported only for licensed spectrum. The following scenarios are supported:

-	PCell change in non-CA scenario and non-DC scenario;

-	PCell and SCell(s) change in CA scenario;

-	Dual connectivity scenario, PCell and MCG SCell(s) change and intra-SN PSCell and SCG SCell(s) change without MN involvement. LTM for simultaneous PCell and PSCell change is not supported.

While the UE has stored LTM candidate configurations the UE can also execute any L3 handover command sent by the network.

##### 9.2.3.5.2	C-Plane Handling

Cell switch command is conveyed in a MAC CE, which contains the necessary information to perform the LTM cell switch.

The overall procedure for LTM is shown in Figure 9.2.3.5.2-1 below. Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other LTM candidate configurations after each LTM cell switch completion. The general procedure over the air interface is applicable to SCG LTM. Further details of SCG LTM can be found in TS 37.340 [21].

Figure 9.2.3.5.2-1. Signalling procedure for LTM

The procedure for LTM is as follows:

1.	The UE sends a MeasurementReport message to the gNB. The gNB decides to configure LTM and initiates LTM preparation.

2.	The gNB transmits an RRCReconfiguration message to the UE including the LTM candidate configurations.

3.	The UE stores the LTM candidate configurations and transmits an RRCReconfigurationComplete message to the gNB.

4a.	The UE performs DL synchronization with the LTM candidate cell(s) before receiving the cell switch command. The UE may activate and deactivate TCI states of LTM candidate cell(s), as triggered by the gNB.

4b.	The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as configured by the gNB.

When UE-based TA measurement is configured, UE acquires the TA value(s) of the candidate cell(s) by measurement. UE performs early TA acquisition with the candidate cell(s) as requested by the network before receiving the cell switch command as specified in clause 9.2.6. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition and the TA value of the candidate cell is indicated in the cell switch command. The UE does not maintain the TA timer for the candidate cell and relies on network implementation to guarantee the TA validity.

5.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the gNB. L1 measurement should be performed as long as RRC reconfiguration (step 2) is applicable.

6.	The gNB decides to execute cell switch to a target cell and transmits an LTM Cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

7.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

8.	The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

The steps 4-8 can be performed multiple times for subsequent LTM using the LTM candidate configuration(s) provided in step 2.

The procedure over the air interface described in Figure 9.2.3.5.2-1 is applicable to both intra-gNB-DU LTM and inter-gNB-DU LTM. The overall LTM procedures over F1-C interface are captured in TS 38.401[4].

##### 9.2.3.5.3	U-Plane Handling

After receiving an LTM cell switch command MAC CE, the UE performs MAC reset. Whether the UE performs RLC re-establishment and PDCP data recovery during cell switch is explicitly controlled by the network through RRC signalling.

Next change

Annex G (informative):
Components of Mobility Latency

Mobility interruption time for L1/L2 Triggered Mobility (LTM) is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

For Rel-18, RAN2 assumed the latency of the mobility procedure for LTM is characterized by the terms illustrated in Figure G-1.

Figure G-1: Components of Mobility Latency for LTM

Each component of mobility latency is described in the table below, the values of which are specified in clause 6.3 of TS 38.133[13]. The value of TRRC is specified in clause 12 of TS 38.331 [12].

Table G-1: Components of Mobility Latency for LTM

| Component | Meaning |
| --- | --- |
| TRRC | Processing time for RRCReconfiguration carrying candidate configurations |
| Tcmd | Time for processing L1/L2-command (HARQ and parsing) |
| TLTM-RRC-processing | Early ASN.1 decoding and validity/compliance check |
| TLTM-Processing | UE processing including applying target cell parameters and L1/L2 change |
| Tfirst-RS | Time for fine tracking and acquiring full timing information |
| TRS-proc | Time for SSB processing |
| TLTM-IU | Interruption uncertainty in acquiring the first UL transmission |
| TRAR | Time for RAR delay |
| Tfirst-data | Time for UE performs the first DL/UL reception/ transmission on the indicated beam of the target cell, after RAR |

For Rel-18, RAN2 assumed that the latency of the RACH-based and RACH-less LTM procedure is characterized by the terms illustrated in Figure G-2 and Figure G-3. In particular, Figure G-2 shows the LTM procedure where UE activates a TCI state of an LTM candidate cell before performing an LTM cell switch which involves a random access procedure. Further, Figure G-3 shows the LTM procedure when the UE performs the UL and DL synchronization before performing an LTM cell switch which does not involve any random access. In both figures, it it highlighted how the overall mobility latency of LTM can be reduced by the early synchronization procedure. Further details on the UE processing timer of the delay components are specified in clause 6.3 of TS 38.133[13].

Figure G-2: Mobility Latency for RACH-based LTM

Figure G-3: Mobility Latency for RACH-less LTM

End of changes