3GPP TSG-RAN WG2 Meeting #125	R2-2401405

# Athens, Greece, 26 February – 1 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | - | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | R18 XR UTO-UCI correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are some outstanding issues for XR related to the interaction between UTO-UCI and uplink skipping. The RRC configuration possibilities for CGs are such that either R15 skipping is active, or R16 skipping is active – there is no possibility to not have CG skipping. With R15 skipping the UE skip regardless whether there is UCI to transmit or not, and in the R16 enhanced UL skipping the UE shall not skip if the transmission has UCI. The enhanced skipping was introduced to avoid the need for a gNB to decode both PUCCH and PUSCH when UCI is expected as NW do not know when the UE will skip a transmission. 1) The MAC spec is not clear on the interaction between the R16 enhanced UL skipping and inclusion of UTO-UCI in the transmissions. That is, is the presence of UTO-UCI a reason for the UE to not skip an UL transmission, even there is no data to transmit? The case that the UE skips a transmission when it have indicated to use a TO in an earlier UTO-UCI is unexpected, but can happen, for example if the UE has discarded some older PDUs or there is no data arrival after a period of unused TOs (where obviously no UTO-UCI is sent). At least two behaviours are possible a) The UE never skip UL when there is UTO-UCI. b) When deciding to skip, the UE ignore UTO-UCI. These will beneficial in different scenarios, a) may be beneficial to inform the gNB of new situation in the UE as the UE sends an updated UTO-UCI where it previously was expecting to send data, b) may be beneficial in some scenarios (for example in congestion where there is lots of discarding happening or when data arrivals have large variations) as it allows for energy saving for the UEs and decreases the interference in the system. We believe behavior b) is better, and it is also in line with how CG-UCI affects skipping (that is the UTO-UCI is present only if there is a CG transmission, and if the CG PUSCH is not transmitted – there will be no PUCCH transmission to convey the UTO-UCI contrary to for example HARQ feeback UCI). This is the same situation like for CG-UCI that was introduced for NR-Unlicensed. 2) When enhanced skipping is active and UTO-UCI is configured, it needs to be clarified how the UE selects to indicate the UTO. If gNB receives UTO-UCI for a TO indicating it as unused (meaning there will be no PUSCH transmission in that TO regardless of other UCI) and there is UCI (for example HARQ feedback), there may not be suficcent time for the gNB to decode this UTO-UCI before gNB needs prepare decoding the PUCCH or PUSCH for that TO. This means the gNB must decode both PUCCH and PUSCH for some TOs, and the UE will not save much energy by sending the PUCCH instead of sending PUSCH. To avoid this, the UE shall not indicate TOs as unused if the UE knows that there is other UCI for that TO, that would result in a PUCCH transmission if PUSCH is not transmitted, when determining the UTO-UCI to signal. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) when enhaced UL skipping is activated, the UE shall ignore the UTO-UCI when deciding to skip a transmission or not. 2) when enhaced UL skipping is activated and the UE determine the UTO-UCI for future TOs, the UE shall take into account known other UCI (that would result in an PUCCH transimssion if the PUSCH is not transmitted) and not indicate a TO with other UCI present as unused (unless the TO have been indicated as unused earlier). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1.3, 5.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes (when eLCID is not used) or 10 bytes (when eLCID is used) while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI, besides UCI that cannot be carried on PUCCH, to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for SL LBT failure according to clause 5.31.2;

-	MAC CE for Timing Advance Report;

-	MAC CE for Delay Status Report;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for (Extended) BSR, with exception of BSR included for padding;

-	MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH;

-	MAC CE for Positioning Measurement Gap Activation/Deactivation Request;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Case-6 Timing Request;

-	MAC CE for (Extended) Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among MAC CEs of same priority is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over NR sidelink transmission.

START OF CHANGE

### 5.8.2	Uplink

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC for a Serving Cell per BWP. Multiple configurations can be active simultaneously in the same BWP. For Type 2, activation and deactivation are independent among the Serving Cells. For the same BWP, the MAC entity can be configured with both Type 1 and Type 2.

A multi-PUSCH configured grant has multiple consecutive configured uplink grants within a periodicity. Both Type 1 and Type 2 can be configured for a multi-PUSCH configured grant by RRC.

Only configured grant Type 1 can be configured for CG-SDT. CG-SDT can only be configured on initial BWP.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	cg-SDT-CS-RNTI: CS-RNTI for CG-SDT retransmission;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	rach-less-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for RACH-less handover;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN = timeReferenceSFN in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]) or startSymbol (i.e. S in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer;

-	timeReferenceSFN: SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration;

-	timeReferenceH-SFN: H-SFN used for determination of the offset of a resource in time domain. The UE uses the closest H-SFN with the indicated number preceding the reception of the configured grant configuration.

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer.

RRC configures the following parameter when retransmissions on configured uplink grant is configured:

-	cg-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process when the UE shall not autonomously retransmit that HARQ process;

-	cg-SDT-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process of the initial CG-SDT transmission with CCCH message when the UE shall not autonomously retransmit the HARQ process;

-	cg-LTM-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process of the initial transmission at LTM cell switch when the UE shall not autonomously initiate a retransmission on the HARQ process;

-	cg-RACH-less-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process of the initial transmission of RACH-less handover when the UE shall not autonomously retransmit the HARQ process.

RRC configures the following additional parameter for a multi-PUSCH configured grant:

-	nrofSlotsInCG-Period: the number of configured uplink grants in a periodicity of a multi-PUSCH configured grant.

For a configured uplink grant, the MAC entity shall:

1>	if the configured uplink grant is associated with a multi-PUSCH configured grant:

2>	if configured uplink grant has not been indicated to the lower layers as to be unused for PUSCH transmission; and

2>	if the configured uplink grant meets the validity conditions specified in the clause 6.1 in TS 38.214 [7]:

3>	consider the configured uplink grant available for use;

1>	else if the configured uplink grant has not been indicated to lower layers as to be unused for PUSCH transmission:

2>	consider the configured uplink grant available for use.

The MAC entity determines if a configured uplink grant is going to be used for PUSCH transmission or not by considering the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources. When the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true, a PUSCH transmission for a configured uplink grant previously not indicated as unused shall be indicated as used by UTO-UCI if the UE knows there is UCI, that will be carried on PUCCH if there is no PUSCH transmission, to be multiplexed on the PUSCH transmission as specified in TS 38.213 [6]. Upon this determination, the MAC entity sends an indication to the lower layers regarding this decision, for use in the procedure for reporting UTO-UCI (as specified in clause 9.3 in TS 38.213 [6]).

Upon configuration of a configured grant Type 1 for a BWP of a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated BWP of the Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset, timeReferenceSFN, and S (derived from SLIV or provided by startSymbol as specified in TS 38.214 [7]), and to reoccur with periodicity.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

-	if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is not configured:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	(timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity)
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

-	else:

[(H-SFN × numberOfSFNperH-SFN + SFN) × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	((timeReferenceH-SFN × numberOfSFNperH-SFN + timeReferenceSFN)
	× numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity)
	modulo (1024 × 1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

For a multi-PUSCH configured grant Type 1, the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within a periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs.

For an uplink grant configured for configured grant Type 1 for CG-SDT on the selected uplink carrier as in clause 5.27, when CG-SDT is triggered and not terminated, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if, after initial transmission for CG-SDT with CCCH message has been performed according to clause 5.4.1, PDCCH addressed to the MAC entity's C-RNTI has not been received:

2>	if the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for initial transmission for CG-SDT with CCCH message (i.e., retransmission of initial transmission of CG-SDT):

3>	select this SSB;

3>	indicate the SSB index corresponding to the configured uplink grant to the lower layer;

3>	consider this configured uplink grant as valid.

1>	else if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

2>	if at least one SSB corresponding to the configured uplink grant with SS-RSRP above the cg-SDT-RSRP-ThresholdSSB is available:

3>	if this is the initial transmission of CG-SDT with CCCH message after the CG-SDT procedure is initiated as in clause 5.27 (i.e., initial transmission for CG-SDT):

4>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant.

3>	else if PDCCH addressed to C-RNTI has been received after the initial transmission of CG-SDT with CCCH message (i.e., subsequent new transmission for CG-SDT):

4>	if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is above cg-SDT-RSRP-ThresholdSSB and this SSB is associated with this configured uplink grant:

5>	select this SSB.

4>	else if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is not above cg-SDT-RSRP-ThresholdSSB:

5>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant.

3>	if SSB is selected above:

4>	indicate the SSB index to the lower layer;

4>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid.

2>	if PDCCH addressed to C-RNTI after the initial transmission of the CG-SDT with CCCH message has been received:

3>	if there is data available for transmission for at least one RB configured for SDT:

4>	initiate Random Access procedure in clause 5.1.

NOTE 1:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

For an uplink grant configured for configured grant Type 1 for LTM cell switch, when there is an on-going RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE, as specified in clause 5.18.35:

2>	select the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE.

2>	indicate the SSB index to the lower layer;

2>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid.

For an uplink grant configured for configured grant Type 1 for RACH-less handover, when RACH-less handover is triggered and not terminated, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if, after the initial transmission of RACH-less handover has been performed according to clause 5.4.1 and 5.33, PDCCH addressed to the MAC entity's C-RNTI has not been received:

2>	if the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for the initial transmission of RACH-less handover (i.e., retransmission of initial transmission of RACH-less handover):

3>	select this SSB;

3>	indicate the SSB index corresponding to the configured uplink grant to the lower layer;

3>	consider this configured uplink grant as valid.

1>	else if at least one SSB corresponding to the configured uplink grant with SS-RSRP above rach-less-RSRP-ThresholdSSB is available:

2>	select an SSB with SS-RSRP above rach-less-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant;

2>	indicate the selected SSB index to the lower layer;

2>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid;

2>	initiate Random Access procedure in clause 5.1.

NOTE 1A:	When the UE determines if there is an SSB with SS-RSRP above rach-less-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity]
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

For a multi-PUSCH configured grant Type 2, the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within the same periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs.

If cg-nrofPUSCH-InSlot or cg-nrofSlots is configured for a configured grant Type 1 or Type 2, the MAC entity shall consider the uplink grants occur in those additional PUSCH allocations as specified in clause 6.1.2.3 of TS 38.214 [7].

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the occurrences of configured uplink grants.

When the configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if at least one configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	if, in this MAC entity, at least one configured uplink grant is configured by configuredGrantConfigToAddModList:

3>	instruct the Multiplexing and Assembly procedure to generate a Multiple Entry Configured Grant Confirmation MAC CE as defined in clause 6.1.3.31.

2>	else:

3>	instruct the Multiplexing and Assembly procedure to generate a Configured Grant Confirmation MAC CE as defined in clause 6.1.3.7.

2>	cancel all triggered configured uplink grant confirmation(s).

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant(s) immediately after first transmission of Configured Grant Confirmation MAC CE or Multiple Entry Configured Grant Confirmation MAC CE which confirms the configured uplink grant deactivation.

Retransmissions use:

-	repetition of configured uplink grants; or

-	received uplink grants addressed to CS-RNTI; or

-	configured uplink grants with cg-RetransmissionTimer, cg-LTM-RetransmissionTimer, cg-RACH-less-RetransmissionTimer or cg-SDT-RetransmissionTimer configured.

END OF CHANGE