3GPP TSG RAN WG2#125	R2-2401803

Athens, Greece, 26th February - 1st March 2024

3GPP TSG-RAN WG4 Meeting #110	R4-2403363

Athens, Greece, 26 February – 01 March, 2024

Title:	Updates on measurement report mapping for Positioning Enhancements WI

Response to:

Release:	Release 18

Work Item:	NR_pos_enh2

Source:	RAN4

To:	RAN2, RAN3

Cc:	RAN1

Contact Person:

#### Name:	Li Zhang

E-mail Address:	zhangli164@huawei.com

Attachments: -

1. Overall Description:

A. Timing related measurements

In RAN4#109, RAN4 provided the report mapping for timing related measurements with k = {-3, -4, -5, -6} in R4-2321545. In RAN4#110, some errors are spotted in the report mapping. The corrections are shown in red.

# Differential RSTD measurement report mapping For k = -3 Minimum reported quantity value: DIFFRSTD_00000 Maximum reported quantity value: DIFFRSTD_65528 Total number of reported quantity values: 65529 For k = -4 Minimum reported quantity value: DIFFRSTD_000000 Maximum reported quantity value: DIFFRSTD_131056 Total number of reported quantity values: 131057 For k = -5 Minimum reported quantity value: DIFFRSTD _000000 Maximum reported quantity value: DIFFRSTD _262112 Total number of reported quantity values: 262113 For k = -6 Minimum reported quantity value: DIFFRSTD _000000000 Maximum reported quantity value: DIFFRSTD _524224 Total number of reported quantity values: 524225 # Differential UE Rx-Tx measurement report mapping For k = -3 Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_00000 Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_65528 Total number of reported quantity values: 65529 For k = -4 Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_000000 Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_131056 Total number of reported quantity values: 131057 For k = -5 Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_000000 Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_262112 Total number of reported quantity values: 262113 For k = -6 Minimum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_000000 Maximum reported quantity value: DIFF_RX-TX_TIME_DIFFERENCE_524224 Total number of reported quantity values: 524225 # Additional path report mapping for UE Rx-Tx measurement For k = -3 Minimum reported quantity value: path_000000 Maximum reported quantity value: path_130801 Total number of reported quantity values: 130802 # Additional path report mapping for UL-RTOA measurement For k = -3 Minimum reported quantity value: path_000000 Maximum reported quantity value: path_130801 Total number of reported quantity values: 130802 # Additional path report mapping for gNB Rx-Tx measurement For k = -3 Minimum reported quantity value: path_000000 Maximum reported quantity value: path_130801 Total number of reported quantity values: 130802

B. Carrier phase measurements

In RAN4#110, RAN4 agreed on the report mapping tables for DL-RSCPD, DL-RSCP and UL-RSCP.

# DL-RSCPD measurement report mapping table is defined as: Reported value; Measured quantity value (DL-RSCPD); Unit. DL-RSCPD_0000; -180 ≤ DL-RSCPD < -179.9; Degree. DL-RSCPD_0001; -179.9 ≤ DL-RSCPD < -179.8; Degree. DL-RSCPD_0002; -179.8 ≤ DL-RSCPD < -179.7; Degree. …; …; …. DL-RSCPD_1798; -0.2 ≤ DL-RSCPD < -0.1; Degree. DL-RSCPD_1799; -0.1 ≤ DL-RSCPD < 0; Degree. DL-RSCPD_1800; 0 ≤ DL-RSCPD < 0.1; Degree. DL-RSCPD_1801; 0.1 ≤ DL-RSCPD < 0.2; Degree. DL-RSCPD_1802; 0.2 ≤ DL-RSCPD < 0.3; Degree. …; …; …. DL-RSCPD_3598; 179.8 ≤ DL-RSCPD < 179.9; Degree. DL-RSCPD_3599; 179.9 ≤ DL-RSCPD < 180; Degree. # DL-RSCP measurement report mapping table is defined as: Reported value; Measured quantity value (DL-RSCP); Unit. DL-RSCP_0000; 0 ≤ DL-RSCP < 0.1; degree. DL-RSCP_0001; 0.1 ≤ DL-RSCP < 0.2; degree. DL-RSCP_0002; 0.2 ≤ DL-RSCP < 0.3; degree. …; …; …. DL-RSCP_3598; 359.8 ≤ DL-RSCP < 359.9; degree. DL-RSCP_3599; 359.9 ≤ DL-RSCP < 360; degree. #UL-RSCP measurement report mapping table is defined as: Reported value; Measured quantity value (UL-RSCP); Unit. UL-RSCP_0000; 0 ≤ UL-RSCP < 0.1; degree. UL-RSCP_0001; 0.1 ≤ UL-RSCP < 0.2; degree. UL-RSCP_0002; 0.2 ≤ UL-RSCP < 0.3; degree. …; …; …. UL-RSCP_3598; 359.8 ≤ UL-RSCP < 359.9; degree. UL-RSCP_3599; 359.9 ≤ UL-RSCP < 360; degree.

RAN4 respectfully asks RAN2 and RAN3 to take the above information into account in defining signalling for bandwidth aggregation and carrier phase measurement.

2. Actions:

To RAN2 and RAN3:

RAN4 respectfully asks RAN2 and RAN3 to take the above information into account in defining signalling for bandwidth aggregation and carrier phase measurement.

3. Date of Next TSG-RAN4 Meetings:

RAN WG4 Meeting #110-bis	Apr 15 – Apr 19, 2024	Changsha, China

RAN WG4 Meeting #111	May 20 – May 24, 2024	Fukuoka, Japan