3GPP TSG-RAN WG2 Meeting #125bis	R2-2403733

Changsha, China, Apr 15th – 19th, 2024

Agenda item:	9.3

Source: 	Session Chair (ZTE Corporation)

Title:	Report from Break-out session on NR-NTN and IoT-NTN

Document for:	Approval

Organizational

- All organization emails and notes will be shared over the following email discussion throughout the meeting:

[AT125bis][300] Organizational – NR-NTN and IoT-NTN session

Scope:

    - Share plans for the meeting and list of ongoing email discussions for the sessions related to NR-NTN and IoT-NTN

    - Share meetings notes and agreements for review and endorsement

Schedule/Plan

|  | Main room | Brk 1 room | Brk 2 room | Brk 3 room |
| --- | --- | --- | --- | --- |
| Monday April 15th |  |  |  |  |
| 09:00 – 10:30 | [1], [2], [3], [7.0] R18 common (Diana) [7.0.1][7.0.2] [7.0.3] ASN.1 Review common [7.0.5] ----- Break out of ASN.1 Review [7.0.4] @NR151617 UP (Diana) | Breakout to start after common session including ASN.1 review --- MUSIM (Erlin) (if ASN.1 common session ends early) 7.17.1 7.17.2 | Breakout to start after common session ASN.1 Review: --- NR18 Positioning (Nathan) [7.2.1] RIL and open issue lists [7.2.4] LPP corrections (as time permits) |  |
| 11:00 – 13:00 |  |  |  |  |
| 14:30 – 16:30 | NR18 URLLC (Diana) NR18 Network Energy Saving (Diana) | @14:30-15:30 Rel-18 MUSIM (Erlin) 7.17.2 (cont.) @15:30 NR18 MIMO evo 7.20.1 7.20.2 7.20.3 IDC (Yi) (email discussion only) NCR(Sasha) (email discussion only) | NRLTE1516 V2X/SL (Kyeongin) NR17 SL (Kyeongin) NR18 SL (if time allows) |  |
| 17:00 – 19:00 | Rel-18 UAV (1hr) NR18 TEI (Diana) SDT, including MT-SDT and related TEI18 | NR18 fCovEnh (Eswar) 7.21.1 Organizational 7.21.2 CP - Aim to treat all CP tdocs and RILs 7.21.3 UP - RO mask issue - Open as many UP docs as possible and determine if any offline(s) are needed until the CB session NR18 Mobile IAB (Johan) | NR18 SL (Kyeongin) |  |
| Tuesday April 16th |  |  |  |  |
| 08:30 – 10:30 | NR18 feMob (Johan) | NR18 eQoE (Dawid) 7.14.1: LSin, RIL resolutions and rapp CR endorsement 7.14.2: RIL issues 7.14.3: Other corrections @09:00 NR18 MBS (Dawid): 7.11.1: RIL resolutions and rapp CR endorsement 7.11.2: High priority ToDo RILs 7.11.3: Other corrections TEI18 MBS: 7.24.2.2: MBS with eDRX/MICO, MBS and (e)RedCap If time allows: NR18 MBS: 7.11.2: Low priority ToDo RILs and non-RIL issues | NR18 SL Relay (Nathan) As much as possible of: [7.9.3] RRC (from open issues list/email report) [7.9.4] SRAP [7.9.6] RLC/PDCP [7.9.7] UE capabilities [7.9.8] Idle mode [7.9.2] Stage 2 (if time) |  |
| 11:00 – 13:00 | NR18 feMob con’t (Johan) @12:00 NR18 XR (Diana) | NR18 eRedCap (Mattias) [7.19.1] [7.19.2] [7.19.3] | NRLTE1516 Pos (Nathan) [5.3] NR17 Positioning and SL Relay (Nathan) [6.4] [6.2] NR18 Pos (Nathan) [7.2.1] LSs [7.2.4] (continued from Monday if not sent offline) [7.2.3] SLPP corrections (as time permits) |  |
| 14:30 -16:30 | NR18 XR (Diana) | R18 NTN IoT (Sergio) [7.6.1] [7.6.2] [7.6.3] | NR18 Pos (Nathan) [7.2.3] SLPP corrections (continued from morning) [7.2.5] RRC corrections [7.2.6] MAC corrections [7.2.7] UE capabilities (as time permits) [7.2.2] Stage 2 (as time permits) |  |
| 17:00– 19:00 | Rel-19 Ambient IoT [2] (Diana) | NR18 NR NTN enh (Sergio) [7.7.1] [7.7.2] [7.7.3] [7.7.4] [7.7.5] | EUTRA&NR151617 (Mattias) As far as possible with: [6.1.3.2] <- Note starting with this AI to treat intraband EN-DC early. [4.1] [4.1.1] [5.1.1] [5.1.1.1] [5.1.3.1] [5.1.3.2] [6.1] [6.1.1] [6.1.3.1] |  |
| Wednesday April 17th |  |  |  |  |
| 08:30 – 10:30 | NR19 Network Energy Saving [1] (Kyeongin) | R18 NTN IoT CB (Sergio) [7.6.4] [7.6.5] R19 IoT-NTN [0.5] (Sergio) [8.9.1] [8.9.2] [8.9.3] | NR18 SONMDT (Mattias) [7.13.1] [7.13.2] [7.13.3] NR19 SONMDT [0.5] (Mattias) [8.10.1] [8.10.2] [8.10.4] |  |
| 11:00 – 13:00 | NR18 Other Diana | NR19 XR [1] (Dawid) | EUTRA&NR151617 (Mattias) Continue from Tuesday maintenance session. |  |
| 14:30 – 16:30 | AI/ML Mobility [1.5] (Diana) | @14:30-15:15 Rel-18 MUSIM /MIMO CBs Details TBD after Monday sessions @15:15-16:30 Rel-19 LP-WUS (Erlin) 8.4.1 8.4.2 8.4.3 | NR18 Pos (Nathan) Remaining agenda items after Tuesday sessionsTEI/POS (Nathan) Positioning and relay documents from: [7.24.2.2] TEI RAN2 [7.24.1] TEI other groups [7.25.3] Other |  |
| 17:00 – 19:00 | AI/ML PHY [1] (Diana) | NR19 feMob [1] (Kyeongin) | Positioning or SL offlines for Rel-18 |  |
| Thursday April 18th |  |  |  |  |
| 08:30 – 10:30 | CB Eswar [8:30 – 9:30] CB Diana Pani XR/NES | R18 NTN IoT CB (Sergio) - report of [303] - other issues marked CB Thursday - TBD NR18 NR NTN CB (Sergio) - report of [305] and [306] - other issues marked CB Thursday - 7.7.4 - 7.7.5 | CB Kyeongin Comebacks SL |  |
| 11:00 – 13:00 | Rel-19 Ambient IoT [2] (Diana) | Rel-19 NTN NR [1] (Sergio) [8.8.1] [8.8.2] [8.8.4] [8.8.5] [8.8.6] | CB Dawid: - QoE - MBS - MBS TEI18 |  |
| 14:30 – 16:30 | NR18 Other (Diana) TEI18 | CB Johan - mIAB - feMob | CB Nathan |  |
| 17:00 – 19:00 | CB Diana [R18 NES] [R18 UAV] 18:00-19:00 AI/ML Mobilitly (Diana) | TBD Kyeongin/Johan/Erlin? | CB Nathan |  |
| Friday April 19th |  |  |  |  |
| 08:30 – 10:30 | CB Diana ASN.1 Review common session TEI 18 CBs NR Others CBs | CB Erlin/Kyeongin TDB | CB Mattias |  |
| 11:00 – 13:00 | CB Diana | CB Sergio TBD | CB Nathan |  |
| 14:30 – 16:00 |  |  |  |  |
| 16:00 – 17:00 |  |  |  |  |

List and details of [AT125bis] offline discussions

NOTE:  No offline email discussions will be kicked off before Monday Apr 15th, 09:00 local time

[AT125bis][301][IoT NTN Enh] Preliminary RILs checking (Huawei)

Scope: Allow checking the PropAgree and PropReject RILs in R2-2403221, if needed, before the online discussion. No technical discussion is expected to happen via email: disagreeing companies are invited to discuss F2F with the WI RRC rapporteur before the online session.

Intended outcome: Updated RILs list (if needed)

Deadline for rapporteur's summary in R2-2403761: Tuesday 2024-04-16 13:00

[AT125bis][302][NR NTN Enh] Preliminary RILs checking (Ericsson)

Scope: Allow checking the PropAgree and PropReject RILs in R2-2403633, if needed, before the online discussion. No technical discussion is expected to happen via email: disagreeing companies are invited to discuss F2F with the WI RRC rapporteur before the online session.

Intended outcome: Updated RILs list (if needed)

Deadline for rapporteur's summary in R2-2403762: Tuesday 2024-04-16 13:00

[AT125bis][303][IoT NTN Enh] Open issues on GNSS enhancements (Nokia)

Scope: Discuss the proposals in R2-2403480 (and possible counter-proposals from other companies on the same issues)

Intended outcome: Report of the offline discussion

Deadline for rapporteur's summary (in R2-2403764):  Wednesday 2024-04-17 22:00

[AT125bis][304][NR NTN Enh] LS to RAN4 on reference point (Apple)

Scope: Draft an LS to RAN4 on on reference point for SSB-TimeOffset

Intended outcome: Agreeable LS

Deadline for draft LS (in R2-2403765):  Friday 2024-04-19 08:00

[AT125bis][305][NR NTN Enh] PDCCH order during satellite switch (Samsung)

Scope: Discuss the proposals p3 and p4 in R2-2402800

Intended outcome: Report of the offline discussion

Deadline for rapporteur's summary (in R2-2403766):  Wednesday 2024-04-17 22:00 (if F2F offline is possible on Wednesday, otherwise Friday 2024-04-19 08:00)

[AT125bis][306][NR NTN Enh] Corrections on (cond)EventD2 (Ericsson)

Scope: Discuss the proposal in R2-2402882 (and corresponding ones from other companies on the same issue)

Intended outcome: Report of the offline discussion

Deadline for rapporteur's summary (in R2-2403767):  Wednesday 2024-04-17 22:00 (if F2F offline is possible on Wednesday, otherwise Friday 2024-04-19 08:00)

## 7.6	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-223519)

Time budget: 0 TU

Tdoc Limitation: 3 tdocs

### 7.6.1	Organizational

LSs, rapporteur inputs and other organizational documents.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs, who can submit a rapporteur CR as part of this AI.

Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Incoming LSs

R2-2402120	LS on Rel-18 RAN1 UE features list for LTE after RAN1#116 (R1-2401824; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-18	IoT_NTN_enh	To:RAN2	Cc:RAN4

- Ericsson indicates that RAN1 is discussing whether they are ok with the implementation of two capabilities (for HARQ and GNSS) that we implemented differently

Noted

R2-2402143	Reply LS on UE Location Information for NB-IoT NTN (S2-2403851; contact: Qualcomm)	SA2	LS in	Rel-18	IoT_NTN_enh	To:RAN2, CT1, RAN3	Cc:SA1, SA3-LI

Noted

R2-2402187	Discussion on SA2 and CT1 reply LS on UE Location Information for NB-IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

RAN2 agree that it is useful for MME to further signal the coarse location information received from the UE in NAS back to eNB. RAN2 can reply to SA2 with this.

R2-2402771	Discussion on the need for eNB to get UE location information from MME	Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

There is no need for the MME to signal back the UE’s coarse location to eNB, and for the eNB to report any updated ULI to the MME.

R2-2402888	Discussion on LS about UE Location Information for NB-IoT NTN	Apple	discussion	Rel-18	IoT_NTN_enh-Core

Proposal: Send a response LS to SA2 that MME does not need to signal the coarse location information to eNB.

R2-2402813	Discussion on reply LS on UE Location Information for NB-IoT NTN	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	Send LS reply that it is useful for MME to further signal the coarse location information received from the UE in NAS back to eNB. Draft LS reply is provided in [2].

- Nokia agrees it would be useful to have the coarse UE location information at the RAN side.

- Ericsson thinks this would be an optimization

- Vivo thinks we could say it’s useful

Send a reply LS saying that, for RAN2 perspective, even if not essential it may be useful for the eNB if the MME furthers signal the coarse location information received from the UE in NAS back to eNB. RAN2 thinks that whether further information can be fed back to the MME based on the received coarse location information from the MME is up to RAN3 to decide.

R2-2402814	[Draft] Reply LS on UE Location Information for NB-IoT NTN	Qualcomm Incorporated	LS out	Rel-18	IoT_NTN_enh-Core	To:SA2, RAN3

- Revised in R2-2403763

R2-2403763	[Draft] Reply LS on UE Location Information for NB-IoT NTN	Qualcomm Incorporated	LS out	Rel-18	IoT_NTN_enh-Core	To:SA2, RAN3

Rapporteur inputs

R2-2403210	Corrections to IOT NTN	Huawei, HiSilicon	CR	Rel-18	36.331	18.1.0	5011	-	F	IoT_NTN_enh-Core	Late

R2-2403211	IOT NTN ASN1 RIL List	Huawei, HiSilicon	report	Rel-18	IoT_NTN_enh-Core	Late

[AT125bis][301][IoT NTN Enh] Preliminary RILs checking (Huawei)

Scope: Allow checking the PropAgree and PropReject RILs in R2-2403221, if needed, before the online discussion. No technical discussion is expected to happen via email: disagreeing companies are invited to discuss F2F with the WI RRC rapporteur before the online session.

Intended outcome: Updated RILs list (if needed)

Deadline for rapporteur's summary in R2-2403761: Tuesday 2024-04-16 13:00

R2-2403761	Updated IOT NTN ASN1 RIL List	Huawei, HiSilicon	report	Rel-18	IoT_NTN_enh-Core

Proposal: Except for C654/E803/E804/K001, the PropAgree/PropReject/ToDo states in R2-2403211 are confirmed.

- For E803/E804 Huawei, QC and Samsung think we should stick to previous agreements.

E803 and E804 are rejected

Except for C654/K001, the PropAgree/PropReject/ToDo states in R2-2403211 are confirmed.

R2-2403630	R18 IoT NTN stage 2 remaining issues	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	The behaviour at failed GNSS acquisition is not further discussed.

Proposal 2	The behaviour for autonomous GNSS acquisition in C-DRX inactive time is not further discussed.

Proposal 3	Triggering of GNSS remaining validity duration report after autonomous GNSS acquisition in C-DRX inactive time is not further discussed.

Proposal 4	There is no need to update Feeder link and Service link definitions.

Proposal 5	First time Kmac is introduced, it is explained to be the RRC parameter k-Mac, and only Kmac is used in stage 2.

In Stage 2 we use the term Kmac saying it refers to the RRC parameter k-Mac

- Vivo thinks we shouls aling this for both 36.300 and 38.300

Proposal 6	Figure 23.21.2.1-1 is updated as above.

Agreed

Proposal 7	In 36.300 23.21.4.3 Measurements, consider adding this (same as in NR NTN): “The time-based measurement initiation may be applicable for the feeder link switchover case for cell (re)selection.”

Agreed

### 7.6.2	Stage 2 corrections

R2-2402772	Correction to Stage 2 on IoT NTN	Huawei, HiSilicon	CR	Rel-18	36.300	18.1.0	1400	-	F	IoT_NTN_enh-Core

- Regarding the first change, Samsung wonders if we should refer to hard satellite switch for IoT-NTN. HW thinks this couls also be the case for IoT-NTN. Oppo thinks this could be too detailed for Stage 2. HW indicates the same wording is used for NR NTN

Second change is endorsed

Can come back in future meetings regarding the first change

R2-2403480	Further discussion on stage-2 open issues for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1:  The GNSS Measurement Command MAC Control Element can indicate whether the UE shall move to RRC Idle or can stay RRC Connected if the GNSS measurement fails during the triggered GNSS measurement gap.

Proposal 1a: For GNSS measurement triggered by the network, the UE shall move directly to RRC idle mode after the end of the GNSS measurement gap if the UE failed to re-acquire the GNSS position, independently of the GNSS position status.

- MTK and Ericsson supports 1a. Nokia thinks that 1a would be enough.

- Google thinks this puts extra constraints on the UE and then it would be very hard to test.

- Oppo thinks this would lead to worse performance than in Rel-17

- Google thinks that the NW could release the UE in case it sees issues.

Continue in offline 303 (Nokia)

Alternative proposals from R2-2402773):

Proposal 2a:	For the network triggered GNSS measurement, upon the measurement failure, the UE doesn’t enter RRC_IDLE in case the GNSS position is valid or the uplink transmission extension is active. (No spec change)

- Google and Oppo support this proposal

- Samsung thinks the UE should at least report the failure case

Proposal 2b:	RAN2 to confirm once GNSS measurement fails, regardless of which kind of GNSS measurement it is, UE will go to RRC IDLE if the GNSS position is outdated and uplink transmission extension is not active. (No spec change)

Proposal 2: For autonomous GNSS acquisition, if the GNSS measurement fails the UE always moves to RRC Idle upon the end of autonomous GNSS measurement timer, except the measurement is triggered autonomously by the UE during C-DRX inactive time.

Proposal 3: For autonomous GNSS acquisition in C-DRX inactive time, the UE shall move to RRC idle mode if either of below conditions is met:

1)	the GNSS position is outdated and uplink transmission extension is not active, or

2)	the UL transmission extension period is expired.

Continue in offline 303 (Nokia)

Proposal 4: UE implementation can determine whether to report the remaining GNSS validity duration based on the presence of ‘gnss-PositionFixDurationReporting-r18’ in SIB2.

Continue in offline 303 (Nokia)

Proposal 5: RAN2 can discuss whether the condition based on ‘gnss-PositionFixDurationReporting-r18’ to report the remaining GNSS validity duration shall be captured in specification.

Continue in offline 303 (Nokia)

[AT125bis][303][IoT NTN Enh] Open issues on GNSS enhancements (Nokia)

Scope: Discuss the proposals in R2-2403480 (and possible counter-proposals from other companies on the same issues)

Intended outcome: Report of the offline discussion

Deadline for rapporteur's summary (in R2-2403764):  Wednesday 2024-04-17 22:00

R2-2403764	Summary of [AT125bis][303][IoT NTN Enh] Open issues on GNSS enhancements	Nokia	discussion	Rel-18	IoT_NTN_enh-Core

R2-2402213	Discussion on Autonomous GNSS Fix in C-DRX Inactive Time	vivo	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: RAN2 confirms UE will trigger GNSS remaining validity duration report after autonomous GNSS acquisition in C-DRX inactive time when the UE is communicating in a network not supporting releases later than Release 17. No spec change is needed.

Proposal 2: It is up to network implementation to release a UE before the time point that the network assumes UE’s GNSS remaining validity duration expiry.

R2-2402373	Open issues with regards to GNSS operation	PANASONIC R&D Center Germany	discussion

Proposal 1: It shall be up to the UE how it defines GNSS validity duration and up to the network how many attempts towards GNSS acquisition it triggers prior to the expiration of the GNSS validity duration. So the answer to the question asked is NO. Text of clause 23.21.2.2 in TS 36.300 V18.1.0 needs to be adjusted correspondingly.

Proposal 2: Adjust existing definition in TS 36.300 V18.1.0, clause 23.21.2.2, in a way that applying GNSS acquisition trials do not function as GNSS validity duration extensions.

Proposal 3: Amend the existing specification TS 36.300 V18.1.0, clause 23.21.2.2, in a way that the rule making up the boxed question above is integrated.

### 7.6.3	RRC Corrections

[V510] (Marked PropAgree)

R2-2402214	[V510] Correction on GNSS Measurement Failure	vivo	CR	Rel-18	36.331	18.1.0	5002	-	F	IoT_NTN_enh-Core

- HW indicates this is already captured in the rapporteur CR

Not Pursued

[K001] (Marked PropAgree)

R2-2402584	[K001] Discussion on T317 expiry during GNSS measurement	ASUSTeK	discussion	Rel-18	36.331	IoT_NTN_enh-Core

Proposal 1: [K001] Consider T317 expiry upon receiving indication that GNSS becomes valid if timer T317 expires during GNSS measurement. Adopt the TP as above..

- HW indicates this is already captured in the rapporteur CR. ASUSTeK agrees with that.

- QC thinks the current text is fine but is ok to further check offline.

Not Pursued

CB Thursday

[C651] (Marked ToDo)

[C652] (Marked PropReject)

[C653] (Marked PropAgree)

[C654] (Marked PropAgree)

R2-2402913	[C651][C652][C653][C654] Corrections on Event D1, Event D2 and condEvent D2	CATT	discussion

Proposal 1 [C651]: Support the configuration of reportOnLeave for Event D1 and Event D2 in ReportConfigEUTRA and specify the corresponding procedure.

- HW supports this

Agreed

Proposal 2 [C652]: Clarify in the Spec that when hysteresisLocation is configured UE shall ignore hysteresis.

Proposal 3 [C653]: E-UTRAN does not configure useAllowedCellList for Event D1 and Event D2. Clarify this in the field description.

Proposal 4 [C654]: Remove the field cellForWhichToTriggerD2 in condEvent D2 and related descriptions.

Discuss in the NR NTN session and attempt to align between IoT NTN and NR NTN

Proposal 5: Adopt the TP in Annex A.

[H004] (Marked ToDo)

R2-2403491	[H004] Addition of polarization parameters	Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: Add polarization information to measObjectEUTRA and SIB33.

- Ericsson thinks this is not needed for IoT NTN and a 3dB budget loss was already considered to take this into account. MTK agrees

- QC thinks could be useful and suggests to send a LS to RAN4

- MTK thinks this is not a correction but an optimization and we don’t need this. Nokia agrees

We don’t add polarization information to measObjectEUTRA and SIB31/SIB33 unless explicitly requested (no LS is sent). We also don’t introduce this change for R17

Proposal 2: Adopt the TP in the Annex.

[S066] (Marked PropAgree)

[S067] (Marked ToDo)

[S068] (Marked ToDo)

R2-2403335	Various RRC connection for IoT NTN including [S066][S067][S068]	Samsung	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: Agree [S066] – remove extension field “...” from NeighSatelliteInfo.

Agreed

Proposal 2: [067] is agreed – T390 is stopped upon RRC re-establishment initiation.

- Apple agrees for the re-establishment case, not sure about the HO case. ZTE agrees

Update the specification to ensure that T390 is stopped upon RRC re-establishment initiation

Proposal 3: [068] is agreed – T390 is stopped upon starting handover.

- Google thinks is related to whether the UE performs GNSS measurements during HO.

- Nokia agrees that stopping the timer is the clean solution, especially in the re-establishment case

Further check this offline

CB Thursday

[X041] (Marked ToDo)

[X042] (Marked ToDo)

R2-2403717	[X041][X042] Correction on GNSS operation enhancement	Beijing Xiaomi Mobile Software	discussion	Rel-18

[X041] revise the wording “GNSS becomes valid” to “GNSS position is fixed” or “GNSS position is updated”.

- QC supports this change

revise the wording “GNSS becomes valid” to “…. new GNSS position becomes valid” (actual wording can be further checked offline).

[X042] Clarify in field description of gnss-AutonomousEnabled that it is only applicable to autonomous GNSS measurement during autonomous gap. And introduce capability signalling to indicate the support of autonomous GNSS measurement during inactive period of C-DRX.

- ZTE disagrees. Vivo agrees.

- Oppo is fine with the first part, not with the second

- Samsung agrees with the proposal. CATT also agrees

- Apple thinks that in any case this would remain a best effort task for the UE. Oppo thinks that in any case a simple capability reporting would not be sufficient. QC agrees. HW also agrees

We don’t introduce capability signalling to indicate the support of autonomous GNSS measurement during inactive period of C-DRX.

Continue offline to discuss whether we clarify in field description of gnss-AutonomousEnabled that it is only applicable to autonomous GNSS measurement during autonomous gap

CB Thursday

[E801] (Marked ToDo)

R2-2403723	[E801] Satellite assistance information for event D2	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	RAN2 to discuss whether event D2 is cell or satellite specific.

Proposal 2	For eventD2/condEventD2 satellite assistance information is included in ReportConfig, outside the event type. This solution applies to both NR and IoT NTN.

Proposal 3	Adopt the Text Proposal in Section 3 as a baseline.

[N021] (Marked ToDo)

R2-2403481	Remaining issue on gap length for autonomous GNSS measurement	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: eNB can indicate the supported GNSS measurement gap length(s) for autonomous GNSS measurement.

- HW thinks this would add extra flexibility which is not needed. Ericsson agrees with HW

Proposal 2: The supported GNSS measurement gap length(s) can be indicated by NW in UE-specific RRC message (e.g., RRCConnectionSetup).

Proposal 3: UE shall report the GNSS position fix duration among the gap length values supported by network.

R2-2402185	Extending scenarios for t-service	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	T-service is also broadcasted for earth moving cells.

Proposal 2	Adopt the following TP for t-service’s field description.

R2-2402203	Correction to 36.331 for IoT NTN	OPPO	CR	Rel-18	36.331	18.1.0	4999	-	F	IoT_NTN_enh-Core

R2-2402383	RRC corrections on T390 and MO for IoT NTN	ZTE Corporation, Sanechips	CR	Rel-18	36.331	18.1.0	5001	-	F	IoT_NTN_enh-Core

GNSS operation enhancements

R2-2402773	Remaining issues on GNSS operation enhancements	Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

- Value reset of T390

Proposal 1:	RAN2 to agree Alt-1a and send a reply LS to RAN1.

-	ZTE thinks we should go for a fized value but it could be the Y value

-	MTK supports 1a and don’t think we should introduce a new option. Nokia agrees

-	Oppo prefers option 1. Google agrees. Vivo agres

-	Ericsson thinks that Alt1 adds extra complexity to the UE and could be avoided. HW agrees. ZTE also agrees and can compromise to 1a.

We go for Alt 1a. Send a LS back to RAN1 accordingly in R2-2403765

- Clarification of UE behaviours related to GNSS acquisition

Proposal 2a:	For the network triggered GNSS measurement, upon the measurement failure, the UE doesn’t enter RRC_IDLE in case the GNSS position is valid or the uplink transmission extension is active. (No spec change)

Proposal 2b:	RAN2 to confirm once GNSS measurement fails, regardless of which kind of GNSS measurement it is, UE will go to RRC IDLE if the GNSS position is outdated and uplink transmission extension is not active. (No spec change)

Proposal 2c:	RAN2 to confirm that upon outdated GNSS position the UE doesn’t enter RRC_IDLE if GNSS acquisition is being performed, regardless of which kind of GNSS acquisition is ongoing.

- GNSS remaining validity duration report to legacy eNB

Proposal 3:	No special handling is needed regarding GNSS remaining validity duration report to the legacy eNB.

- Impact on TA report

Proposal 4:	Before sending the TA report triggered during the GNSS measurement gap, the triggering condition should be re-evaluated after the GNSS measurement gap.

R2-2403765	[Draft] Reply LS on …	Huawei	LS out	Rel-18	IoT_NTN_enh-Core	To:RAN1

R2-2402186	Discussion on GNSS operation for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

R2-2402705	Discussion on IOT NTN GNSS operation enhancement related open issues	Xiaomi	discussion	Rel-18	IoT_NTN_enh-Core

R2-2403081	Remaining Issues on the GNSS Operation Enhancements	Google Inc.	discussion	Rel-18

On satellite assistance information

R2-2402908	On the necessity of satellite assistance information for measurement in IoT NTN	CATT	discussion

### 7.6.4	MAC corrections

R2-2403220	Discussion on remaining MAC issues for Rel-18 IoT NTN	MediaTek Inc.	discussion	IoT_NTN_enh-Core

Proposal 1: When timeAlignmentTimer is not infinity, T390 is reset with length equal to configured timeAlignmentTimer value when receiving the UL Transmission Extension Update MAC CE.

Proposal 2: For single TB scheduled by DCI, for a HARQ process configured as HARQ feedback disabled by RRC and further reversed to HARQ feedback enabled by DCI, NB-IoT UE behaviour on DRX follows the case when HARQ feedback is disabled.

Proposal 3: For multiple TBs scheduled by DCI, for a HARQ process configured as HARQ feedback disabled by RRC and further reversed to HARQ feedback enabled by DCI, NB-IoT UE behaviour on DRX follows the case when HARQ feedback is enabled.

Proposal 4: For NB-IoT, when multiple TBs are scheduled by PDCCH for the non-interleaved case or for the interleaved case when HARQ-ACK bundling is not configured, RAN2 does not change the HARQ RTT timer.

R2-2403482	On MAC open issues for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: When UL Transmission Extension Update MAC CE is received, the T390 restarts with a timer value equal to the configured TAT time length.

Proposal 2: If UL Transmission Extension is enabled, upon original GNSS validity duration expires the T390 starts with a timer value equal to the configured TAT time length.

Proposal 3: For single TB scheduling case in NB-IoT: for a HARQ process configured as HARQ feedback disabled by RRC and further reversed to HARQ feedback enabled by DCI, UE behavior on DRX follows the case when HARQ feedback is disabled.

Proposal 4: For multiple TB scheduling case in NB-IoT: for HARQ process(es) configured as HARQ feedback disabled by RRC and further reversed to HARQ feedback enabled by DCI, UE behavior on DRX follows the case when HARQ feedback is enabled.

R2-2402204	Discussion on remaining issue on GNSS validity duration reporting	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	If a GNSS Validity Duration Report MAC MAC CE is included in a MAC PDU for Msg3 transmission, all triggered GNSS validity duration reports shall be cancelled when the Random Access procedure is successfully completed.

R2-2402215	Discussion on MAC Remaining Issues	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2402704	Discussion on issue related to UL Transmission Extension Update MAC Control Element	Xiaomi	discussion	Rel-18	IoT_NTN_enh-Core

R2-2403631	R18 IoT NTN GNSS extension	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

R2-2403221	Corrections on UE behaviour on DRX for IoT NTN	MediaTek	CR	Rel-18	36.321	18.1.0	1585	-	F	IoT_NTN_enh-Core

Withdrawn

R2-2402384	MAC corrections to IoT NTN	ZTE Corporation, Sanechips	CR	Rel-18	36.321	18.1.0	1584	-	F	IoT_NTN_enh-Core	Withdrawn

### 7.6.5	Corrections to other specs

Corrections to other affected specs, including corrections on UE capabilities

Corrections on issues affecting multiple Stage 3 specs (e.g. RRC and MAC) can also be submitted here

R2-2403152	Miscellaneous correction for IoT-NTN	Nokia 	CR	Rel-18	36.304	18.1.0	0873	-	F	IoT_NTN_enh-Core

R2-2402385	Corrections to idle mode measurement for IoT NTN	ZTE Corporation, Sanechips	CR	Rel-18	36.304	18.1.0	0874	-	F	IoT_NTN_enh-Core

R2-2402915	Corrections on Location-based Measurement Initiation in TS 36.304	CATT	discussion

Proposal 1: For the location-based measurement initiation procedure, change the conditions “If the distance between the UE and the serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements” and “Else, the UE shall perform intra-frequency measurements” into a lower-level condition under the loop “If referenceLocation is set to fixedReferenceLocation and if the UE supports location-based measurement initiation for fixed cell, referenceLocation is used as serving cell reference location.”.

Proposal 2: Change “fixed cell” and “moving cell” to “quasi-Earth fixed cell” and “Earth moving cell” respectively in 36.304 to align with TS36.331 and TS36.306.

Proposal 3: Support location-based measurement initiation for earth fixed cell in TS 36.304 (in addition to quasi-earth fixed cell).

Proposal 4: Add definitions for quasi-Earth fixed cell, Earth fixed cell and Earth moving cell in TS 36.304.

Proposal 5: Adopt the TP in Annex for Proposal 1-4.

R2-2403336	On procedures and capabilities related to GNSS fix during C-DRX	Samsung	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: Clarify that UE reports GNSS Validity Duration Report MAC CE after successful GNSS position fix during C-DRX.

Proposal 2: “GNSS measurement during inactive” time is changed to a signalled capability.

Proposal 3: Agree text proposal to 36.306 in Appendix A.

R2-2403614	Corrections on uplink transmission extension	Samsung	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: Feature should be named “GNSS invalidity duration”.

Proposal 2: UE supports both procedures for timeAlignmentTimer infinity and non-infinity if ntn-UplinkTxExtension is supported.

Proposal 3: Clarify that UE shall support receiving Uplink Transmission Extension Update MAC CE if ntn-UplinkTxExtension is supported.

Proposal 4: Agree text 36.306 proposal.

R2-2402812	Remaining issues on out-of-date GNSS fix	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

## 7.7	NR NTN enhancements

(NR_NTN_enh -Core; leading WG: RAN1; REL-18; WID: RP-232669)

Time budget: 0 TU

Tdoc Limitation: 3 tdocs

### 7.7.1	Organizational

LSs, rapporteur inputs and other organizational documents.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs, who can submit a rapporteur CR as part of this AI.

Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Incoming LSs

R2-2402114	Reply LS on Satellite Switch with Resync (R1-2401748; contact: Apple)	RAN1	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2	Cc:RAN4

Noted

R2-2402129	Reply LS on RAN2 agreements for satellite switch with resync (R4-2403493; contact: Apple)	RAN4	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2	Cc:RAN1

Noted

R2-2402542	Discussion on reply LS from RAN1 and RAN4 for unchanged PCI	CMCC	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: No additional specification work for RAN2 is needed to support UE to perform the downlink synchronization with the target satellite and keep the communication with the source satellite of the same serving cell simultaneously in soft satellite switch based on the reply LS from RAN1 and RAN4.

-	CATT thinks we could still discuss a few aspects related to soft switch

Rapporteur inputs

R2-2403632	Rapporteur input R18 NR NTN RRC	Ericsson	CR	Rel-18	38.331	18.1.0	4761	-	F	NR_NTN_enh-Core	Late

R2-2403633	Rapporteur’s input R18 NR NTN RRC RILs	Ericsson	discussion	Rel-18	NR_NTN_enh-Core	Late

[AT125bis][302][NR NTN Enh] Preliminary RILs checking (Ericsson)

Scope: Allow checking the PropAgree and PropReject RILs in R2-2403633, if needed, before the online discussion. No technical discussion is expected to happen via email: disagreeing companies are invited to discuss F2F with the WI RRC rapporteur before the online session.

Intended outcome: Updated RILs list (if needed)

Deadline for rapporteur's summary in R2-2403762: Tuesday 2024-04-16 13:00

R2-2403762	Updated Rapporteur’s input R18 NR NTN RRC RILs	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

For the following RILs, the conclusion proposed by the rapporteur can be considered for agreement:

C604, C650, C651, E145, E301, H063, H116, H117, H500, K005, K006, K007, N131, N132, S486, and V501.

The proposed conclusion is agreed for C604, C650, C651, E145, E301, H063, H116, H117, H500, K005, K006, K007, N131, N132, S486, and V501.

The following RILs need further discussion:

E252, H010, O600, V500, E251, E300, E302, and H115.

### 7.7.2	Stage 2 corrections

R2-2402798	Stage-2 corrections	Samsung	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: If satellite switch with resync is a new trigger for NW to send PDCCH order CFRA, add stage-2 description of PDCCH order CFAR triggered by satellite switch with resync.

### 7.7.3	RRC corrections

Satellite switching with re-sync

- [K005][V500][V501] epochTime/ ntn-UlSyncValidityDuration clarification

[K005] (Marked ToDo)

R2-2402585	[K005] Discussion on epoch time for satellite switch	ASUSTeK	discussion	Rel-18	38.331	NR_NTN_enh-Core

Proposal 1: [K005] Add “For satellite switch with resynchronization, it indicates the SFN nearest to the frame where the message indicating the epochTime is received” to the EpochTime field descriptions.

-	Ericsson and QC think the change is not needed.

R2-2402188	Discussion on soft switch unchanged PCI	OPPO	discussion	Rel-18	NR_NTN_enh-Core

- Revised in R2-2403951 (to suggest changes based on the latest version of 38.331)

R2-2403951	Discussion on soft switch unchanged PCI	OPPO	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	To cover soft switch unchanged PCI, adopt the above TP.

-	ZTE supports the clarification suggested by Oppo

Introduce a clarification to the EpochTime field description to cover satellite switch with resync, e.g. along the lines of what suggested in R2-2403951

[V500/V501] (Marked ToDo)

R2-2402216	[V500][V501] Correction on NTN-Config in case of Satellite Switch	vivo	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 clarifies that UE uses the epochTime or ntn-UlSyncValidityDuration of the serving satellite if epochTime or ntn-UlSyncValidityDuration for the neighbor cell is not provided in SIB19.

V500/V501 are rejected

RAN2 understands that epochTime or ntn-UlSyncValidityDuration in the configuration for satellite switch with resync are only used for this procedure

Proposal 2: RAN2 adopts the TP in the Annex if proposal 1 is agreed.

- SSB-TimeOffset

Opt1: reference point in UL RP

R2-2402189	Discussion on ssb-TimeOffset	OPPO	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	Keep the current spec wording that ssb-TimeOffset is the time offset between the SSB from source and target satellite at RP. Add the following TP in the field description of ssb-TimeOffset.

ssb-TimeOffset

Indicates the time offset between the SSB from source and target satellite at the uplink time synchronization reference point. It is given in number of subframes. It is used by the UE to derive the SSB timing of the target satellite together with the UE-RP propagation delay difference between source and target satellite.

-	Oppo thinks no matter which option we decide to go, we need to have some clarification in the specs

-	vivo agrees and thinks option 2 (reference point in the gNB) is the best approach. LG agrees

-	CMCC and ZTE think that both option work but option 2 is simpler. Nokia agrees. CATT agrees

-	Google thinks that option 2 is more aligned to idle mode behaviour.

-	Apple is fine with both but thinks that RAN4 is defining requirements based on option 1 so in case we need to inform them

-	HW thinks the granularity of 1ms would be ok in this case.

Send an LS to RAN4 saying that RAN2 is considering to adopt the gNB as the reference point and asking whether this would be a problem from RAN4 perspective

Opt2: reference point in gNB

R2-2402218	Further Discussion on  ssb-TimeOffset	vivo	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: The ssb-TimeOffset within the SatSwitchWithReSync IE indicates the time offset between the SSB from source and target satellite at the gNB.

Proposal 2: Capture in the field description of the ssb-TimeOffset that UE shall adjust the actual ssb-TimeOffset based on the actual service link and feeder link propagation delay difference between source satellite and target satellite.

Proposal 3: If Proposal 1 and Proposal 2 are agreed, RAN2 adopts the Text proposal in Annex.

R2-2402844	Discussion on Remaining Open Issues for Unchanged PCI Mechanism	CATT	discussion

Observation 1: Per current procedures in 5.7.19, the agreement "UE shall apply the acquired DL timing and start accessing the target satellite with related operations (e.g. restart T430, reset N_TA, resume UL operations) not before t-Service" is missing in the specification for a UE that has already acquired Target DL timing before t-Service.

Proposal 1: Add a Note in 5.7.19 clarifying that the UE which has already acquired the timing of the target satellite may omit acquiring the DL timing when the UE starts synchronising to the DL of the target satellite.

Can come back to this after checking the latest version of the CR

Proposal 1a: Adopt the TP for Option 2 in Annex A.

Proposal 2: Clarify that reference point of ssb-TimeOffset is the gNB, and specify this in the field description of ssb-TimeOffset as "Indicates the time offset between the SSB from source and target satellite at the gNB".

Observation 2: The ssb-PositionsInBurst and ssb-periodicityServingCell do not need to be changed, as they are for the same serving cell after satellite switch with unchanged PCI.

Proposal 3: Clarify that the ssb-TimeOffset refers to the time offset between the start subframe of the SSB burst from source and target satellite.

-	HW agrees with the issue and thinks we could refer to the specific SSB index

-	CATT thinks this proposal is to avoid referring to SSB index.

-	ZTE is not sure anything else needs to be clarified: the ssb-TimeOffset is the same between different SSB indices

We don’t introduce additional clarifications

Proposal 4: Adopt the TP in Annex B.

R2-2403068	Remaining issues on unchanged PCI	ZTE Corporation, Sanechips	discussion	Rel-18	NR_NTN_enh-Core

- ssb-Timeoffset clarification

Proposal 1: The RP for provided ssb-TimeOffset is at gNB, and UE shall calculate offset with consideration actual propagation delay. Update the field description ssb-TimeOffset as provided in TP1.

Proposal 2: If P1 is not agreed, then the field description ssb-TimeOffset is updated clarify the SSB offset is provided per UL RPs from both source and target, and UE shall calculate offset with consideration actual propagation delay. Update the field description ssb-TimeOffset as provided in TP2.

- RACH-based satellite switch with resync

Proposal 3: For satellite switch with resync, existing mechanism (e.g., PDCCH order) can be used to trigger RACH, there is no need to introduce new RACH trigger.

R2-2402831	Discussion on the remaining issues for NR NTN	Xiaomi	discussion

Opt3: Use an equation for the calculation

R2-2402335	SMTC configuration of target satellite for satellite switch with re-sync	NTU	discussion	Rel-18

Proposal 1: For hard switch, SMTC configuration is applied by UE based on functions (1) and (2).

Proposal 2: For soft switch, SMTC configuration is applied by UE based on functions (3) and (4).

[AT125bis][304][NR NTN Enh] LS to RAN4 on reference point (Apple)

Scope: Draft an LS to RAN4 on on reference point for SSB-TimeOffset

Intended outcome: Agreeable LS

Deadline for draft LS (in R2-2403765):  Friday 2024-04-19 08:00

R2-2403765	[Draft] LS on reference point for SSB-TimeOffset	Apple	LS out	Rel-18	NR_NTN_enh-Core	To:RAN4

- RACH-based

[H063] (Marked as PropReject)

R2-2403192	[H063] RACH-based satellite switching with re-sync	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1:  For satellite switching with re-sync, both RACH-less and RACH-based procedure are supported.

-	MTK thinks there should be no UE related issue (all UEs should be able to support RACH-less procedure) but maybe in some scenarios it would be useful to rely on a RACH-based approach

-	QC agrees with MTK and thinks that in case we can rely on PDCCH order

-	HW thinks that if we go for PDCCH order we need to rely on CFRA, which is an issue for the NW.

-	Samsung thinks that for PDCCH order both CFRA and CBRA with TAT expired are supported in current specs.

Proposal 2:  Downselect from the options on RACH-based satellite switching with re-sync:

-	Option 1: Network uses dedicated RRC signalling to configure whether RACH is performed during satellite switching with re-sync;

-	Option 2: Network uses 1-bit in the broadcast signalling to configure whether RACH is performed during satellite switching with re-sync;

-	Option 2b: On top of the 1-bit indication, network also indicates a maximum delay/time window for RACH attempts;

-	Option 3: It is up to UE implementation to decide whether RACH is performed, and the RACH resources are configured by the network in SIB1 (as in legacy).

We only rely on a PDCCH order based solution to trigger RACH during satellite switch with resync in Rel-18 (no other option is considered in Rel-18)

Continue in offline 305 to check what we need to add to make a PDCCH-order based solution work in the satellite switch with resync case

R2-2402543	[H063] Discussion on RACH-based solution for unchanged PCI	CMCC	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: [H063] Support legacy CB RACH solution for unchanged PCI.

Proposal 2: Reuse the rachlessHandoverNTN-r18 and add some description to indicate whether UE supports RACH-less operation in unchanged PCI case.

Proposal 3: It is proposed to provide RA resource configuration (i.e. RACH-ConfigCommon, MsgA-ConfigCommon) of the incoming target satellite in the SatSwitchWithReSync optionally.

Proposal 4: Alliteratively, if the RA resource configuration of target satellite is absent, the RA resource configured in SIB1 for current satellite can be applied to the target satellite.

Proposal 5: If P1 is agreed, UE should apply the TA command of RAR/MsgB whether TAT is running or not to keep TA alignment between UE and network.

Proposal 6: Kindly suggest RAN2 to adopt the TP in the annex.

R2-2402800	RIL S486, V500, V501, H063	Samsung	discussion	Rel-18	NR_NTN_enh-Core

Proposal 3: RAN2 confirms for satellite switch with resync NW can send PDCCH order CFAR/CBRA as legacy trigger (i.e., DL/UL date arrives but TAT is expired).

Continue in offline 305

Proposal 4: RAN2 discuss if NW can send PDCCH order CFRA for the purpose of resynchronization in satellite switch even there is no DL/UL data or TAT is still running (i.e., satellite switch with resync become a new trigger for PDCCH order CFRA).

Continue in offline 305

R2-2403635	Remaining issues for soft switch with unchanged PCI	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	A RACH-based solution to access the target cell during satellite switch with re-synchronization is down prioritized in Release 18.

Proposal 2	If RACH-based access is agreed, revert the previous agreement which mandates all UEs to switch cell upon t-Service since it will cause RACH congestion.

Proposal 3	If RACH-based access is agreed, UE derives the UE-specific switch time based on a (pre)configured rule without dedicated signalling.

R2-2403301	On RACH-based Satellite Switching with Resynchronization and Confirming Successful Switching	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 to discuss means for reducing the NW uncertainty in case of satellite switching with resynchronization.

Proposal 2: RAN2 to agree on a NW-controlled timer that starts at t-Service, stops upon UE’s UL confirmation and allows the NW to release UE-specific resources upon timer’s expiration.

Proposal 3: In case a UE successfully completed the satellite switching but NW did not receive any UL confirmation, upon timer expiration a UE can send an SR or trigger RACH procedure.

Proposal 4: Timer controlling how long the NW keeps the UE-specific resources in satellite switching with resynchronization can be signalled in SIB19.

Proposal 5: Timer controlling how long the NW keeps the UE-specific resources in satellite switching with resynchronization can be UE-specific and depend on UE’s capabilities or UE’s UL buffer.

Proposal 6: RAN2 to support RACH-less and RACH-based access for satellite switching with re-sync.

Proposal 7: NW indicates RACH as the procedure access and the maximum delay time to perform RACH attempt.

Moved here from 7.7.5

R2-2402586	Discussion on RACH-based satellite switch	ASUSTeK	discussion	Rel-18	38.321	NR_NTN_enh-Core

[AT125bis][305][NR NTN Enh] PDCCH order during satellite switch (Samsung)

Scope: Discuss the proposals p3 and p4 in R2-2402800

Intended outcome: Report of the offline discussion

Deadline for rapporteur's summary (in R2-2403766):  Wednesday 2024-04-17 22:00 (if F2F offline is possible on Wednesday, otherwise Friday 2024-04-19 08:00)

R2-2403766	Summary of [AT125bis][305][NR NTN Enh] PDCCH order during satellite switch	Samsung	discussion	Rel-18	NR_NTN_enh-Core

- Idle mode support

[N131] (Marked PropAgree)

[N132] (Marked PropReject)

R2-2403302	RRC Corrections and Proposed RIL [N131] and [N132] Resolutions for Rel-18 NTN	Nokia	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 is asked to adopt the Text Proposal (TP) from Annex A.

Proposal 2: if t-ServiceStart is configured, IDLE UEs shall continue in the same (logical) cell without performing cell reselection.

Proposal 3: RAN2 to discuss the UE behaviour in case S-Quality condition triggers right before t-ServiceStart. A corresponding LS to RAN4 can be considered.

Proposal 4: RAN2 to agree that UEs in RRC_IDLE can use SSB time-offset in SIB19.

- Measurement relaxation

R2-2402217	Remaining Issue on Measurement during Hard Satellite Switch	vivo	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 to capture the agreement that the UE is allowed to skip measurements on other cells and satellites than the target satellite from T-service until the satellite switch completion.

-	Nokia thinks we already agreed that the UE will reset filters and that in case this will be reflected in RAN4 specs.

-	Apple thinks we need to capture the agreement somewhere

Can come back to this if nothing will be captured for this in RAN4 specs

Proposal 2: RAN2 to adopt the text proposal in the Annex.

SIB19 in TN

[O600] (Marked ToDo)

R2-2402190	[O600] Discussion on TN cell broadcasting NTN info	OPPO	discussion	Rel-18	NR_NTN_enh-Core

- Revised in R2-2403952 (to suggest changes based on the latest version of 38.331)

R2-2403952	[O600] Discussion on TN cell broadcasting NTN info	OPPO	discussion	Rel-18

Proposal 1	The field description of epochTime is corrected as:

-	QC supports this, apart fronm the reference to or SatSwitchWithReSync

Text in proposal 1 is agreed, removing the reference to SatSwitchWithReSync

R2-2403636	UE behaviour upon absence of EpochTime in TN cells	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	RAN2 to decide whether epochTime is a mandatory or optional parameter when SIB19 is broadcast in TN cells.

Proposal 2	If epochTime is an optional parameter when SIB19 is broadcast in TN cells, clarify UE behaviour upon its absence taking the TP presented in section 3 as a baseline.

(Cond)EventD2

[C650] (Marked Duplicate)

[C651] (Marked PropReject)

R2-2402850	[C650] [C651] Corrections on EventD2 and condEventD2	CATT	discussion

Proposal 1 [C650]: Clarify in the description of Ml2 in 5.5.4.15a that the epoch time and satellite ephemeris for the candidate cell refer to the configuration in measObjectNR.

Proposal 2 [C651]: Modify the field description of distanceThreshFromReference1/ distanceThreshFromReference2 in EventTriggerConfig as follows: "Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 or a moving reference location determined by the UE based on the serving cell movingReferenceLocation broadcast in SIB19 or referenceLocation2 and their corresponding satellite ephemeris and epoch time. Each step represents 50m."

Proposal 3: Adopt the TPs in Annex A and B.

R2-2402882	Correction on referenceLocation2	Apple	discussion	Rel-18	NR_NTN_enh-Core

Proposal: RAN2 to select one from the two options to fix the problem.

- Option 1: Move referenceLocation2 to MeasObjectNR

- Option 2: Introduce cellForWhichToTriggerD2-r18 into eventD2-r18 and condEventD2-r18 in TS38.331.

Continue in offline 306

[H115] (Marked PropReject)

[H116] (Marked Duplicate)

R2-2403490	RRC corrections on RILs [H115][H116]	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Clarify in clause 5.2.2.2.1 that only SIB19 is re-acquired after satellite switching with re-sync. Add the UE behaviour of obtaining SIB19 in clause 5.2.2.4.21.

Proposal 2: Adopt the TP for [H115] in the Annex.

Proposal 3: For association with the ephemeris in MeasObjectNR, a PCI is added to the neighbour cell reference location in ReportConfigEUTRA.

Proposal 4: Adopt the TP for [H116] in the Annex.

[AT125bis][306][NR NTN Enh] Corrections on (cond)EventD2 (Ericsson)

Scope: Discuss the proposal in R2-2402882 (and corresponding ones from other companies on the same issue)

Intended outcome: Report of the offline discussion

Deadline for rapporteur's summary (in R2-2403767):  Wednesday 2024-04-17 22:00 (if F2F offline is possible on Wednesday, otherwise Friday 2024-04-19 08:00)

R2-2403767	Summary of [AT125bis][306][NR NTN Enh] Corrections on (cond)EventD2	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

VSAT UEs

R2-2403634	Remaining issue on VSAT UEs	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Observation 1	Mechanically steered VSATs are limited to blind handover for service link switch.

Observation 2	RAN4 has extended the maximum interruption time and handover delay for mechanically steered VSATs.

Observation 3	Worst-case, the new delay component could accumulate up to 7.3 seconds and consume a significant portion of T304.

Observation 4	Mechanically VSAT UEs may require the network to configure larger values of T304.

Proposal 1	Upon handover, a VSAT UE starts T304 when it has aligned its beam towards the target satellite.

Other

R2-2402867	Open issues on NR NTN measurement enhancement	Apple	discussion	Rel-18	NR_NTN_enh-Core

R2-2403082	Provision of the TN PLMN ID in an NTN Cell	Google Inc., Continental Automotive	discussion	Rel-18	R2-2400501

On support of Satellite Switch with Resync

R2-2402799	Discussion on LS replies for Satellite Switch with Resync	Samsung	discussion	Rel-18	NR_NTN_enh-Core

R2-2403193	Discussion on satellite switch with re-sync	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

Withdrawn

R2-2402263	SMTC configuration on satellite switch with re-sync	NTU	discussion	Withdrawn

R2-2402264	SMTC configuration on satellite switch with re-sync	NTU	discussion	Withdrawn

### 7.7.4	MAC corrections

R2-2402774	Discussion on HARQ buffer flush during satellite switch with re-synchronization	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: During satellite switch with re-synchronization, UE doesn’t flush the HARQ buffers.

R2-2403637	TAT handling in RACH-less CHO	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Observation 1	During a RACH-less handover, the MAC entity starts TAT for the target cell when the UE applies the RRC reconfiguration message.

Observation 2	In a conditional handover, the exact time when the triggering conditions are fulfilled, and the UE applies the conditional reconfiguration may be unknown to the network.

Observation 3	In RACH-less conditional handover, the network may not know the exact time when TAT for the selected target cell is started.

Proposal 1	Start TAT for the (selected) target cell upon successful completion of the RACH-less (C)HO.

Proposal 2	If P1 is agreed, RAN2 to discuss the TPs presented in section 3.

### 7.7.5	Corrections to other specs

Corrections to other affected specs, including corrections on UE capabilities

Corrections on issues affecting multiple Stage 3 specs (e.g. RRC and MAC) can also be submitted here

R2-2403300	On Scheduling Restrictions in Satellite Soft Switching with Resynchronization – RAN1 and RAN4 feedback	Nokia	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: The SSBs received from two satellites during soft-switching with resynchronization need to be spaced in time by at least 1 OFDM symbol at the UE’s receiver.

Observation 2: The UEs performing satellite soft-switching with resynchronization may be subject to scheduling restrictions.

Observation 3: The NW might have an issue to effectively schedule the UE during soft-switching period if the UE does not support parallelMeasurementWithoutRestriction and simultaneousRxDataSSB-DiffNumerology.

Observation 4: simultaneousRxDataSSB-DiffNumerology may not be fully relevant to soft-satellite switching with resynchronization, as serving cell is the same as the neighbour, so the same configuration (including the numerology) is used, for both SSB and PDSCH/PDCCH from any of these satellites.

Proposal 1: In case the UE supports softSatelliteSwitchResyncNTN-r18, the UE shall also support parallelMeasurementWithoutRestriction.

Observation 5: Service link propagation delay difference (PDD) can be used to measure the service link between the inbound and outbound satellite in satellite soft-switching with resynchronization.

Proposal 2: In case the UE supports softSatelliteSwitchResyncNTN-r18, the UE shall also support serviceLinkPropDelayDiffReporting-r17.

Proposal 3: Confirm with RAN4 that service link propagation delay difference helps in reducing the scheduling restriction duration during satellite soft-switching with resynchronization. RAN2 asks how early such measurements, calculations and reporting should be done.

Proposal 4: Adopt the 38.306 and 38.331 Text Proposals in the Annex A and B.

Moved here from 7.7.3

R2-2402866	Clarification on UE operation during soft satellite switch with resync	Apple	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: If NW configures soft satellite switch with resync, network implementation should ensure the SSB reception time in UE side is not be overlapped and with at least 1 symbol difference.

Proposal 2: If UE supports soft satellite switch with resync, UE should support the simultaneous transmission/reception in source satellite and DL sync in target satellite during the switch period.

-	CMCC thinks we don’t need this restriction. CATT agrees

-	Nokia supports p2

Agreed

Proposal 3: Clarify that UE supporting soft satellite switch with resync also supports parallelMeasurementWithoutRestriction-r17; in the different SCS case, UE also supports multaneousRxDataSSB-DiffNumerology.

-	Nokia thinks we need to support p3, otherwise scheduling restrictions will have to be considered and this would kill the feature

-	Apple thinks that if we don’t go for p3 then we need to make the softSatelliteSwitchResync capability per band

-	QC is ok with the first part of p3

Can come back Thursday to check if we need to make the softSatelliteSwitchResync capability per band

CB Thursday

R2-2403069	Discussion on NTN FR2 UE capability	ZTE Corporation, Sanechips	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Add the reference of FR2 band (i.e., Table 5.2.3-1 of TS38.101-5) for UE capability: uplink-TA-Reporting, uplinkPreCompensation,ue-specific-K-Offset and k1-RangeExtension.

Proposal 2: Update TS 38306 to allow indicate below UE capabilities for NTN FR2:

- SDT related: cg-SDT-r17, mt-CG-SDT-r18

- MBS related: maxDynamicSlotRepetitionForSPS-Multicast-r17, maxNumberG-CS-RNTI-r17, maxNumberG-RNTI-r17, multiPUCCH-HARQ-ACK-ForMulticastUnicast-r17, priorityIndicatorInDCI-Multicast-r17, priorityIndicatorInDCI-SPS-Multicast-r17, releaseSPS-MulticastWithCS-RNTI-r17, sps-MulticastMultiConfig-r17, re-LevelRateMatchingForMulticast-r17, twoHARQ-ACK-CodebookForUnicastAndMulticast-r17

Proposal 3: RAN2 discuss P1/P2 and agree on the corresponding CR in R2-2403070.

R2-2403070	CR to 38306 on NTN FR2 UE capability	ZTE Corporation, Sanechips	CR	Rel-18	38.306	18.1.0	1074	-	F	NR_NTN_enh-Core

R2-2402852	Correction on Location-based Measurement Initiation for Earth Fixed Cell in TS 38.304	CATT	discussion

Proposal 1: Add the support of location-based measurement initiation for Earth-fixed system in section 5.2.4.2, and add the definition of Earth-fixed system in section 3.1 in TS 38.304.

Proposal 2: Change “Quasi-Earth-fixed cell” and “Earth-moving cell” to “Quasi-Earth-fixed system” and “Earth-moving system” in section 3.1 in TS 38.304 for alignment between the definition and procedure.

Proposal 3: Adopt the TP in Annex.

## 8.8	NTN for NR Ph3

(NR_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-240775

LTE_TN_NR_NTN_mob, leading WG: RAN2, Rel-19 WID: RP-240846)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.8.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2402357	Work plan for Rel-19 NR_NTN_Ph3	CATT, Thales	Work Plan	Rel-19

R2-2403638	NR NTN phase 3 scope	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.2	Downlink coverage enhancements

Contributions should take into account corresponding progress in RAN1.

R2-2402219	RAN2 Aspects For Downlink Coverage Enhancements	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402547	Downlink coverage enhancement for NR NTN	CMCC,CSPG	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402702	Discussion on downlink coverage enhancements for NTN	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402712	Network energy saving for downlink coverage enhancement in NTN	Lenovo	discussion	Rel-19

R2-2402805	Downlink Coverage Enhancement	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402825	Discussion on downlink coverage enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402883	DL coverage enhancement in NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2403034	DL coverage enhancements	Nokia, Nokia Shanghai Bell	discussion	NR_NTN_Ph3-Core

R2-2403071	Consideration on downlink coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2403276	Discussion on RAN2 Aspects for Downlink Coverage Enhancements in NR NTN evolution	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2403319	Downlink coverage enhancement for NTN	InterDigital	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2403649	Discussion for DL coverage enhancement	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.3	Uplink Capacity Throughput Enhancement

No contributions are expected for this AI at this meeting.

### 8.8.4	Support of Broadcast service

Contributions should address the signaling of the intended service area of a broadcast service.

R2-2402152	Signaling of indicating service area in NR NTN	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402199	Discussion on providing MBS service area in NTN network	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402220	Discussion on MBS Broadcast Provision in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402280	Discussions on signaling of the intended service area of a broadcast service	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402284	Discussion on the support of broadcast service in NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402355	Discussion on support of broadcast service via NR NTN	CATT, China Broadnet	discussion	Rel-19

R2-2402544	Discussion on MBS broadcast enhancements for NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402695	Discussion on the support of Broadcast service	HONOR	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402708	Discussion on MBS service in NTN system	CAICT	discussion

R2-2402713	On support of MBS broadcast in NTN	Lenovo	discussion	Rel-19

R2-2402806	MBS Broadcast Service Area in NTN	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402807	MBS broadcast service area information	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402826	Discussion on MBS over NTN	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402833	Discussion on the service area of a broadcast service	Xiaomi	discussion

R2-2402884	Broadcast service support over NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2403072	Consideration on broadcast service enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2403093	Discussion on MBS Broadcasting Control over NTN access	TCL	discussion	Rel-19

R2-2403121	Discussion on support of broadcast service in NTN	LG Electronics France	discussion	Rel-19	NR_NTN_Ph3

R2-2403275	Discussion on MBS broadcast additional features for NR NTN Evolution	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2403306	On MBS Service Area Signalling in Rel-19 NTN	Nokia	discussion	Rel-19	NR_NTN_Ph3

R2-2403320	Support for broadcast service in NTN	InterDigital	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2403587	Discussion on support of broadcast service	ITL	discussion	Rel-19

R2-2403648	Discussion on Intended Service Area for NTN-MBS	NTT DOCOMO INC.	discussion	Rel-19

R2-2403650	Discussion on MBS service support for NR NTN	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.5	Support of regenerative payload

Contributions should focus on the needed updates for Stage 2 description.

R2-2402153	Stage-2 impact of regenerative payload in NR NTN	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402196	Discussion on stage-2 update on the support of regenerative payload	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402356	Discussion on support of regenerative payload in Rel-19 NR NTN	CATT, China Broadnet	discussion	Rel-19

R2-2402714	On support of regenerative payload in NTN	Lenovo	discussion	Rel-19

R2-2402808	Discussion on regenerative payload	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402818	Support of regenerative payload	NEC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2403092	Discussion on Regenerative NTN Architecture	TCL	discussion	Rel-19

R2-2403409	Discussion on Regenerative NTN Payload Architecture	TCL	discussion	Rel-19

R2-2403606	Regenerative NTN payload support in NR NTN Evolution	THALES, CATT, Huawei, ZTE, Inmarsat, Viasat	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2403639	Stage 2 updates for regenerative payload	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.6	LTE to NR NTN mobility

Support for idle mode mobility between LTE and NR NTN

R2-2402154	Support of LTE TN to NR NTN mobility	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402195	Discussion on LTE to NR NTN idle mode mobility	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402221	Discussion on LTE TN to NR NTN Mobility	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402545	Discussion on idle mode mobility enhancements for E-UTRAN TN to NR-NTN	CMCC	discussion	Rel-19

R2-2402809	Idle mode mobility from LTE to NR NTN	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2402827	Discussion on LTE TN to NR NTN mobility	Huawei, HiSilicon, Turkcell	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

R2-2402834	Discussion on the cell reselection from LTE to NR NTN	Xiaomi	discussion

R2-2402885	Mobility from LTE TN to NR NTN	Apple	discussion	Rel-19

R2-2403035	Support of Idle Mode Mobility from EUTRA TN to NR NTN	CATT	discussion

R2-2403066	Support for LTE to NR-NTN idle mode mobility	Telit Communications S.p.A. ; Thales	discussion

R2-2403073	Consideration on idle mode mobility between LTE TN and NR NTN	ZTE Corporation, Sanechips	discussion	Rel-19

R2-2403123	Discussion on support of LTE to NR NTN cell reselection	LG Electronics France	discussion	Rel-19	LTE_TN_NR_NTN_mob

R2-2403205	Discussion on LTE to NR NTN mobility	Interdigital, Inc.	discussion	Rel-19	LTE_TN_NR_NTN_mob

R2-2403226	Discussion on cell reselection from E-UTRA TN to NR NTN	MediaTek Inc.	discussion	NR_NTN_Ph3-Core

R2-2403307	On E-UTRA TN to NR NTN Mobility in IDLE mode	Nokia	discussion	Rel-19	NR_NTN_Ph3

R2-2403339	E-UTRAN TN to NR NTN mobility	Samsung	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

R2-2403640	E-UTRAN TN to NR-NTN mobility	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

## 8.9	IoT NTN Ph3

(IoT_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-240776)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.9.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2402941	Work Plan for Rel-19 IoT NTN 	MediaTek Inc.	Work Plan	IoT_NTN_Ph3-Core

R2-2403641	IoT NTN phase 3 scope	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

### 8.9.2	Support of Store & Forward

Contributions should focus on possible impacts to the radio interface.

R2-2403274	Discussion on Store and Forward support for IoT NTN Phase 3	THALES	discussion	Rel-19	NR_IoT_NTN_req_test_enh-Core

Proposal 1	Align regenerative payload architecture definition for IoT NTN with the NR NTN

Proposal 2	The following requirements should be considered for the RAN2 Store and Forward work

-	Store data on the satellite when the feeder link is unavailable; and forward the data once the feeder link between the satellite and the ground segment becomes available as illustrated on the figure above.

-	A 5G system with satellite access shall be able to inform a UE whether S&F Satellite operation is applied

-	Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to support forwarding of the stored data from one satellite to another satellite (e.g., which has an available feeder link to the ground network), through ISLs.

R2-2403689	RAN aspects of S&F operation for IoT NTN	Sateliot	discussion

Observation#1. Assuming an architecture with full eNB on board the satellite, it’s our view that support for S&F can be introduced with a minimum impact on the RAN functions excepting the broadcasting of system information (SI) intended to advertise UEs whether the network is operating in S&F mode or normal (default) mode.

Proposal#1. The information indicating whether/when a satellite cell is operating in S&F mode or normal/default mode should be broadcasted via system information.

R2-2402155	The consideration of supporting Store & Forward in IoT NTN	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Add satellite work mode into SIB to indicate whether S&F function is supported.

Proposal 2: RAN2 could focus on single satellite solution for S&F at first. For multi-satellite solution, RAN2 could wait for RAN3 conclusion on the UE context transmission.

Proposal 3: RAN2 needs to define the RRC connection suspend and resume mechanism for S&F function.

R2-2402541	Discussion on IoT NTN Store and Forward	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

Architecture

- Architecture

Proposal 1: It is proposed to specify single satellite architecture in S&F mode first. Waiting for SA2 conclusion on multiple satellite architecture in S&F mode (as well as ISL).

- Access control

Proposal 2: It is proposed that eNB broadcasts supporting S&F mode and carrying S&F information in SIB message (e.g. SIB31 or SIB32).

Proposal 3: It is proposed to add new IE in SIB1 (e.g. cellBarredSF-r18) and use one bit to indicate whether UE(s) are allowed to camp on the S&F cell.

Proposal 4: It is proposed to add new UE capability to support S&F operation.

Proposal 5: It is proposed to configure UE with S&F information in advance. FFS how UE is configured (e.g. through pre-configuration, broadcast message or dedicated signaling).

Proposal 6: RAN2 to discuss RA procedure enhancement. e.g. eNB may reject the first UE attach request with the cause value of S&F when the feeder link is unavailable.

Proposal 7: If the eNB rejects UE RA request, it can send a new waiting timer within the rejection message. UE does not need to send RA request until the timer expired.

Proposal 8: Before the waiting timer expires, UE can attach to a normal IoT-NTN cell if there is an available IoT-NTN satellite.

Proposal 9: Normal IoT-NTN cell has a higher priority than the S&F cell. Only when there is no available normal IoT NTN cell, UE supporting S&F mode will try to camp on the available S&F cell.

- Suspension

Proposal 10: Before the service link is unavailable, an indication needs to be gave from eNB to UE to indicate the interruption time. Then UE can know the estimated suspension time for next data transmission occasion.

Proposal 11: RAN2 to discuss UE behaviour during the suspension period e.g. expand the discard timer/retransmission timer due to S&F huge latency.

R2-2402942	RAN2 impact on S&F mode	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core

Proposal 1: Wait for SA2/CT1 discussion progress to see if the legacy UE can access the Rel-19 store and forward enabled network.

Proposal 2: The information indicating enabled Store and Forward mode can be broadcasted via system information.

Proposal 3: The information indicating feeder link status can be broadcasted via system information.

Proposal 4: The information indicating the time with feeder link can be broadcasted via system information.

Proposal 5: With the store and forward assistant information, UE can by implementation skip a cell during the cell selection/reselection or initiate RRC connection establishment at the right time.

R2-2402475	Overview of the Store and Forward satellite operation	Huawei, HiSilicon, Turkcell 	discussion	Rel-19	IoT_NTN_Ph3-Core

- Architecture and principle for S&F

Proposal 1: RAN2 should start with the regenerative payload architecture with at least eNB on board for the discussion of S&F satellite operation. Whether and which CN components are on board is up to SA2.

Proposal 2: RAN2 assumes:

-	The S&F satellite operation is common for NB-IoT and eMTC.

-	The S&F satellite operation is applied to both CP solution and UP solution.

-	Basic procedure for S&F satellite operation

Proposal 3: RAN2 to start from Modelling 1, i.e., UE is served by the same satellite/eNB after sending the RRC connection request and before being released to IDLE, and can discuss Modelling 2 after sufficient progress is made in RAN3/SA2.

Proposal 4: For the uplink/downlink messages transmission, the following steps/principles are taken as baseline for S&F operation:

1) The UE sends uplink data/signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/signalling to the CN.

3) The eNB receives the downlink data/signalling from the CN and stores it when feeder link is available.

4) The eNB sends the downlink data/signalling to the UE when service link is available again.

-	Access control for S&F satellite operation

Proposal 5: Legacy UEs should be barred by cellBarred and cellBarredNTN in a cell performing S&F satellite operation.

Proposal 6: UEs not supporting S&F should consider a cell performing S&F satellite operation as barred.

Proposal 7: A new S&F barring indication is broadcasted in the SIB.

-	UE’s RRC state when service link is not available

Proposal 8: RAN2 to discuss UE’s RRC state and the related behaviours when service link is becomes unavailable for S&F operation.

R2-2403148	Radio Interface Imapcts for Store-Forward mode operation of IoT-NTN	Nokia, Nokia Shanghai Bell	discussion

Proposal 1:  RAN2 to discuss the radio interface aspects for Store-and-Forward for eNB-only at Satellite and eNB+MME at Satellite until SA2 concludes on the supported components in NTN node for Store and Forward (S&F) operation.

Proposal 2:  RAN2 to identify the application scenarios that requires specific changes for the radio interface operation in S&F mode.

Proposal 3:  RAN2 to investigate access control and scheduling related changes for S&F operation.

Proposal 4:  RAN2 to analyze additional security requirements for handling control plane CIOT in Store-Forward mode of operation.

Proposal 5:  For User Plane solution for IoT-NTN in S&F mode, security handling of same UE context across two successive NTN nodes without direct connectivity needs to be investigated further.

R2-2402193	Discussion on Store & Forward satellite operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402222	RAN2 Aspects For Store & Forward	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402380	RAN2 impacts of supporting Store&Forward operation in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402454	Discussion on support of Store&Forward	Transsion Holdings	discussion	Rel-19

R2-2402693	Discussion on Store and Forward operations in IoT-NTN	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402710	Discussion on Store & Forward satellite operation IoT NTN	CAICT	discussion

R2-2402715	On support of Store and Forward operations in NTN	Lenovo	discussion	Rel-19

R2-2402810	S&F satellite operation with full eNB as regenerative payload	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402819	Support  of Store and Forward	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402821	Considerations on Store & Forward Satellite Operation	SHARP Corporation	discussion	Rel-19

R2-2402835	Initial views on the support of store and forward satellite operation	Xiaomi	discussion

R2-2402886	Support of S&F operation in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2403044	Discussion on support of store and forward operation	CATT	discussion

R2-2403321	Support for Store and Forward operation in NTN	InterDigital	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2403337	On RAN2 aspects of Store and Forward	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

### 8.9.3	Uplink Capacity Enhancement

At this meeting contributions should only focus on the possible enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction (Msg3 transmission without msg1/RAR; efficient delivery of msg4 / RRCEarlyDataComplete).

R2-2403206	Clarifications on the Scope of EDT enhancement for IoT-NTN	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Clarify whether the scope of uplink capacity enhancements is limited to NB-IoT, or whether eMTC is also within scope.

Proposal 2: Clarify whether the scope of uplink capacity enhancements is only for C-plane optimization, or whether U-plane optimization is also within scope.

Proposal 3: Clarify whether the scope of uplink capacity enhancements is only for CIoT EPS optimizations, or whether CIoT 5GS optimizations are also within scope.

Proposal 4: Clarify that MO-EDT and MT-EDT can utilize uplink capacity enhancements, however no MT-EDT specific enhancement is in scope.

Proposal 5: Clarify whether the scope of uplink capacity enhancements is only for NTN, or whether enhancements may apply to TN.

Proposal 6: Clarify the scope of uplink capacity enhancements study is to

-	Identify shortcomings with the existing EDT/PUR feature for NTN.

-	Identify potential enhancements to the existing EDT/PUR feature.

-	Show the gain of potential enhancements compared to the existing EDT/PUR

R2-2402381	Consideration on enhanced early data transmission in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: RAN2 clarify that the main task of the objective “to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction” is to support contention-based Msg3 transmission or contention-based Shared PUR.

Proposal 2a: It’s suggested that, if an IoT NTN UE in IDLE state is activated to use R19 contention-based Shared PUR, the UE needs to verify/update the TA just before triggering PUR.

Proposal 2b: If the proposal 2a can be agreed, RAN2 further discuss whether the pur-TimeAlignmentTimer is no longer needed.

Proposal 3: It’s suggested that, if contention-based Shared PUR is supported, when the UE changes the camping cell, the UE doesn’t need to stop using this enhanced PUR function.

Proposal 4a: With reference to legacy PUR configuration, RAN2 can take the following resources types as start point for the discussion on shared PUR resources configuration:

•	Time domain resources, e.g., Periodicity, Offset, start time (H-SFN, frame, SFN etc.) of PUR occasion

•	Frequency domain resources, e.g., Carrier, SubCarrier

•	TBS

•	PUR MPDCCH/NPDCCH search space window

•	RSRP change threshold

•	OCC resource

Proposal 4b: RAN2 is suggested to open discuss the following alternatives for configuring shared PUR resources:

•	Alt1: To provide a common PUR resource configuration via SIB.

•	Alt2: To provide shared PUR resources via dedicated signaling. It can be allowed to configure some different resources for different UEs while the other resources may be same for all the UEs.

•	Alt3: To provide multiple sets of shared PUR resources via the SIB and NW can indicate index of the resource set for a specific UE via the dedicated signaling.

Proposal 5: RAN2 is suggested to open discuss the following alternatives for RNTI design and the scheduling of Msg3:

•	Alt1: Common PUR-RNTI configured in the common PUR resources configuration.

•	Alt2: UE-specific PUR-RNTI allocated by NW that is similar as the one for dedicated PUR.

•	Alt3: Resource-specific PUR-RNTI which can be respectively calculated by UE and eNB according to the resources used/selected by the UE.

Proposal 6: The existing types of “Msg4” for normal PUR process, e.g., Physical layer L1 ACK, Timing Advance Command MAC CE, and RRCEarlyDataComplete/RRCConnectionRelease/RRCConnectionSetup/ RRCConnectionResume, need to be supported for shared PUR scheme. The very simple Physical layer L1 ACK can be used in suitable cases to achieve the expected more efficient Msg4 transmission.

Proposal 7: RAN is suggested to consider the following alternatives for the scheduling of Msg4:

•	Alt1: Common PUR-RNTI configured in the common PUR resources configuration.

•	Alt2: UE-specific PUR-RNTI allocated by NW that is similar as the one for dedicated PUR.

•	Alt3: Resource-specific PUR-RNTI which can be respectively calculated by UE and eNB according to the resources used/selected by the UE.

Proposal 8: The contention resolution scheme in legacy random access procedure can be reused for shared PUR, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 9: RAN2 is suggested to discuss the following other issues:

•	Issue#1: How to handle the coexistence of PUR and contention-based shard PUR

•	Issue#2: Whether a backoff scheme is needed when Msg3 collision occurs

R2-2402703	Discussion on uplink capacity enhancements for IOT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1	For Msg3-based EDT, RAN2 to take OCC based CFS-PUR and OCC based CBS-PUR as a start point.

Proposal 2	CBS-PUR resource can be configured through system information, which doesn’t require TAT configuration.

Proposal 3	For CBS-PUR, resource request is not supported.

Proposal 4	RAN2 to consider the design of RNTI for OCC based CBS-PUR.

Proposal 5	RAN2 to consider reuse the legacy mechanism of including first 48 bits of CCCH SDU in msg4 for contention resolution.

Proposal 6	For EDT enhancement, assume that at most  one downlink packet and one uplink packet are supported.

Proposal 7	RAN2 to discuss whether UE can fallback to legacy EDT/PUR procedure when OCC based CBS-PUR procedure fails multiple times.

R2-2402202	Discussion on enhanced EDT for IoT NTN	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402223	Discussion on EDT Enhancement for IoT-NTN	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402336	Uplink Capacity Enhancement for EDT transaction	Spreadtrum Communications	discussion	Rel-19

R2-2402476	Overview of capacity enhancement for uplink	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402546	Discussion on early data transmission enhancements for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402694	Discussion on EDT optimisation in IoT-NTN	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402709	Discussion on enhanced EDT of IoT NTN	CAICT	discussion

R2-2402716	EDT for uplink capacity enhancement in NTN	Lenovo	discussion	Rel-19

R2-2402811	Discussion on EDT enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402887	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2402943	Discussion on enhanced EDT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core

R2-2403042	On signalling overhead reduction for EDT in IoT NTN	CATT	discussion

R2-2403126	Consideration on EDT enhancement for IoT-NTN	NEC Corporation.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2403338	Initial discussions on uplink capacity enhancements	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2403483	On uplink capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

# Summary

Agreed CRs

NR-NTN

IoT-NTN

Approved LSs out

[Post125bis] Email discussions

Short

Medium

Long