3GPP TSG-RAN WG2 Meeting #125bis	R2-2402556

# Changsha, China, April 15th – 19th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | - | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RSCP measurement info in PRU DL info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For UE-based positioning, LMF may provide target UE with the PRU measurement info, e.g., RSCP/RSCPD measurements. In TS 38.214, the RSCP/RSCPD measurements are always sent along with the legacy measurements. The UE may be provided with [nr-PruInformation-Ue-based-DL-CPP ] which contains DL RSCP/RSCPD measurements together with DL RSTD, DL PRS-RSRP, and/or DL PRS-RSRPP measurement(s) associated with the RSCP/RSCPD measurements performed by a positioning reference unit (PRU) [20, TS 38.305] the timestamps associated with the measurements, and the location information of the PRU.. However, in the LPP spec, the RSCP-MeasInfo can be provided independently. As the RSCP/RSCPD measurement info is already included in the NR-DL-TDOA-SignalMeasurementInformation, the nr-PRU-RSCP-MeasInfo in NR-PRU-DL-Info can be deleted. |
| Summary of change: |  | Delete the nr-PRU-RSCP-MeasInfo in NR-PRU-DL-Info. Impact analysis Impacted functionality: PRU in carrier phase positioning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RSCP-MeasInfo can be provided independently, which is not aligned with TS 38.214. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

6.4.3	Common NR Positioning Information Elements

< omitted>

NR-PRU-DL-Info

The IE NR-PRU-DL-Info is used by the location server to provide the carrier phase measurements with associated measurements and additional information reported by a PRU for UE-based DL-TDOA to a target UE.

-- ASN1START

NR-PRU-DL-Info-r18 ::= SEQUENCE {

nr-PRU-LocationInfo-r18			LocationCoordinates						OPTIONAL, -- Need ON

nr-PRU-DL-TDOA-MeasInfo-r18		NR-DL-TDOA-SignalMeasurementInformation-r16

OPTIONAL, -- Need ON

nr-PRU-DL-AoD-MeasInfo-r18		NR-DL-AoD-SignalMeasurementInformation-r16

OPTIONAL, -- Need ON

...

}

-- ASN1STOP

NR-PRU-DL-Info field descriptions

nr-PRU-LocationInfo This field provides the location coordinates of the PRU.

nr-PRU-DL-TDOA-MeasInfo This field specifies the list of carrier phase measurement RSCPD together with the other measurement information in DL-TDOA by the PRU.

nr-PRU-DL-AoD-MeasInfo This field specifies the list of other measurement information in DL-AoD by the PRU.

End of change