3GPP TSG-RAN WG2 Meeting #126	R2-2404291

Fukuoka, Japan, 20 – 25 May 2024

Agenda item:	8.7.1

Source:	Nokia, Qualcomm (Rapporteurs)

Title:	RAN3 Overview

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Overview

The following topics have been discussed in RAN3:

-	Bearer types for NR-DC: RAN3 agreed to support MN-terminated MCG bearer, SN-terminated SCG bearer, MN-terminated SCG bearer, and SN-terminated MCG bearer. For split bearer, the main discussion was how to support the split bearer in case of UL PSI based discarding. In the end, RAN3 agreed to support split bearer but not consider UL PSI based discard coordination between MN and SN.

-	PDU Set QoS based handling: RAN3 agreed to describe the behaviour of handling the PDU Set QoS Parameters IE in S-NG-RAN node Addition Preparation procedure and M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure. RAN3 also reached a working assumption that SN reports the PDU Set based Handling Indicator in S-NG-RAN node Addition Preparation procedure and M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure for the MN-terminated SCG bearer, SN-terminated MCG bearer and SN-terminated SCG bearer. RAN3 will discuss the working assumption at the next meeting (i.e. whether non-homogenous DC is supported or not and whether the indicator is actually needed).

-	PSI based discard: RAN3 agreed that the PDCP hosting gNB informs the corresponding gNB about whether the UL PSI based SDU discarding is (re)configured/released for MN-terminated SCG bearer and SN-terminated MCG bearer over XnAP. UL PSI based discard coordination for split bearer between MN and SN will not be considered for NR-DC. FFS whether MN/SN can inform SN/MN about the (de)activation of the UL PSI based discard for the split bearer. FFS on whether and how to support DL PSI based discard for NR-DC.

-	ECN Marking and Congestion Information exposure: RAN3 agreed to enhance the S-NG-RAN node Addition Preparation procedure and M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure, to transfer the ECN Marking or Congestion Information Reporting Request IE to S-NG-RAN node, and support the S-NG-RAN node to report ECN Marking or Congestion Information Reporting Status to M-NG-RAN node. There were some discussions on whether the ECN Request/Status should be per DRB or per QoS Flow. RAN3 agreed to use per DRB level request/status.

The corresponding endorsed CRs for TS 37.340 and TS 38.423 can be found in R3-242192 and R3-242241 respectively.

# 2	Conclusion

For the Rel-19 work on XR, RAN3 discussed and made new agreements on bearer types for NR-DC, PDU Set QoS based handling, PSI based discard and ECN marking. From RAN2 viewpoint, the following is worth noting:

-	UL PSI based discard coordination for split bearer between MN and SN will not be considered for NR-DC;

-	FFS whether MN/SN can inform SN/MN about the (de)activation of the UL PSI based discard for the split bearer;

-	FFS on whether and how to support DL PSI based discard for NR-DC.