# 3GPP TSG RAN WG2 Meeting #126      	                             R2-2404662
Fukuoka, Japan, May 20-24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0036 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for egress link determination in L2 U2U Relay UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current spec, the egrees link determination procedure text is specifed in a way that the L2 U2U relay UE determining the egress link solely depends on the UE IDs, w/o considering the ingress PC5 link. This has put a stricter requirements on UE ID allocaiton that every UE ID pair is allocated per directional end-to-end link and cannot be reused by the other direction, or any other e2e links. In other words, the local ID pairs assigned to each remtoe UE must be completely orthogonal. However, RAN2 only agrees that the local ID allocaiton is completely up to relay UE implemetation, and has not agreed on the above restriction. RAN2#123bis agreement: The UE ID assignment for U2U remote UEs is up to U2U relay UE implementation, i.e., no specification impact on how to assign the local ID is needed.. To fix this error, we think the current procedure text of egress link determination needs to be relaxed so that the relay UE only need to check against the local ID allocaitons towards a single remote UE without checking all the local ID configurations acrossing all the e2e links. |
| Summary of change: |  | - Added “configured to the U2U remote UE“ to indicate that only configuration related to source L2 U2U remote UE is relevant in the matching check. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some local ID allocation scheme will be unnecessarily invalidated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3a.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: added changes 2 - 6 |  |  |  |  |  |  |  |  |

Being of Change

#### 5.3a.3.2	Egress link determination

For a U2U SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If there is an entry in sl-LocalID-PairToAddModList configured to the U2U Remote UE, in which the sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity match the UE ID fields in the U2U SRAP Data PDU:

-	Determine the egress link on PC5 interface towards the peer U2U remote UE identified by sl-PeerRemoteUE-L2Identity configured for the concerned sl-PeerRemoteUE-LocalIdentity as specified in TS 38.331 [3].

End of Change