3GPP TSG-RAN WG2 #126 	R2-2405197

Fukuoka, Japan, May 20th – 24th, 2024

Agenda Item:	8.9.2 Support of Store & Forward

Source:	NEC

Title:	support of store and forward

Document for:	Discussion, Decision

# 1.Introduction

Objective of support store and forward satellite operation is as following:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

This topic was discussed for the first time during last RAN2 meeting, and we made following agreements:

1. S&F implies that at least the full eNB will be onboard 2. An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this) 3. The S&F satellite operation is common for NB-IoT and eMTC. 4. The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

In this contribution, we share our view on capability, scenario and procedure aspects.

# 2 Discussion

## 2.1 Capability

In WID justification, it is mentioned the benefits associated with the support of Store and Forward (S&F) Satellite operation:

- S&F operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas)

- Along with support for discontinuous coverage, support for S&F operation is key to facilitate cost-effective deployment of IoT NTN services and enable an immediate operational service with (1) sparse LEO constellations and (2) reduced ground segment infrastructure.

- The support of S&F puts the 3GPP IoT NTN solution on par with other non-3GPP solutions intended for massive satellite IoT, which natively already supports S&F services

From above justification, a common scenario would be

- From gateway and core network perspective, there is no continuous feeder link availability.

- from UE /service link perspective, There is likely no enough satellite/cells to provide a continuous coverage, which means UE/NW support store and forward need to support discontinuous coverage feature.

Proposal 1 : UE who support store&forward feature, shall support discontinuous coverage feature

## 2.2 Scenarios

Even though SA2 is discussing one and multiple satellite scenarios, it sounds too restrict to limit to one satellite scenario.  At least from network perspective, there is likely more than one eNB providing the store&forward service together. This means a UE could see one satellite/eNB at this time but see another satellite/eNB at another time. An example scenario is shown in below, there are two eNBs flying around with only one NTN gateway infrastructure on ground to support store and forward service, the same UE could see/connect to eNB1 and eNB2 sequentially:

Figure 1: Example of two satellite/eNBs circulate around for “store and forward” service

On the other hand, considering there is no continuous full CN connection, a NAS signaling or MO data would need to be sent and stored at satellite when UE firstly connects to a eNB, the responding NAS signaling or data would need to be transmitted back to UE when UE secondly connects to a eNB. It would be easier if a UE connect to the same eNB at the first and second time. More consideration e.g., UE context fetching would be needed if at the first time and second time, the UE connects to different satellites.

Proposal 2: RAN2 discuss if UE should be encouraged to reconnect to the same satellite/satellite sets if there is unfinished store and forward data/signalling transmission.

Proposal 3: RAN2 discuss multiple satellites scenario, where a given UE’s camping cell/serving cell could be different at different time window

## 2.3 Procedure

Following figure shows our understanding on the store and forward procedure based on CP CIOT optimisation feature:

Data transport in NAS PDU

We focus on signalling flow over Uu interface, which happens while service link is available but feeder link is not available:

- eNB may send paging message to the UE, if the eNB has stored DL data and the UE may be in its coverage

- the UE initiates RRC connection for uplink data transmission or in response to paging

- the UE transmits NAS data PDUs if any to eNB, eNB store them, without trigger S1-AP procedures immediately .

- eNB forward stored downlink data if any to the UE.

- When there is no more data in both downlink and uplink, release the UE’s RRC connectivity

In summary, data encapsulated in (initial and following) NAS messages can be sent to eNB via RRC signalling without a UE context established. Because the data transmission can be processed before the UE context establishment, this will less dependent on feeder link availability. And the procedure over the Uu interface could be reused as baseline.

In another hand, lack of UE context and full CN connection, there is a risk of misusing. RAN2 may discuss the limitations/restrictions to use this feature, e.g., only a group of UE could be allowed to use a particular satellite/eNB for data and forward, only limit amount data could be stored and forward via CP-CIOT procedure:

Proposal 4: Store and forward based-on CP-CIOT procedure,

  - The procedure over the Uu interface could be reused as baseline

  - RAN2 discuss how to avoid that a UE misuses/overuses CP-CIOT based store and forward

Following figure shows our understanding on the store and forward procedure based on UP CIOT optimisation procedure:

Data transport via resumed DRB

A suspended UE context with configured DRB(s) can be resumed for data transmission. Opposite to CP-CIOT, the data transmission will rely on stored UE context at UE and network side. this requests UE context establishment and maintenance, which might become very complicated when feeder link and service link are not available at the same time. moreover, if the UE connects to a eNB different from the anchor eNB, context retrieving is needed, without available feeder link, this will cause further complication and delay. To be simpler, it would be better that the UE connect to the same eNB where its UE context was suspended.

With this option, same as CP solution, potentially the procedure over Uu interface could be reused as baseline without big change. And it can support larger data transmission than CP-CIOT

Proposal 5: Store and forward based-on UP-CIOT procedure,

  - The procedure over the Uu interface could be reused as baseline

  - UE prioritizes the eNB where its UE context was suspended for RRC connection resume

# 3 Summary

This contribution provided our initial view on support “S&F” feature as follows:

Proposal 1: UE who support store&forward feature, shall support discontinuous coverage feature

Proposal 2: RAN2 discuss if UE should be encouraged to reconnect to the same satellite/satellite sets if there is unfinished store and forward data/signalling transmission.

Proposal 3: RAN2 discuss multiple satellites scenario, where a given UE’s camping cell/serving cell could be different at different time window

Proposal 4: Store and forward based-on CP-CIOT procedure,

  - The procedure over the Uu interface could be reused as baseline

  - RAN2 discuss how to avoid that a UE misuses/overuses CP-CIOT based store and forward

Proposal 5: Store and forward based-on UP-CIOT procedure,

  - The procedure over the Uu interface could be reused as baseline

  - UE prioritizes the eNB where its UE context was suspended for RRC connection resume