3GPP TSG-RAN2 Meeting #126	R2-2405485

Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4839 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the RSRP measurement for SDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2024-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the RAN2#116-e meeting, RAN2 made the following agreement: - Highest N SSBs of all SSBs actually transmitted as indicated in SIB1 is used for RSRP based TA validation. According to section 5.7.17 of 38.331, the downlink pathloss reference RSRP for SDT is derived from the beam measurement quantity. For the INACTIVE UE configured with SDT, the UE can be configured with ssb-PositionsInBurst in SIB1 and sdt-SSB-Subset-r17 in RRCRelease. It is unclear whether the UE uses “ssb-PositionsInBurst in SIB1” or “sdt-SSB-Subset-r17 in RRCRelease” to derive the downlink pathloss reference RSRP for SDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.7.17, a NOTE is added to clarify that the the downlink pathloss reference RSRP for SDT is derived from the SSB configured via SIB1. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: SDT Inter-operability: If the network implements the CR and the UE does not, there is no inter-operability observed. If the UE implements the CR and the network does not, there is no inter-operability observed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear whether the UE uses the SSB configured by “ssb-PositionsInBurst in SIB1” or the SSB configured by “sdt-SSB-Subset-r17 in RRCRelease” to derive the downlink pathloss reference RSRP for SDT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 5.7.17	Derivation of pathloss reference for TA validation of SRS for Positioning transmission and CG-SDT in RRC_INACTIVE

Upon request from lower layer for pathloss reference derivation for TA validation for SRS for Positioning transmission or CG-SDT in RRC_INACTIVE, the UE shall:

1>	acquire SIB2, if stored version is invalid;

1>	if nrofSS-BlocksToAverage or absThreshSS-BlocksConsolidation is not present or if absThreshSS-BlocksConsolidation is present and the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation:

2>	derive the downlink pathloss reference RSRP for TA validation as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [24];

1>	else:

2>	derive the downlink pathloss reference RSRP for TA validation as the linear average of the power values of up to nrofSS-BlocksToAverage of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation, where each beam measurement quantity is described in TS 38.215 [24].

NOTE 1: The downlink pathloss reference RSRP for TA validation is derived from the SSB configured by SIB1.

END OF CHANGE