3GPP TSG-RAN WG2 Meeting #126	R2-2405511

Fukuoka, Japan, 20th – 24th May 2024

Agenda Item:	7.25.1.7

Source:	Huawei, HiSilicon

Title:	Discussion on 3Tx SAR for inter-band CA with PC1.5

Document for:   Discussion and decision

Introduction

Currently, the UE capability maxUplinkDutyCycle-interBandCA-PC2 is used to indicate the duty threshold for PC2 inter-band UL CA combinations. According to RAN4 LS[1], the capability will be re-used by RAN4 for 3Tx inter-band UL CA with PC1.5. RAN2 was asked to provide the feedback on the feasibility to reuse the existing capability, and update the RAN2 specs accordingly if no technical problem is found.

In this contribution, we provide our views on questions from the RAN4 LS.

- Question#1: From RAN2’s view, is there any technical problem of reusing the capability of maxUplinkDutyCycle-interBandCA-PC2 to the SAR solution in PC1.5? - Question#2: If there is no other technical problem in Q1, would RAN2 consider the approach above as a reference to update TS38.306?

Discussion

To ensure the SAR requirements, the UE capability maxUplinkDutyCycle-interBandCA-PC2 was used to indicate the maximum percentage of symbols during the evaluation period that can be scheduled for uplink. According to the RAN4 LS[1], before Rel-18, there is only PC2 inter-band UL CA, so the field is only used for the inter-band band combinations with PC2. Now in Rel-18, RAN4 has specified 3Tx band combinations, which is able to perform PC1.5 transmission with either UL MIMO or Tx Diversity in one band. To ensure the SAR requirement for such 3Tx band combinations, RAN4 has agreed to extend the applicable scenario of the existing capability maxUplinkDutyCycle-interBandCA-PC2 to PC1.5 inter-band UL CA. From RAN2 perspective, this would bring a change on the capability definition in TS38.306.

In our view, since the correction is defined for completely new band combinations (i.e. PC1.5 inter-band UL CA combinations), there will be no backward compatible issue. In other words, there is no legacy NW supporting such PC1.5 inter-band UL CA combinations, so the updated capability signalling will cause no impact to the legacy NW. For the PC1.5 inter-band CA combination signalled by a new UE, only the upgraded gNB will identify the combination and follow the updated requirement of the capability defined in RAN2 and RAN4 specs. Thus, we see there is no NBC issue and no technical problem to reuse the existing capability of maxUplinkDutyCycle-interBandCA-PC2 to the SAR solution in PC1.5 inter-band UL CA.

Proposal 1: RAN2 confirms there is no technical problem and no NBC to reuse the existing capability of maxUplinkDutyCycle-interBandCA-PC2 to the SAR solution in PC1.5 inter-band UL CA.

To reuse the existing signalling for PC1.5 inter-band CA, we see following corrections are needed for RAN2 spec. First, in the field description of maxUplinkDutyCycle-interBandCA-PC2, the reference to the RAN4 spec should be extended to cover the applicable PC1.5 band combinations and requirements defined in TS 38101-1. Besides, when the capability field is absent, the default behaviour is not to work on PC2, but to work on the actual power class supported by the UE based on the reported power class for the BC, i.e. either PC2 or PC1.5. Lastly, since the field name includes “PC2”, a note in the field description is suggested to be added to clarify that this field is for both power class2 and power class 1.5.

Since RAN4 has agreed that such 3Tx inter-band UL CA will be supported from Rel-17 in release independent manner, the corrections on the capability should also start from Rel-17 in RAN2 spec. Corresponding 38.306 CRs are provided in [2][3].

Proposal 2: Agree the TS38.306 CRs in [2][3] to update the field description of maxUplinkDutyCycle-interBandCA-PC2 for PC1.5 inter-band UL CA.

Conclusion

In this contribution, we have following proposals:

Proposal 1: RAN2 confirms there is no technical problem and no NBC to reuse the existing capability of maxUplinkDutyCycle-interBandCA-PC2 to the SAR solution in PC1.5 inter-band UL CA.

Proposal 2: Agree the TS38.306 CRs in [2][3] to update the field description of maxUplinkDutyCycle-interBandCA-PC2 for PC1.5 inter-band UL CA.

References

- R4-2406579, Draft LS on 3Tx SAR solution for inter-band CA with PC1.5.

- R2-2405512, Correction on 3Tx SAR solution for inter-band CA with PC1.5.

- R2-2405513, Correction on 3Tx SAR solution for inter-band CA with PC1.5.