3GPP TSG-RAN2 Meeting#129	R2-2500114

# Athens, Greece, Feburary 17th - Feburary 21st, 2025

Agenda Item:	7.6

Source:	OPPO

Title:	Remaining issues for MCSt

Document for:	Discussion, Decision

- Introduction

This is to discuss the left issue on MCSt.

R2-2410612	Stage 2 Corrections on MCSt	InterDigital Inc, Apple	CR	Rel-18	38.300	18.3.0	0939	-	F	NR_SL_enh2

Check what was agreed/implemented in RAN2, whether both approach 1 and approach 2 (regarding MCSt and PCFCH) are enough covered, offline.

CR will be revised in R2-2410943

[LG, Vivo]: To be more accurate, it should be restricted to single TB case. [OPPO]: Not sure if it is clear whether there is the restriction in MAC spec. If we don’t have such restriction in MAC, this CR should be ok. [Huawei]: In MAC, there is a case of MCSt where ACK is used and assume that it is for the multiple TB case. [Qualcomm]: Have different understanding. We agreed with no MCSt for both single TB case and multiple TB case, with PSFCH. [Qualcomm]: In addition, the current MAC is not clear on the differentiation of approach 1 and approach 2.

[AT128][112][V2X/SL] (ZTE)

Scope: Check what was agreed/implemented in RAN2, whether both approach 1 and approach 2 (regarding MCSt and PSFCH) are enough covered, whether we need to update MAC, actual MAC changes if needed, and attempt to reach RAN2 consensus.

Intended outcome: Discussion summary in R2-2410945. F2F offline discussion schedule will be announced by offline discussion rapporteur.

Deadline: Comeback in CB session.

R2-2410945	Discussion on MCSt	ZTE Corporation, Sanechips	Discussion	NR_SL_enh2

Proposal1: Case 1c is supported, that is N=1 or no value N is indicated to PHY layer for sensing, for a resource pool configured with PSFCH resource, MAC layer may select consecutive slots for transmissions of multiple TBs, if minimum time gap for HAQR feedback is ensured.

Postponed.

[Qualcomm]; PSFCH needs 4 symbols. Gap should be less than 16us/25us for MCSt. [Session Chair]: Question is if a resource pool with PSFCH is supported in RAN1? [Huawei]: Understand it is clear PSFCH with 4 symbols between two TBs will not impact MCSt in RAN1. Should be allowed. [OPPO]: Think it is good to postpone the discussion and it will be good to see the corresponding spec impacts if we allow case 1c. [Ericsson]: If 1c is related only to approach 1 then since approach 1 is done based on best effort manner, if the UE receives ACK in the middle of MCSt, then another solution is that the UE can stop MCSt operation for the remaining slots.

R2-2410943	Stage 2 Corrections on MCSt	InterDigital Inc, Apple	CR	Rel-18	38.300	18.3.0	0939	1	F	NR_SL_enh2

Postponed.

- Discussion

During Rel-18, what 3GPP concluded for MCSt is as follows.

As captured in R1-2308664, R1 agreed that

Agreement

In Mode 2 resource allocation,

- The higher layer can indicate a “number of consecutive slots for MCSt” (Nslot,MCSt) larger than 1 for L1 reporting multi-slots candidates to the higher layer. The candidate multi-slots resource definition is applied.

  - Otherwise, the candidate single-slot resource definition is applied (same as R16/17).

- The higher layer selects resources from the reported SA according to one of the following based on UE implementation:

  - Random selection as per R16/17

  - Higher layer is not restricted to select resources at random, and can select in consecutive slots

    - It is up to RAN2 to define detailed behaviour as needed

  - It is RAN1 intention that, once the higher layer selects a multi-slots candidate from the set SA, it will use all the single-slot resources of the selected multi-slots candidate for transmission. This RAN1 agreement has no intention on potential RAN2 discussion about how SL resource selection processes are defined in MCSt.

- Note, the above is intended to support Approach 1 and 2 only.

Where for the definition of approach-1 and 2, they are defined in R1-2304257

Approach 1: “best effort for multiple TBs”

- Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) - R16/17 behavior.

- Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior.

- Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt.

- Step 4: Repeat Step 1-3 for different TB if required.

Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs”

- Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means.

- Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change)

- Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior).

- Step 4: Repeat Step 1-3 for different TB if required.

When R2 ruled out the resource pool with PSFCH for MCSt of same-TB, it was mainly because it goes against the minimum gap requirement for feedback-based SL transmission, as captured in R2-2309511.

Some other left issues:

- Send an LS to RAN2 informing that it is up to RAN2 to decide in regards to the HARQ RTT timing (minimum time gap)

whether a single TB transmitted over consecutive slots is supported in a resource pool configured with PSFCH resource

Firstly, upon the usable of MCSt, how to handle the minimum gap requirement (a + b) for data requiring HARQ feedback

6>	randomly select the time and frequency resources for one or more transmission opportunities from the available resources which occur within the SL DRX Active time if configured as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL-CSI by ensuring the minimum time gap between any two selected resources in case that PSFCH is configured for this pool of resources, and that a retransmission resource can be indicated by the time resource assignment of a prior SCI according to clause 8.3.1.1 of TS 38.212 [9];

For which, the view from contribution ias follows

R2-2307145, Proposal-9: Blind Retransmission of the same TB is allowed for MCSt based transmission.	NEC

Q2-2: In order to support MCSt, do you agree to support “a single TB transmitted over consecutive slots is supported in a resource pool configured with PSFCH resource”?

- Yes

- No

While for multi-TB scenario, since the consecutive slots are used for different TBs, the minimum gap requirement is not violated, which can be implied based on the following R2#124 conclusion

1. For remaining slot(s) in case transmission is successful for one TB in MCSt (multiple TB case), the UE still performs retransmission for this TB in the remaining slot(s).

R2 conclusion ruled out RP with PSFCH for same-TB MCSt, but not for multi-TB MCSt. For which R1 agreed to support via approach-1 and approach-2.

While during 128 meeting, another point was raised, i.e., for multi-TB case, it is hard to reach MCSt gain, i.e., consecutive transmission anyway, due to the PSFCH existence.

R2-2410945	Discussion on MCSt	ZTE Corporation, Sanechips	Discussion	NR_SL_enh2

Proposal1: Case 1c is supported, that is N=1 or no value N is indicated to PHY layer for sensing, for a resource pool configured with PSFCH resource, MAC layer may select consecutive slots for transmissions of multiple TBs, if minimum time gap for HAQR feedback is ensured.

Postponed.

[Qualcomm]; PSFCH needs 4 symbols. Gap should be less than 16us/25us for MCSt. [Session Chair]: Question is if a resource pool with PSFCH is supported in RAN1? [Huawei]: Understand it is clear PSFCH with 4 symbols between two TBs will not impact MCSt in RAN1. Should be allowed. [OPPO]: Think it is good to postpone the discussion and it will be good to see the corresponding spec impacts if we allow case 1c. [Ericsson]: If 1c is related only to approach 1 then since approach 1 is done based on best effort manner, if the UE receives ACK in the middle of MCSt, then another solution is that the UE can stop MCSt operation for the remaining slots.

Our view on this point is

- It is true that when there is a PSFCH between two slots, the CPE cannot secure 16us gap in this case, and thus there is a possibility that the consecutive transmission fails due to LBT failure. But, for RP with PSFCH, there are still consecutive slots without PSFCH in-between.

- On the other hand, since MAC layer perform random selection for each HARQ process, nothing prevents the UE to select consecutive slots for two different TBs by coincidence.

- Furthermore, for the term “MCSt”, R1 did not intend to always aim at transmission with 16us gap. It merely refers to transmission with “consecutive slots”. R1 uses the term “SL transmission burst” instead to refer to the transmission with “16us gap”.

Agreement

A sidelink transmission burst is defined as a set of SL transmissions from a UE without any gaps greater than 16us. Transmissions from a UE separated by a gap of more than 16us are considered as separate sidelink transmission bursts. A UE can transmit SL transmission(s) after a gap of up to 16us within a sidelink transmission burst without sensing the corresponding channel(s) for availability.

MAC spec cannot prevent consecutive slots for multi-TB case due to the random resource selection per HARQ process.

R1 did not intend to secure MCSt transmission without >16us gap, for which a different term “SL transmission burst” was used.

For resource pool with PSFCH, there are still consecutive slots without PSFCH in-between where “SL transmission burst” can be achieved.

These points lead to a conclusion that R2 does not need to prevent the MCSt case for multi-TB in resource pool with PSFCH.

- R2 keep the conclusion of allowing MCSt for multi-TB in resource pool with PSFCH, and not pursue optimization for the case of PFSCH between two selected consecutive slots.

- Conclusion

Proposal 1	R2 keep the conclusion of allowing MCSt for multi-TB in resource pool with PSFCH, and not pursue optimization for the case of PFSCH between two selected consecutive slots.