3GPP TSG-RAN WG2 Meeting #129	R2-2500216

Athens, Greece, Feb 17th – 21st, 2025

Source:	vivo

Title:	Simulation results of measurement event prediction and SLS

Agenda Item:	8.3.3

Document for:	Discussion and Decision

- Introduction

At the RAN2#128 meeting[1], RAN2 agreed on the simulation assumption for the measurement event prediction. In addition, the handover modeling was provided for system-level simulation.

Agreements on measurements events 1 Agreed values for Case A. NOTE1 indirect prediction only Parameters; baseline value; Note. A3 event offset (db); 2; Open for 3db. TTT (ms); 320; Open for one shorter value. UE speed (km/h); 90; Open for 60 and 120km/h. OW length (ms, note1); N/A; Up to implementation. PW length (ms, note1); 400; Open for more values. Max ETD (ms, note1); 80; Open for more values. 2 Agreed values for Case B. NOTE1 indirect prediction only Parameters; baseline value; Note. A3 event offset (db); 2; Open for 3db. TTT (ms); 320; Open for one shorter value. UE speed (km/h); 30; Open for 60 and 90km/h. OW length (ms,note1); N/A; Up to implementation. PW length (ms,note1); 200 (non-sliding) 40ms (sliding); Open for more values. Max ETD (ms, note1); 80; Open for more values. MRRT; 50%; Open for more values. 3 Company can report which filtering options is being used for the input L3 RSRP of sub-use case 2: option 1, option 2, option 3. 4 to capture the 3 options into TR

In this contribution, we will provide simulation results of indirect measurement event prediction for both temporal domain case A and case B, and the corresponding system-level performance results.

- Discussion

  - Temporal domain Case A

Based on the agreed observation, for the FR2 intra-frequency temporal domain Case A, sub-use case 2 achieves better performance than sub-use cases 1 and 3. Thus we carry out the measurement event prediction simulation with sub-use case 2 for Case A. Some key simulation parameters are shown in the following table:

Table 2.1-1: Key parameters of temporal domain prediction Case A

| Parameter | Values |
| --- | --- |
| Sub-use case | 2 |
| L1/L3 filtering | Sliding |
| L3 filtering | Option 1 |
| OW length | 5*80=400 ms |
| PW length | 5*80=400 ms |
| Pattern |  |

As to the measurement event specific parameters, most of the baseline values in Table 2.1-2 are selected in the simulation. To analyze the impact of RRM prediction accuracy on the event prediction, all the candidate UE speeds are simulated.

Table 2.1-2: measurement event specific parameters for Case A

| Parameters | Value |
| --- | --- |
| A3 event offset (dB) | 2 |
| TTT (ms) | 320 |
| UE speed (km/h) | 60/90/120 |
| Max ETD (ms) | 80 |

    - Methodology

In indirect measurement event prediction for intra-frequency temporal domain case A, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW via an RRM measurement prediction model. Afterwards, predicted and optionally actual historical measurement result(s) of the same cell are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

Based on the pattern in Table 2.1-1, 5 potential cases as illustrated in Figure 2.2-1 may happen when UE performs RRM measurement prediction:

Figure 2.1-1: Cases to perform measurement event prediction in Case A

- Case 1: 4 historical measurement instances already fulfill the entering condition, UE can derive whether the measurement event will be fulfilled based on the prediction result of the first instance.

- Case 2: 3 historical measurement instances already fulfill the entering condition, UE can derive whether the measurement event will be fulfilled based on the prediction result of the first 2 instances.

- Case 3: 2 historical measurement instances already fulfill the entering condition, UE can derive whether the measurement event will be fulfilled based on the prediction result of the first 3 instances.

- Case 4: 1 historical measurement instances already fulfill entering condition, UE can derive whether the measurement event will be fulfilled based on the prediction result of the first 4 instances.

- Case 5: no historical measurement instance fulfills the entering condition, UE can derive whether the measurement event will be fulfilled based on the prediction result of all instances within the prediction window.

Another aspect that may have an impact on the simulation performance is the inference timing. From our understanding, at least the following 2 candidate options can be considered:

- Option 1: Continuous inference. Consistent with Case A, a measurement event prediction result will be derived for each sampling period/measurement period;

- Option 2: Event-triggered inference. Inference is executed when UE moves to the edge of the serving cell, e.g., the entering condition for an event is met or about to be met based on actual measurement.

Proposal 1: For indirect measurement event prediction based on Case A, the inference timing should be reported in the simulation, the following two inference options can be considered:

- Option 1: Continuous inference. Inference is executed every measurement period.

- Option 2: Event-triggered inference. Inference is executed when UE moves to the edge of the serving cell, e.g., the entering condition for an event is met or about to be met based on actual measurement.

    - Results of Measurement Event Prediction

The simulation results of measurement event prediction for Case A (continuous inference) are illustrated in Table 2.1-3.

Table 2.1-3: Simulation results of measurement event prediction for Case A

(Continuous inference)

|  |  | Precision | Recall | F1 score | Average prediction accuracy of RRM prediction（dB） |
| --- | --- | --- | --- | --- | --- |
| 60 km/h | AI | 96.70% | 97.39% | 0.97 | 1.09 |
|  | Non AI | 95.67% | 84.88% | 0.90 | 2.08 |
| 90 km/h | AI | 96.04% | 96.13% | 0.96 | 1.53 |
|  | Non AI | 94.85% | 85.89% | 0.90 | 2.56 |
| 120 km/h | AI | 95.82% | 95.43% | 0.96 | 1.69 |
|  | Non AI | 94.84% | 85.77% | 0.90 | 2.66 |

From our understanding, non-AI mechanisms can be compared as a benchmark. One of the simplest non-AI mechanisms is based on sample-and-hold, i.e., once the entering condition is met, the measurement event is predicted as fulfilled, otherwise, the measurement event is predicted as not fulfilled.

Proposal 2: To evaluate the gain of AI-based event prediction, introduce a non-AI mechanism based on sample-and-hold as the benchmark, i.e., once the entering condition is met, the measurement event is predicted as fulfilled, otherwise, the measurement event is predicted as not fulfilled.

Based on the above simulation result, we observe that:

Observation 1: The indirect measurement event prediction based on temporal domain Case A RRM measurement prediction can achieve better performance (Precision, Recall and F1 score) than the non-AI approach based on sample-and-hold.

In addition, with the results for different UE speeds, it can be observed that higher RRM measurement prediction accuracy results in better performance.

Observation 2: For indirect measurement event prediction, higher RRM measurement prediction accuracy results in better performance (Precision, Recall and F1 score).

The confusion matrix for Continuous inference is provided in Table 2.1-4, and the total inference number is 11500.

Table 2.1-4: Confusion Matrix of Measurement Event Prediction for Case A

(Continuous inference)

| 60km/h-AI |  | Ground Truth |  |
| --- | --- | --- | --- |
|  |  | Positive (1925) | Negative (9575) |
| Prediction | Positive | 1904 | 65 |
|  | Negative | 51 | 9480 |
| 60km/h-Non AI |  | Ground Truth |  |
|  |  | Positive (1925) | Negative (9575) |
| Prediction | Positive | 1656 | 75 |
|  | Negative | 295 | 9474 |
| 90km/h-AI |  | Ground Truth |  |
|  |  | Positive (2304) | Negative (9196) |
| Prediction | Positive | 2233 | 92 |
|  | Negative | 90 | 9085 |
| 90km/h-Non AI |  | Ground Truth |  |
|  |  | Positive (2304) | Negative (9196) |
| Prediction | Positive | 2008 | 109 |
|  | Negative | 330 | 9053 |
| 120km/h-AI |  | Ground Truth |  |
|  |  | Positive (2405) | Negative (9095) |
| Prediction | Positive | 2318 | 101 |
|  | Negative | 111 | 8970 |
| 120km/h-Non AI |  | Ground Truth |  |
|  |  | Positive (2405) | Negative (9095) |
| Prediction | Positive | 2097 | 114 |
|  | Negative | 348 | 8941 |

To study the impact of inference timing, the simulation results of measurement event prediction for Case A (event-triggered inference) are illustrated in Table 2.1-5. For the event-triggered inference, the inference is performed when the entering condition for the event is met, i.e., Cases 1~4 in Figure 2.1-1.

Table 2.1-5: Simulation results of measurement event prediction for Case A

(Event-triggered inference)

|  |  | Precision | Recall | F1 score | Average prediction accuracy（dB） |
| --- | --- | --- | --- | --- | --- |
| 60 km/h | AI | 97.99% | 98.77% | 0.98 | 1.09 |
|  | Non AI | 95.67% | 100.00% | 0.98 | 2.08 |
| 90 km/h | AI | 97.34% | 98.38% | 0.98 | 1.53 |
|  | Non AI | 94.85% | 100.00% | 0.97 | 2.56 |
| 120 km/h | AI | 97.67% | 98.00% | 0.98 | 1.69 |
|  | Non AI | 94.84% | 100.00% | 0.97 | 2.66 |

Based on the above simulation result, we observe that:

Observation 3: For indirect measurement event prediction for Case A, the F1 score of Option 2 (Event-triggered inference) is better than Option 1 (Continuous inference).

The confusion matrix is provided in Table 2.1-6, and the total inference number for UE speed at 90km/h is 2117.

Table 2.1-6: Confusion Matrix of Measurement Event Prediction for Case A

(Event-triggered inference)

| AI-based prediction (60 km/h) |  | Ground Truth |  |
| --- | --- | --- | --- |
|  |  | Positive (1623) | Negative (108) |
| Prediction | Positive | 1605 | 33 |
|  | Negative | 20 | 73 |
| Non-AI (60 km/h) |  | Ground Truth |  |
|  |  | Positive (1623) | Negative (108) |
| Prediction | Positive | 1656 | 75 |
|  | Negative | 0 | 0 |
| AI-based prediction (90 km/h) |  | Ground Truth |  |
|  |  | Positive (1965) | Negative (152) |
| Prediction | Positive | 1940 | 53 |
|  | Negative | 32 | 92 |
| Non-AI (90 km/h) |  | Ground Truth |  |
|  |  | Positive (1965) | Negative (152) |
| Prediction | Positive | 2008 | 108 |
|  | Negative | 0 | 0 |
| AI-based prediction (120 km/h) |  | Ground Truth |  |
|  |  | Positive (2051) | Negative (160) |
| Prediction | Positive | 2012 | 48 |
|  | Negative | 41 | 110 |
| Non-AI (120 km/h) |  | Ground Truth |  |
|  |  | Positive (2051) | Negative (160) |
| Prediction | Positive | 2097 | 114 |
|  | Negative | 0 | 0 |

Observation 4: For indirect measurement event prediction, the inference number of Option 2 ( Event-triggered inference) is much lower than Option 1 (Continuous inference).

Option 1 (Continuous inference) will result in a higher number of inferences, which is due to that the UE is located at the center of the cell in most cases, thus the inference is unnecessary. On the contrary, Option 2 (Event-triggered inference) can effectively reduce the number of inferences and is sufficient to avoid too-late handover. Therefore, Option 2 should be the baseline.

Proposal 3. For indirect measurement event prediction for Case A, the event-triggered inference should be considered as the baseline.

    - Results of SLS

At the RAN2#128 meeting[1], two handover modeling options below were provided for system-level simulation.

2 For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

The simulation results of SLS based on measurement event prediction for Case A are provided in Table 2.1-7.

Table 2.1-7: Simulation results of SLS based on measurement event prediction for Case A

|  | HOF rate | HO_num/UE/sec |
| --- | --- | --- |
| Legacy HO | 5.98% | 0.3505 |
| AI-based HO (Option 2) | 5.11% | 0.3649 |
| AI-based HO (Option 3 with Continuous inference) | 3.36% | 0.3765 |
| AI-based HO (Option 3 with Event-triggered inference) | 3.77% | 0.3705 |

Based on the above SLS result, we observe that:

Observation 5: The system-level performance (i.e., HOF rate and HO number) difference of AI-based HO (temporal domain Case A) with HO modeling Option 2 (HO command based on actual measurement event) and legacy HO mechanism is not significant.

Observation 6: The AI-based HO (temporal domain Case A) with HO modeling Option 3 (HO command based on predicted measurement event) outperforms the legacy HO mechanism in terms of HOF rate, with a minor increase in HO number.

For AI-based HO with HO modeling Option 3, the system-level performance of continuous inference and event-triggered inference is quite close. It’s worth noting that the above simulation result of event-triggered inference is based on the entry condition, in which Event-triggered inference is only executed when the entering condition for an event is met. Therefore the HOF of Continuous inference is slightly lower than Event-triggered inference. If the event-triggered inference(Case 5 shown in Figure 2.2-1) is executed when the entry condition is about to be met, the HOF performance should be quite close to continuous inference, but can save more power.

Observation 7: For AI-based HO (temporal domain Case A) with HO modeling Option 3, the system-level performance of continuous inference and event-triggered inference is quite close.

  - Temporal domain Case B

Based on our previous simulation results, for the FR1 intra-frequency temporal domain, sub-use case 1 achieves better performance than sub-use cases 2 and 3. Thus we carry out the measurement event prediction toward sub-use case 1 for FR1. Some key simulation parameters are shown in the following table:

Table 2.2-1: Key parameters of temporal domain prediction Case B

| Parameter | Values |
| --- | --- |
| Sub-use case | 1 |
| L1/L3 filtering | Non-sliding |
| OW length | 10*200=2000 ms |
| PW length | 200 ms |
| MRRT | 50% |
| Pattern |  |

As to the measurement event specific parameters, the baseline values in Table 2.2-2 are selected in the simulation.

Table 2.2-2: measurement event specific parameters for Case B

| Parameters | baseline value |
| --- | --- |
| A3 event offset (db) | 2 |
| TTT (ms) | 320 |
| UE speed (km/h) | 30 |
| Max ETD (ms, note1) | 80 |

    - Methodology

Based on the pattern in Table 2.2-1, 2 potential cases may happen when UE performs RRM measurement prediction:

- Case 1: One historical measurement instance already fulfills entering condition, UE can derive whether the measurement event will be fulfilled based on the prediction result of the next instance.

- Case 2: One prediction instance already fulfills entering condition, UE can derive whether the measurement event will be fulfilled based on the measurement result of the next instance.

Figure 2.2-1: Cases to perform measurement event prediction in Case B

    - Results of Measurement Event Prediction

The simulation results are illustrated in Table 2.2-3.

Table 2.2-3: Simulation results of measurement event prediction for Case B

|  |  | Precision | Recall | F1 score | Average prediction accuracy（dB） |
| --- | --- | --- | --- | --- | --- |
| 30 km/h | AI | 99.52% | 99.05% | 0.99 | 0.67 |
|  | Non AI | 99.48% | 98.87% | 0.99 | 0.79 |
| 60 km/h | AI | 99.30% | 98.93% | 0.99 | 1.24 |
|  | Non AI | 99.26% | 98.71% | 0.99 | 1.41 |
| 90 km/h | AI | 99.28% | 98.84% | 0.99 | 1.78 |
|  | Non AI | 99.24% | 98.62% | 0.99 | 1.95 |

Based on the above simulation result, we observe that:

Observation 8: For indirect measurement event prediction based on temporal domain Case B, both AI-based and non-AI approaches can achieve good performance due to the high prediction accuracy of RRM measurement, especially for low UE speed at 30km/h.

    - Results of SLS

At the RAN2#128 meeting[1], below handover modeling options were provided for system level simulation.

For System level simulation 1 For measurement event prediction for temporal domain case B: UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation. Remove T0 from the picture.

The simulation results of measurement event prediction for Case B are illustrated in Table 2.2-4.

Table 2.2-4: Simulation results of measurement event prediction for Case B

|  | HOF rate | HO_num/UE/sec |
| --- | --- | --- |
| Legacy HO | 0.58% | 0.1310 |
| AI-based HO | 0.87% | 0.1314 |

Based on the above simulation result, we observe that:

Observation 9: With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Temporal domain Case A

Methodology

Proposal 1: For indirect measurement event prediction based on Case A, the inference timing should be reported in the simulation, the following two inference options can be considered:

- Option 1: Continuous inference. Inference is executed every measurement period.

- Option 2: Event-triggered inference. Inference is executed when UE moves to the edge of the serving cell, e.g., the entering condition for an event is met or about to be met based on actual measurement.

Results of Measurement Event Prediction for Case A

Observation 1: The indirect measurement event prediction based on temporal domain Case A RRM measurement prediction can achieve better performance (Precision, Recall and F1 score) than the non-AI approach based on sample-and-hold.

Observation 2: For indirect measurement event prediction, higher RRM measurement prediction accuracy results in better performance (Precision, Recall and F1 score).

Observation 3: For indirect measurement event prediction for Case A, the F1 score of Option 2 (Event-triggered inference) is better than Option 1 (Continuous inference).

Observation 4: For indirect measurement event prediction, the inference number of Option 2 ( Event-triggered inference) is much lower than Option 1 (Continuous inference).

Proposal 2: To evaluate the gain of AI-based event prediction, introduce a non-AI mechanism based on sample-and-hold as the benchmark, i.e., once the entering condition is met, the measurement event is predicted as fulfilled, otherwise, the measurement event is predicted as not fulfilled.

Proposal 3. For indirect measurement event prediction for Case A, the event-triggered inference should be considered as the baseline.

Results of SLS for Case A

Observation 5: The system-level performance (i.e., HOF rate and HO number) difference of AI-based HO (temporal domain Case A) with HO modeling Option 2 (HO command based on actual measurement event) and legacy HO mechanism is not significant.

Observation 6: The AI-based HO (temporal domain Case A) with HO modeling Option 3 (HO command based on predicted measurement event) outperforms the legacy HO mechanism in terms of HOF rate, with a minor increase in HO number.

Observation 7: For AI-based HO (temporal domain Case A) with HO modeling Option 3, the system-level performance of continuous inference and event-triggered inference is quite close.

Temporal domain Case B

Results of Measurement Event Prediction for Case B

Observation 8: For indirect measurement event prediction based on temporal domain Case B, both AI-based and non-AI approaches can achieve good performance due to the high prediction accuracy of RRM measurement, especially for low UE speed at 30km/h.

Results of SLS for Case B

Observation 9: With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism.

- Reference

- R2_128_ChairNotes