3GPP TSG-RAN WG2 Meeting #129	R2-2500400

# Athens, Greece, 17 – 21 February 2025

Agenda item:		8.6.3

Source:	Xiaomi

Title:	Remaining issues of L1 event triggered measurement reporting

Document for:		Discussion and Decision

Introduction

In RAN2#128 meeting, following was agreed regarding L1 event triggered measurement reporting [1]:

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

In this contribution, we discuss the remaining issues of the L1 event triggered measurement reporting.

Discussion

Report contents

Current agreements for L1 MR MAC CE are to report the L1 beam-level measurement results. With regards to the L3 cell-level measurement results, we think that the network can provide an extra configuration to control whether the L3 cell-level measurement results of either the SpCell and/or the candidate cell can be reported, so that the gNB can have more information (e.g. cell quality) to determine whether to handover the UE to a candidate cell, e.g. to avoid ping-pong handover.

Proposal 1: The report of the L3 cell-level measurement result of the SpCell is configurable.

Proposal 2: The report of the L3 cell-level measurement result of the candidate cell is configurable.

Cell specific offset

In the RAN2#126 meeting, RAN2 made the following agreement for the offset configured for LTM event evaluation:

- For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

3GPP TS 38.331: Entering condition of measurement event Event 2: Ms + Hys < Thresh Event 3: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off Event 4: Mn + Ofn + Ocn – Hys > Thresh Event 5: Mp + Hys < Thresh1 Mn + Ofn + Ocn – Hys > Thresh2

According to the L3 measurement events based on the cell-level measurement results, one event could have the following offset (as the example of Event A3 given above):

- Frequency/measurement object specific offset (e.g. Ofn/ Ofp)

- Cell specific offset (e.g. Ocn/ Ofp)

According to the RAN2#127 meeting agreement, the cell level measurement result is not be used for the L1 LTM measurement event.

- Beam level measurement result, not cell level measurement result, is used for LTM event evaluation.

Since the gNB is able to control the L1 measurement report for each beam separately by using the beam specific offset and the L1 measurement event is triggered by beam-level measurement result, there is no need to introduce cell specific offset for event evaluation. Thus, the entering/leaving condition for LTM measurement event would be as follows:

Examples of entering/leaving condition of LTM measurement event: LTM Event 2: Entering condition: Ms + Hys < Thresh Leaving condition: Ms – Hys > Thresh LTM Event 3: Entering condition: Mn + Obn – Hys > Mp + Obp + Off Leaving condition: Mn + Obn + Hys < Mp + Obp + Off LTM Event 4: Entering condition: Mn + Obn – Hys > Thresh Leaving condition: Mn + Obn + Hys < Thresh LTM Event 5: Entering condition 1: Mp + Hys < Thresh1 Entering condition 2: Mn + Obn – Hys > Thresh2 Leaving condition 1: Mp – Hys > Thresh1 Leaving condition 2: Mn + Obn + Hys < Thresh2

According to the example given above, the beam specific offset (including Obn of the beam specific offset of the candidate beam and Obp of the beam specific offset of the SpCell) will be used for LTM Event 3/4/5. According to the L3 measurement configuration, the cell specific offset can be configured independently for each cell including both the SpCell and the neighbouring cell(s). Thus, we think that the network can provide the beam specific offset for each beam, including both the serving beam and the candidate beams.

Proposal 3: For LTM event evaluation, cell specific offset is not needed.

Priority of L1 MR MAC CE

In the RAN2#127bis meeting, RAN2 made the following agreements for the SR used for the L1 MR MAC CE:

11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. 12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

In general, we think that the triggering/cancellation/priority of the dedicated SR for the L1 MR MAC CE can reuse the procedure defined for the dedicated SR for BFR MAC CE. This means that the dedicated SR is triggered when a L1 measurement report is triggered. When the L1 measurement report is cancelled (e.g. due to the report/cancellation of the L1 MR MAC CE), the dedicated SR is also cancelled. The related note in TS 38.321 is copied below:

NOTE 3:	When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid.

According to TS 38.321 as quoted above, the BFR SR has higher priority than the BSR SR, because the beam failure recovery is more urgent than the data transmission. From our understanding, the transmission of L1 MR MAC CE should also have higher priority, so that the LTM cell switch can be triggered in time. This means that the priority of the SR for MR MAC CE should be higher than that of the SR for BSR.

Proposal 4: The dedicated SR for L1 Measurement Report MAC CE has higher priority than the SR for BSR.

Logical channel priority of MAC CEs and LCHs are defined in LCP procedure, with related part of TS 38.321 copied below:

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for SL LBT failure according to clause 5.31.2;

-	MAC CE for Timing Advance Report;

-	MAC CE for Delay Status Report;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for SL-PRS Resource Request;

-	MAC CE for (Extended) BSR, with exception of BSR included for padding;

…

Logical channel priority of L1 Measurement Report MAC CE should be high enough to avoid mobility problems. It is proposed that logical channel priority of L1 Measurement Report MAC CE is the same as that of BFR MAC CE.

Proposal 5: Logical channel priority of L1 Measurement Report MAC CE is the same as that of BFR MAC CE.

MAC CR FFS issue

In the draft MAC CR in email discussion “[POST128][109][MOB] MAC Running CR (Vivo)”, there is restriction that only UL resource on SpCell is considered, as from below (clause 5.x.4 of draft CR):

For the L1 event triggered measurement reporting, the MAC entity shall:

1>	if at least one L1 measurement report has been triggered as specified in 5.x.3 [and not cancelled]:

2>	if UL-SCH resources are available for a new transmission in the SpCell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the L1 measurement report MAC CE;

However, in TS 38.321, only LBT failure MAC CE has the restriction for SpCell, while other MAC CEs (BSR, SL-BSR, Pre-emptive BSR, TAR, DSR, BFR, SL-PRS Resource Request, Positioning Measurement Gap Activation/Deactivation Request, SL LBT failure) do not have such restriction. LBT failure MAC CE handling in SpCell is based on TS 38.300 clause 5.6.1:

When the UE detects consistent uplink LBT failures, it takes actions as specified in TS 38.321 [6]. The detection is per Bandwidth Part (BWP) and based on all uplink transmissions within this BWP. When consistent uplink LBT failures are detected on SCell(s), the UE reports this to the corresponding gNB (MN for MCG, SN for SCG) via MAC CE on a different serving cell than the SCell(s) where the failures were detected. If no resources are available to transmit the MAC CE, a Scheduling Request (SR) can be transmitted by the UE. When consistent uplink LBT failures are detected on SpCell, the UE switches to another UL BWP with configured RACH resources on that cell, initiates RACH, and reports the failure via MAC CE.

There is no motivation to apply LBT failure MAC CE behavior to L1 Measurement Report MAC CE, therefore there should be no restriction that L1 Measurement Report MAC CE can be only transmitted on SpCell.

Proposal 6: There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on.

RRC CR FFS issue

Following issue was categorized as FFS by rapporteur in email discussion “[POST128][108][MOB] RRC running CR (Huawei)”:

| Issue | Suggestion |
| --- | --- |
| Field description for ltm3-Offset-r19 | Whether we need to add “The actual value is field value * 0.5 dB.” similar to a3-Offset and a6-Offset, i.e. whether the granularity is 0.5 dB instead of 1 dB? [Rapp] This can be further discussed. Added an FFS for this. |

This is related to the range and granularity of ltm3-Offset-r19, which is the parameter Off for LTM3 event entering condition below:

Mn + Obn – Hys > Ms + Obs + Off

Related ASN.1 is copied below:

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

...

ltm-ReportConfigType-r18                           CHOICE {

eventTriggered-r19                        SEQUENCE {

eventId-r19                                     CHOICE {

...

eventLTM3-r19                                     SEQUENCE {

ltm3-Offset-r19                                   MeasTriggerQuantityOffset,

hysteresis-r19                                    Hysteresis,

timeToTrigger-r19                                 TimeToTrigger,

...

},

...

},

...

}

},

...

}

MeasTriggerQuantityOffset ::=               CHOICE {

rsrp                                        INTEGER (-30..30),

rsrq                                        INTEGER (-30..30),

sinr                                        INTEGER (-30..30)

}

With 1 dB and 0.5 dB granularity, the range of ltm3-Offset-r19 is (-30..30) dB and (-15..15) dB, respectively. Our understanding is that (-15..15) dB range is sufficient for L1 event evaulation, similar to L3 event. Therefore 0.5 dB granularity is preferred for more precise control.

Proposal 7: For field ltm3-Offset-r19, the granularity is 0.5 dB.

Conclusion

In this contribution, we discuss the remaining issues of the L1 event triggered measurement reporting, and propose the following:

Proposal 1: The report of the L3 cell-level measurement result of the SpCell is configurable.

Proposal 2: The report of the L3 cell-level measurement result of the candidate cell is configurable.

Proposal 3: For LTM event evaluation, cell specific offset is not needed.

Proposal 4: The dedicated SR for L1 Measurement Report MAC CE has higher priority than the SR for BSR.

Proposal 5: Logical channel priority of L1 Measurement Report MAC CE is the same as that of BFR MAC CE.

Proposal 6: There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on.

Proposal 7: For field ltm3-Offset-r19, the granularity is 0.5 dB.

- References

[1] R2-2410911, Vice Chairman (Samsung), "Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES"